# Assessment Requirements for MSTTN2001 Set up and operate a dry laid web forming machine

Release: 1

Assessment Requirements for MSTTN2001 Set up and operate a dry laid web forming machine

# Modification History

Release 1. Supersedes and is equivalent to LMTTN2001B Set up and operate a dry laid web forming machine

# Performance Evidence

Evidence required to demonstrate competence in this unit must be relevant to and satisfy the requirements of the elements and performance criteria on at least two occasions and include:

reading and following work instructions, standard operating procedures (SOPs), safe work practices and other reference material

applying relevant standards

checking machine safety equipment and reporting any faults

preparing materials for web forming

loading and unloading materials

starting and stopping dry laid web forming machines according to specifications on at least two (2) occasions

monitoring machine operations, including machine production readings, on at least two (2) occasions

recognising, rectifying and reporting machine faults or problems as required

maintaining clean machines and work environment, as required

undertaking quality checks and recognising and responding to faulty product

applying workplace procedures

maintaining accurate records.

# Knowledge Evidence

Evidence must be provided that demonstrates knowledge of:

safe work practices and procedures and use of personal protective equipment (PPE)

relevant Australian Standards

fibre types, including:

rayon and other cellulosics

nylon

polyester

polypropylene

cotton

wool

glass

bi-component

fibre characteristics, including:

absorbency

abrasion resistance

modulus

web ‘handle’

moisture absorption

chemical and temperature resistance

melting point

fibre measuring units, including decitex and denier and fibre diameter - equation

fibre or filament crimp

advantages of fibre opening and blending:

component independent system

guarantee a homogeneous mixture

constant performance

flexible operation

high degree of automation

working and stripping principles of carding

different types of carded webs, such as:

parallel laid web

parallel laid web with spreading device

parallel laid web with scrambling

random laid web

random laid web with scrambling

layered and cross laid webs

WHS practices, including hazard identification and control measures

quality practices

workplace procedures

recording and reporting practices.

# Assessment Conditions

Assessors must:

satisfy the assessor competency requirements that are in place at the time of the assessment, as set by the VET regulator

have vocational competency in setting up and operating a dry laid web forming machine, at least to the level being assessed, with relevant industry knowledge and experience.

Assessment should occur in operational workplace situations. Where this is not possible, or where personal safety or environmental damage are limiting factors, assessment must occur in a sufficiently rigorous simulated environment reflecting realistic operational workplace conditions. This must cover all aspects of workplace performance, including environment, task skills, task management skills, contingency management skills and job role environment skills.

Assessment processes and techniques must be appropriate to the language, literacy and numeracy requirements of the work being performed and the needs of the candidate.

Conditions for assessment must include access to all tools, equipment, materials and documentation required, including relevant workplace procedures, product and manufacturing specifications associated with this unit.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=a203ec5c-de7d-406b-b3e1-8f1a9b76e92e