# Assessment Requirements for MSTTX3011 Set up, adjust and monitor a machine for TCF production

Release: 1

Assessment Requirements for MSTTX3011 Set up, adjust and monitor a machine for TCF production

# Modification History

Release 1. Supersedes:

LMTTX3011A Set up, adjust and maintain circular knitting machines

LMTTX3012A Set up, adjust and maintain  flat knitting machines

LMTTX3013A Set up, adjust and maintain spinning machines

LMTTX3014A Set up, adjust and maintain carding machines

LMTTX3015A Set up, adjust and maintain tufting looms

LMTTX3016A Set up, adjust and maintain domestic sewing machines

LMTTX3020A Identify and correct weaving process faults

LMTTX3021A Apply knowledge of weaving processes to production requirements

LMTTX3022A Set up and operate weaving looms for production

Not equivalent

# Performance Evidence

Evidence required to demonstrate competence in this unit must be relevant to and satisfy the requirements of the elements and performance criteria, and include:

reading and following work instructions, standard operating procedures (SOPs), safe work practices, manufacturer operational guidelines

applying relevant standards

interpreting production specifications accurately

performing sample runs and analysing results to ensure quality outcomes

identifying and correcting product related faults and making repairs or corrections, within scope of own responsibilities and manufacturer operational guidelines

identifying and making appropriate machine readjustments and repairs, according to own responsibilities, workplace procedures and manufacturer specifications, on at least two (2) occasions

starting, operating, stopping, re-starting and monitoring at least one (1) machine in accordance with manufacturer requirements to produce textile material or product, on at least two (2) occasions

determining root cause and solving operational problems or referring to relevant personnel

setting and operating machine safely and correctly to run at optimum quality and efficiency levels to produce required quantity and quality of product

applying work health and safety (WHS) and workplace procedures

maintaining accurate records.

# Knowledge Evidence

Evidence must be provided that demonstrates knowledge of:

safe work practices and procedures and use of personal protective equipment (PPE)

relevant Australian Standards

setting up and adjustment requirements for the identified machine

specifications for operation of particular machines

textile production machines and equipment used in the enterprise:

machines for knitting :

circular knitting machines

electronic and automatic flat knitting machines

machines for spinning:

spinning machines

drawing machines

roving machines

twisting machines

winding machines

carding machines:

burr handling systems

air filtration equipment

combs

doffer waste system

blending systems

compressed air and hydraulic systems

tufting looms:

winding machines

finishing equipment

other ancillary equipment

domestic sewing machines:

lockstitch and chain stitch sewing machines

overlock sewing machines and sergers

high volume automatic and manually operated hem, selvedge and embroidery sewing machines

weaving machines:

weaving looms

broadloom

narrowloom

all shuttle types

Jacquard looms

microprocessor or computer-controlled looms

dyeing machines

finishing machines

ancillary machines typically used in the textile production sector concerned, including:

production and service equipment used in the enterprise

loading and unloading equipment

labelling machine or printer

packaging machine

stud machine

folding machine

typical products produced by textile production machine, including knitting, sewing, weaving, spinning, carding, tufting and finishing machines:

fibres, including polyester, cotton, wool, viscose and rayon

yarns, including ring spun, open-ended spun, air jet spun and friction spun

fabrics

finished products, including socks, sweaters, towels, sheets, blankets, carpets, rope and twine, mops and industrial textiles

product specifications required for machine

machine methods and the various types of effects they can produce

sewing, knitting, weaving, spinning, carding methods and the various types of products and effects they can produce

fibre types and their method of production

fibre attributes and parameters

yarn characteristics, twist and application

yarn counts and quality indicators

fabrics and knit effects

production process and types of production

pattern specifications

factors affecting machine performance, including yarn strength, sizing and ambient conditions

machine manufacturer specifications

fault identification in materials and textile products, and methods of repair, including threading faults, joining of yarn and tensioning, and setting of clearances, limits and fits

identification of machine faults and methods of repair, including removal and replacement of faulty or worn parts, and re-setting or re-programming of controls and sensors

WHS and environmental aspects of relevant enterprise activities

quality practices

workplace procedures

recording and reporting practices

# Assessment Conditions

Assessors must:

satisfy the assessor competency requirements that are in place at the time of the assessment, as set by the VET regulator

have vocational competency in setting up, adjusting and monitoring a machine for TCF production, at least to the level being assessed, with relevant industry knowledge and experience.

Assessment should occur in operational workplace situations. Where this is not possible, or where personal safety or environmental damage are limiting factors, assessment must occur in a sufficiently rigorous simulated environment reflecting realistic operational workplace conditions. This must cover all aspects of workplace performance, including environment, task skills, task management skills, contingency management skills and job role environment skills.

Assessment processes and techniques must be appropriate to the language, literacy and numeracy requirements of the work being performed and the needs of the candidate.

Conditions for assessment must include access to all tools, equipment, materials and documentation required, including relevant workplace procedures, product and manufacturing specifications associated with this unit.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=a203ec5c-de7d-406b-b3e1-8f1a9b76e92e