# NWPGEN014 Sample and test trade wastewater

Release: 1

NWPGEN014 Sample and test trade wastewater

# Modification History

| Release | Comments |
| --- | --- |
| 1 | This unit was released in NWP Water Training Package release 1.0 and meets the Standards for Training Packages. |

# Application

This unit describes the skills required to collect and prepare trade wastewater samples, perform basic field based tests and conduct and record analysis.

This unit applies to those working in the water industry but might be relevant to others.

The skills and knowledge described in this unit must be applied within the legislative, regulatory and policy environment in which they are carried out. Organisational policies and procedures must be consulted and adhered to.

Those undertaking this unit would work under indirect supervision, while performing complex tasks in familiar contexts.

No licensing, legislative or certification requirements apply to unit at the time of publication.

# Competency Field

General

# Elements and Performance Criteria

| ELEMENTS | PERFORMANCE CRITERIA |
| --- | --- |
| Elements describe the essential outcomes | Performance criteria describe the performance needed to demonstrate achievement of the element. Where bold italicised text is used, further information is detailed in the range of conditions section. |
| 1. Prepare for trade wastewater sampling and testing | 1 1 Identify the need to sample based on trade waste customer risk, sewer catchment and infrastructure issues. 1.2 Schedule sampling and testing in accordance with a monitoring program. 1.3 Identify wastewater quality parameters to be tested for. 1.4 Identify appropriate sampling location and type of sample to be taken in accordance with the monitoring program. 1.5 Communicate with an external laboratory regarding sampling, preservation, parameters to be analysed and transportation requirements. 1.6 Select and apply sample preservation methods. 1.7 Select, check, calibrate and prepare equipment and resources. 1.8 Select, fit and use personal protective equipment. |
| 2. Conduct trade wastewater sampling and testing | 2.1 Adopt hygienic work practices. 2.2 Carry out job safety analysis or risk assessment. 2.3 Determine sampling method, either manual or automatic and time based or flow based. 2.4 Sample and determine the trade wastewater related to the typical operation of the business. 2.5 Collect samples ensuring that types, locations and times comply with requirements. 2.6 Maintain integrity of samples during sampling and field testing. 2.7 Conduct basic field trade wastewater tests. 2.8 Assess and report abnormal characteristics and undertake corrective action. 2.9 Limit hazards and contamination to self, work area and environment. |
| 3. Finalise work | 3.1 Apply chain of custody procedures. 3.2 Organise transportation of samples to the laboratory. 3.3 Report observations or measurements requiring further action. 3.4 Clean testing equipment and sampling site. 3.5 Record analysis results from field testing and laboratory testing. 3.6 Interpret test results, assess compliance and reschedule sampling to confirm findings. |

# Foundation Skills

The foundation skills demands of this unit have been mapped for alignment with the Australian Core Skills Framework (ACSF). The following tables outline the performance levels indicated for successful attainment of the unit.

Further information on ACSF and the foundation skills underpinning this unit can be found in the Foundation Skills Guide on the GSA website.

# Range of Conditions

| Field testing includes: | pH temperature conductivity metals colour visual assessment odour sulphides |
| --- | --- |
| Sampling processes include: | automatic flow proportional composites time proportional composites discreet manual |

# Unit Mapping Information

No equivalent unit.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=26336bc0-04e5-49d9-8c31-46c49b6a0037

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=26336bc0-04e5-49d9-8c31-46c49b6a0037

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=26336bc0-04e5-49d9-8c31-46c49b6a0037

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=26336bc0-04e5-49d9-8c31-46c49b6a0037

Assessment Requirements for NWPGEN014 Sample and test trade wastewater

# Modification History

| Release | Comments |
| --- | --- |
| 1 | These Assessment Requirements were released in NWP Water Training Package release 1.0 and meet the Standards for Training Packages. |

# Performance Evidence

Evidence required to demonstrate competence must satisfy all of the requirements of the elements and performance criteria. The candidate must be assessed in both types of sampling processes and at least two types of field tests listed in the Range of Conditions. If not otherwise specified the candidate must demonstrate evidence of performance of the following on at least one occasion.

conducting hazard and risks assessments

planning working activities

applying procedures and standards

performing task-related calculations

maintaining a safe work environment

using automatic samplers and flow measurement equipment

using recording equipment (i.e. pH or temperature recorder) and field test kits

conducting trade waste tests on-site according to specifications

disposing of waste and spent samples

identifying which parameters can be analysed in-field and which need laboratory testing

identifying necessary equipment and resources for sampling and testing (i.e. field test kits, preservation reagents)

selecting appropriate sample jars and preparing documentation (labels, chain of custody and other required forms, i.e. inspection sheets)

installing sampling, flow measuring (if applicable) and recording equipment in accordance with manufacturers’ specifications and site specific conditions

applying labels to samples and prepare appropriate documentation for external analyses

# Knowledge Evidence

Evidence required to demonstrate competence must satisfy all of the requirements of the elements and performance criteria. If not otherwise specified the depth of knowledge demonstrated must be appropriate to the job context of the candidate.

relevant policies and procedures including chain of custody

relevant standard methods for testing of water

range and purpose of basic water quality testing and sampling

abnormal characteristics of water samples

relevant calculations

storage of reagents

relevant legislative and organisational requirements

trade waste sample preparation:

prevention of contamination

volume of sample

appropriate containers

preservation

location selection

equipment

documentation and transportation procedures for trade waste samples

disposal procedures for excess trade waste samples

sampling and testing requirements

water quality parameters of significance to a range of trade waste discharges

hazards associated with collection of trade waste samples

# Assessment Conditions

Competency should be assessed in an actual workplace or in a simulated environment, with access to equipment and infrastructure appropriate to the outcome. Competency should be demonstrated over time to ensure the candidate is assessed across a variety of situations.

Assessors must satisfy the NVR/AQTF mandatory competency requirements for assessors.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=26336bc0-04e5-49d9-8c31-46c49b6a0037

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=26336bc0-04e5-49d9-8c31-46c49b6a0037

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=26336bc0-04e5-49d9-8c31-46c49b6a0037

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=26336bc0-04e5-49d9-8c31-46c49b6a0037