# NWPTRT083 Operate and control aerobic bioreactor processes

Release: 1

NWPTRT083 Operate and control aerobic bioreactor processes

# Modification History

| Release | Comments |
| --- | --- |
| 1 | This unit was released in NWP Water Training Package release 1.0 and meets the Standards for Training Packages. This unit supersedes and is equivalent to NWP350B Monitor, operate and control trickling filter processes. Unit code updated Content and formatting updated to comply with the new standards All PC transitioned from passive to active voice Unit title changed to better reflect unit outcomes |

# Application

This unit of competency describes the skills required to monitor, operate and control fixed film aerobic bioreactor processes, including trickling filters and rotating biological contactors. This unit of competency also describes the outcomes required to measure and report on system performance and process quality control.

This unit supports the attainment of skills and knowledge required for operational staff with a specific responsibility for fixed film aerobic bioreactor processes in treatment plants.

The skills and knowledge described in this unit must be applied within the legislative, regulatory and policy environment in which they are carried out. Organisational policies and procedures must be consulted and adhered to, particularly those relating to safety.

Those undertaking this unit would work under supervision, while performing routine tasks, within a range of familiar contexts.

No licensing, legislative or certification requirements apply to unit at the time of publication.

# Competency Field

Treatment

# Elements and Performance Criteria

| ELEMENTS | PERFORMANCE CRITERIA |
| --- | --- |
| Elements describe the essential outcomes | Performance criteria describe the performance needed to demonstrate achievement of the element. Where bold italicised text is used, further information is detailed in the range of conditions section. |
| 1. Monitor fixed film aerobic bioreactor process performance | 1.1 Monitor test results and processes. 1.2 Identify and report faults and the operational condition of the process. |
| 2. Operate and control fixed film aerobic bioreactor processes | 2.1 Select and check equipment and tools required to meet safety requirements of task and site. 2.2 Select, fit and use personal protective equipment. 2.3 Carry out routine plant inspections. 2.4 Collect process samples and conduct standard tests. 2.5 Carry out system adjustments and process calculations to enhance system performance. |
| 3. Complete documentation | 3.1 Collect, interpret and record process data. 3.2 Report observations outside defined parameters for further action. |

# Foundation Skills

The foundation skills demands of this unit have been mapped for alignment with the Australian Core Skills Framework (ACSF). The following tables outline the performance levels indicated for successful attainment of the unit.

Further information on ACSF and the foundation skills underpinning this unit can be found in the Foundation Skills Guide on the GSA website.

# Unit Mapping Information

This unit supersedes and is equivalent to NWP350B Monitor, operate and control trickling filter processes.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=26336bc0-04e5-49d9-8c31-46c49b6a0037

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=26336bc0-04e5-49d9-8c31-46c49b6a0037

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=26336bc0-04e5-49d9-8c31-46c49b6a0037

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=26336bc0-04e5-49d9-8c31-46c49b6a0037

Assessment Requirements for NWPTRT083 Operate and control aerobic bioreactor processes

# Modification History

| Release | Comments |
| --- | --- |
| 1 | These Assessment Requirements were released in NWP Water Training Package release 1.0 and meet the Standards for Training Packages. Assessment Requirements created drawing upon specified assessment information from superseded unit |

# Performance Evidence

Evidence required to demonstrate competence must satisfy all of the requirements of the elements and performance criteria. If not otherwise specified the candidate must demonstrate evidence of performance of the following on at least one occasion.

identifying and correcting operational and control system problems

taking samples and performing tests:

settling tests

microscopic observation

pH

dissolved oxygen

nutrient analyses, including at least one of:

nitrogen

phosphorus

temperature

completing logs and reports

performing relevant process calculations:

hydraulic and organic loading rates

recirculation ratios

monitoring test results

identifying and reporting faults

conducting routine plant inspections

making basic process adjustments:

pH correction

dissolved oxygen levels

recirculation rates

collecting data and completing required documentation

# Knowledge Evidence

Evidence required to demonstrate competence must satisfy all of the requirements of the elements and performance criteria. If not otherwise specified the depth of knowledge demonstrated must be appropriate to the job context of the candidate.

process layout

features and components of fixed film aerobic bioreactor systems

theory of process operation and monitoring

chemicals used for pH control, odour control, nutrients

flow measurement

risk factors and potential hazards relating to fixed film aerobic bioreactor systems

risk control requirements including safety equipment and material safety data sheets

lockout procedures for mechanical and electrical installations and hydraulic isolation

equipment operation, capacity and limitations

relevant utilities and service bodies

process calculation

hazardous materials handling

chemical and biological principles that form the basis of fixed film aerobic wastewater treatment

types of fixed film processes

recirculation principles

operational problems

# Assessment Conditions

Competency should be assessed in an actual workplace or in a simulated environment, with access to equipment and infrastructure appropriate to the outcome. Competency should be demonstrated over time to ensure the candidate is assessed across a variety of situations.

Assessors must satisfy the NVR/AQTF mandatory competency requirements for assessors.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=26336bc0-04e5-49d9-8c31-46c49b6a0037

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=26336bc0-04e5-49d9-8c31-46c49b6a0037

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=26336bc0-04e5-49d9-8c31-46c49b6a0037

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=26336bc0-04e5-49d9-8c31-46c49b6a0037