# PMAOPS220 Monitor chemical reactions in the process

Release: 1

PMAOPS220 Monitor chemical reactions in the process

# Modification History

Release 1. Supersedes and is equivalent to PMAOPS220B Monitor chemical reactions in the process

# Application

This unit of competency covers the skills and knowledge required to operate a reaction system which forms an integral part of a production process.

The reactor and its control will be relatively simple, operated in a stand-alone manner. The reaction may, or may not, include the use of catalyst (either homogeneous or heterogeneous phase).

The vessel in which the reaction is occurring may be a purpose built ‘kettle’ or other reaction vessel or it may simply be a stirred tank. These are typically found in small batch plants, although appropriate examples might also exist in larger plants. Processes may be batch or continuous.

This unit of competency applies to operators who are required to monitor the reactor operation, make adjustments to process or product properties as directed or to procedure and identify operational problems and take appropriate action.

This unit of competency applies to an individual who may work alone although under routine direction and supervision. They may work as part of a team or group and will work in liaison with other shift team members and the control room operator, as appropriate.

This competency does not cover operating a control panel. Where this is required the appropriate control unit should be used.

No licensing, legislative or certification requirements apply to this unit at the time of publication.

# Pre-requisite Unit

Nil

# Competency Field

Operations

# Unit Sector

# Elements and Performance Criteria

| Elements describe the essential outcomes. |  | Performance criteria describe the performance needed to demonstrate achievement of the element. |  |
| --- | --- | --- | --- |
| 1 | Prepare for work | 1.1 | Receive and give shift handover |
|  |  | 1.2 | Identify work requirements |
|  |  | 1.3 | Identify and control hazards |
|  |  | 1.4 | Coordinate with appropriate personnel |
|  |  | 1.5 | Check for recent work undertaken on plant item |
|  |  | 1.6 | Note any outstanding/incomplete work |
|  |  | 1.7 | Check operational status of equipment |
| 2 | Operate reaction system in accordance with procedures | 2.1 | Identify the type of reactor and duty |
|  |  | 2.2 | Complete routine checks, logs and paperwork taking appropriate action on unexpected readings |
|  |  | 2.3 | Adjust reactor, as appropriate to type and duty |
|  |  | 2.4 | Change rate, grade or specification smoothly |
|  |  | 2.5 | Charge/discharge reactor |
| 3 | Recognise and take action on abnormal situations in accordance with procedures | 3.1 | Monitor reactor frequently and critically throughout shift using measured/indicated data and senses |
|  |  | 3.2 | Identify impacts of any changes upstream and downstream |
|  |  | 3.3 | Recognise situations which may require action |
|  |  | 3.4 | Resolve routine problems |
|  |  | 3.5 | Take actions on other abnormal situations to make safe and have the situation resolved |
| 4 | Isolate and de-isolate plant | 4.1 | Complete any required pre-start checks |
|  |  | 4.2 | Start up/shut down /changeover reactor according to the plant type and duty in liaison with other personnel |
|  |  | 4.3 | Isolate reactor |
|  |  | 4.4 | Make safe for required work |
|  |  | 4.5 | Check reactor is ready to be returned to service |
|  |  | 4.6 | De-isolate and prepare reactor for return to service |

# Foundation Skills

This section describes those language, literacy, numeracy and employment skills that are essential to performance.

Foundation skills essential to performance are explicit in the performance criteria of this unit of competency.

# Range of Conditions

| This field allows for different work environments and conditions that may affect performance. Essential operating conditions that may be present (depending on the work situation, needs of the candidate, accessibility of the item, and local industry and regional contexts) are included. |  |
| --- | --- |
| Regulatory framework | The latest version of all legislation, regulations, industry codes of practice and Australian/international standards, or the version specified by the local regulatory authority, must be used, and include one or more of the following: legislative requirements, including work health and safety (WHS) industry codes of practice and guidelines environmental regulations and guidelines Australian and other standards licence and certification requirements All operations to which this unit applies are subject to stringent health, safety and environment (HSE) requirements, which may be imposed through state/territory or federal legislation, and these must not be compromised at any time. Where there is an apparent conflict between performance criteria and HSE requirements, the HSE requirements take precedence. |
| Procedures | All operations must be performed in accordance with relevant procedures. Procedures are written, verbal, visual, computer-based or in some other form, and include one or more of the following: emergency procedures work instructions standard operating procedures (SOPs) safe work method statements (SWMS) formulas/recipes batch sheets temporary instructions any similar instructions provided for the smooth running of the plant |
| Hazards | Hazards include one or more of the following: electricity gas gases and liquids under pressure structural hazards structural collapse equipment failures industrial (machinery, equipment and product) equipment or product mass noise, rotational equipment or vibration plant services (steam, condensate and cooling water) working at heights, in restricted or confined spaces, or in environments subjected to heat, noise, dusts or vapours flammability and explosivity hazardous products and materials unauthorised personnel sharp edges, protrusions or obstructions slippery surfaces, spills or leaks extreme weather other hazards that might arise |
| Routine problems | Routine problems must be resolved by applying known solutions. Routine problems are predictable and include one or more of the following: variations in material composition variation in ambient conditions control of reaction temperature adjustments to meet product specifications Known solutions are drawn from one or more of the following: procedures training remembered experience Non-routine problems must be reported according to according to relevant procedures. |
| Action | Action in accordance with procedures includes the following: determining problems needing action determining possible fault causes rectifying problem using appropriate solution within area of responsibility following through items initiated until final resolution has occurred reporting problems outside area of responsibility to designated person |
| Start up/shut down as required | Start up/shut down as required includes the following: start up and shut down to/from normal operating conditions start up and shut down to/from isolated, cold or empty start up and shut down to/from other conditions/situations experienced on the plant |
| Operate | Operate is to monitor, adjust/change the plant item/unit/system to meet specifications, by one or more of the following: manually in the plant using local controller in the plant using the process control system in the control room |

# Unit Mapping Information

Release 1. Supersedes and is equivalent to PMAOPS220B Monitor chemical reactions in the process

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=9fc2cf53-e570-4e9f-ad6a-b228ffdb6875