# PMAOPS233 Monitor wells and gathering systems

Release: 1

PMAOPS233 Monitor wells and gathering systems

# Modification History

Release 1. Supersedes and is equivalent to PMAOPS233A Monitor wells and gathering systems

# Application

This unit of competency covers the skills and knowledge required to monitor wells and associated equipment in the field.

This unit of competency applies to field operators who are required to take readings; undertake inspections; recognise, record and report problems; and identify and control hazards. The field operator is not required to rectify problems or make adjustments as part of this competency.

This unit of competency applies to field operators who are responsible for a number of wells and their associated systems. In a typical scenario, the operator will be driving alone, on and off roads between wells and also to and from the base site or plant. While at a site, they will be monitoring well and equipment performance by taking readings, making checks, and recording and reporting their findings in accordance with procedures. They will also be expected to identify hazards and take appropriate action.

This unit of competency applies to an individual who may work alone although under routine direction and supervision. They may work as part of a team or group and will work in liaison with other shift team members and the control room operator, as appropriate.

No licensing, legislative or certification requirements apply to this unit at the time of publication.

# Pre-requisite Unit

Nil

# Competency Field

Operations

# Unit Sector

# Elements and Performance Criteria

| Elements describe the essential outcomes. |  | Performance criteria describe the performance needed to demonstrate achievement of the element. |  |
| --- | --- | --- | --- |
| 1 | Prepare for work | 1.1 | Receive and give shift handover |
|  |  | 1.2 | Identify work requirements |
|  |  | 1.3 | Identify and control hazards |
|  |  | 1.4 | Coordinate with appropriate personnel |
|  |  | 1.5 | Check for recent work undertaken on wells/gathering systems |
|  |  | 1.6 | Note any outstanding/incomplete work |
|  |  | 1.7 | Check operational status of wells/gathering systems |
|  |  | 1.8 | Determine appropriate route/schedule for day’s work |
| 2 | Complete site equipment checks | 2.1 | Check equipment condition and operation |
|  |  | 2.2 | Check required levels |
|  |  | 2.3 | Top up levels as required |
|  |  | 2.4 | Complete logs and reports as required |
| 3 | Use well control systems as required | 3.1 | Check well control systems validity |
|  |  | 3.2 | Perform other required well control system tasks |
|  |  | 3.3 | Complete logs and reports as required |
| 4 | Take required readings | 4.1 | Complete all required readings for site |
|  |  | 4.2 | Compare all read values with the desired range |
|  |  | 4.3 | Compare read values with previous log sheet values |
|  |  | 4.4 | Complete logs and reports as required |
| 5 | Complete required lease maintenance | 5.1 | Inspect lease area for items requiring action |
|  |  | 5.2 | Complete required lease maintenance actions |
|  |  | 5.3 | Complete logs and reports as required |
| 6 | Finalise shift activities | 6.1 | Complete shift tasks as appropriate |
|  |  | 6.2 | Ensure identified faults are correctly logged/reported for action |
|  |  | 6.3 | Ensure incomplete tasks are scheduled for follow-up |
|  |  | 6.4 | Ensure all logs and reports are complete and understood |

# Foundation Skills

This section describes those language, literacy, numeracy and employment skills that are essential to performance.

Foundation skills essential to performance are explicit in the performance criteria of this unit of competency.

# Range of Conditions

| This field allows for different work environments and conditions that may affect performance. Essential operating conditions that may be present (depending on the work situation, needs of the candidate, accessibility of the item, and local industry and regional contexts) are included. |  |
| --- | --- |
| Regulatory framework | The latest version of all legislation, regulations, industry codes of practice and Australian/international standards, or the version specified by the local regulatory authority, must be used, and include one or more of the following: legislative requirements, including work health and safety (WHS) industry codes of practice and guidelines environmental regulations and guidelines Australian and other standards licence and certification requirements All operations to which this unit applies are subject to stringent health, safety and environment (HSE) requirements, which may be imposed through state/territory or federal legislation, and these must not be compromised at any time. Where there is an apparent conflict between performance criteria and HSE requirements, the HSE requirements take precedence. |
| Procedures | All operations must be performed in accordance with relevant procedures. Procedures are written, verbal, visual, computer-based or in some other form, and include one or more of the following: emergency procedures work instructions standard operating procedures (SOPs) safe work method statements (SWMS) formulas/recipes batch sheets temporary instructions any similar instructions provided for the smooth running of the plant |
| Hazards | Hazards include one or more of the following: electricity gas gases and liquids under pressure structural hazards structural collapse equipment failures industrial (machinery, equipment and product) equipment or product mass noise, rotational equipment or vibration plant services (steam, condensate and cooling water) working at heights, in restricted or confined spaces, or in environments subjected to heat, dusts or vapours flammability and explosivity hazardous products and materials unauthorised personnel sharp edges, protrusions or obstructions slippery surfaces, spills or leaks extreme weather other hazards that might arise |
| Routine problems | Routine problems must be resolved by applying known solutions. Routine problems are predictable and include one or more of the following: leakage solids (formation fines) vibration loss of control of pressure and/or flow hydrate formation and blockages liquid slugging corrosion erosion sulphate reducing bacteria scale formation equipment failure Known solutions are drawn from one or more of the following: procedures training remembered experience Non-routine problems must be reported according to according to relevant procedures. |
| Site | Site includes one or more of the following: a well a nominated area in the gathering system another location where the operator is required to work |
| Site equipment | Site equipment includes one or more of the following: wellheads choke and control valves meters flow lines high point vents low point drains valves, including non-return and pressure/vacuum relief pumps, pump speed and their prime movers product separation units instrumentation and control systems (variable speed drive (VSD) and proportional integral derivative (PID)) testing equipment power units drive heads flares chemical injection equipment storage tanks/vessels autodumps leaks |
| Levels | Levels include, as appropriate to the site, one or more of the following: chemical storage levels lubricating oil levels water and gas levels battery levels drain levels |
| Lease maintenance areas requiring checking | Lease maintenance areas requiring checking include one or more of the following: land erosion fence and gate integrity weeds and other growth actions of feral or other fauna other required items |
| Identified faults | Identified faults include one or more of the following: instrumentation failure/malfunction electrical failure/malfunction mechanical failure/malfunction control system failure/malfunction mismatch between flow rates and system requirements wear, tear and corrosion of plant and equipment |

# Unit Mapping Information

Release 1. Supersedes and is equivalent to PMAOPS233A Monitor wells and gathering systems

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=9fc2cf53-e570-4e9f-ad6a-b228ffdb6875