# Assessment Requirements for PMAOPS347 Create and conduct isolations in the workplace

Release: 1

Assessment Requirements for PMAOPS347 Create and conduct isolations in the workplace

# Modification History

Release 1. This version was released with PMA Chemical, Hydrocarbons and Refining Training Package, Release 2.0.

# Performance Evidence

There must be evidence the candidate has completed the tasks outlined in the elements and performance criteria of this unit, and:

at least 2 times, each in a separate work context.

# Knowledge Evidence

There must be evidence the candidate has knowledge of:

physical plant and equipment including:

the equipment/process being isolated

energy sources relevant to plant equipment

policies and procedures including:

methods of creating isolations

methods of de-energising and isolating energy sources

the scope/boundaries of own role when creating and conducting the isolations

safety, emergency and hazard control

work permit systems

risk control measures and procedures

hierarchy of control

isolation requirements

methods of purging, venting, bleeding, draining and flushing

isolation chain of custody and handover protocols

communications

environmental management including controlling releases

duty of care requirements

standard operating procedures (SOPs)

work instructions

safe work method statements (SWMS)

document control

operational parameters including:

methods and equipment used for draining and purging to attain zero energy

decontamination methods and requirements for various materials and situations

significance of time allowed for draining, purging and ventilation

fluid dynamics relating specifically to draining piping systems, including:

the ability of a liquid to ‘hang-up’ in pipework

the importance of identifying high-point vents to release gas/vapours and low point drains to release liquids

how to determine the amount of liquid drained from a piping section to ascertain/prove that draining process has been effective

the potential effects of creating a vacuum by not draining correctly (e.g. by pulling a vacuum behind a slug of liquid)

the ALARP (as low as reasonably practicable) principle

importance of appropriate rates of change for pressure and temperature for vessels and other plant equipment

risk management including:

the chemical and physical properties of the energy sources being isolated including one or more of the following:

electricity (mains, solar and by generator)

chemicals and fuels

heat and steam

pressure, such as compressed air and water, hydraulic oil and other fluids under pressure

energy-storing devices such as batteries, springs, flywheels, accumulators and capacitors.

trapped pressure

site requirements in completion of bleed logs.

# Assessment Conditions

Skills must have been demonstrated in the workplace or in a simulated environment that reflects workplace conditions and contingencies. The following conditions must be met for this unit:

use of suitable facilities, equipment and resources, including:

real industrial equipment and its associated energy sources

operating procedures

paperwork must include:

isolation register(s)

lock-out/tag-out register(s)

appropriate sign-offs

any reports, permits/work packs, and documentation required by the job/organisation

filing of documentation in accordance with workplace protocols.

Assessors must satisfy the NVR/AQTF mandatory competency requirements for assessors.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=9fc2cf53-e570-4e9f-ad6a-b228ffdb6875