# Assessment Requirements for PMBPROD242E Bond polymers to surfaces

Release: 1

Assessment Requirements for PMBPROD242E Bond polymers to surfaces

# Modification History

Release 1. Unit code and title changed. Application changed. Elements and Performance Criteria changed. Range of Conditions removed. Assessment Requirements changed. Supersedes and is equivalent to PMBPROD242 Bond polymers to surfaces.

# Performance Evidence

There must be evidence the candidate has completed the tasks outlined in the elements and performance criteria of this unit, and demonstrated the ability to:

bond polymers to surfaces at least once to meet job specifications, and:

check substrate suitability, including:

surface soundness

surface preparation

substrate porosity

surface pits, protrusions or other irregularities

prepare joining material, including one or more of:

mixing chemicals or solutions

blending two (or more) pot-material components

thinning (e.g. with solvent) if required

measuring components for blends

# Knowledge Evidence

There must be evidence the candidate has knowledge of:

principles of polymer bonding

production workflow sequences and materials demand

types of adhesives, cohesives, solvents and their differences and range of applications

joining behaviour of polymers to relevant substrates and the role of adhesives, cohesives, solvents and other materials

criteria for confirming pre-operational checks

types and application of common joining processes

types of tools and equipment typically used, including one or more of the following:

brushes

syringes

application scrapers

expected quality standards resulting from the bonding process

key variables that may affect product quality and production output, including:

application patterns

adhesive consistency

adhesive viscosity

solvent/thinner blend ratios

drying/setting times

pressure/contact between surfaces

typical process problems, including early warning signs, possible causes and corrective actions

organisational procedures relevant to the work environment and job role

hazards that may arise in the job or work environment, and:

their possible causes

potential consequences

appropriate risk controls.

# Assessment Conditions

Skills must have been demonstrated in the workplace or in a simulated environment that reflects workplace conditions and contingencies.

Assessors must satisfy the NVR/AQTF mandatory competency requirements for assessors.

# Links

Companion Volume implementation guides are found in VETNet – - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=932aacef-7947-4c80-acc6-593719fe4090