# Assessment Requirements for PMBPROD251E Apply gel coat or other polymer surface finish

Release: 1

Assessment Requirements for PMBPROD251E Apply gel coat or other polymer surface finish

# Modification History

Release 1. Unit code changed. Application changed. Elements and Performance Criteria changed. Range of Conditions removed. Assessment Requirements changed. Supersedes and is equivalent to PMBPROD251 Apply gel coat or other polymer surface finish.

# Performance Evidence

There must be evidence the candidate has completed the tasks outlined in the elements and performance criteria of this unit, and demonstrated the ability to:

apply gel coat or other polymer surface finish at least once to meet job specifications.

# Knowledge Evidence

There must be evidence the candidate has knowledge of:

the function of gel coat in the composite products being manufactured

the importance of gel coat application to the standard of the finished work

the common types of coatings, their differences and range of applications

equipment commonly used in the application of polymer surface finishes, including:

measurement devices including depth gauges, containers, weighing machines, scales and meters

hand application equipment including brushes

spray equipment

the methods of application of coatings

processing behaviour and the role of additives

the criteria for overall process/equipment readiness for the process or equipment being used

polymer surface finish workflow sequences and materials demand

key variables that may affect product quality and production output, including:

thickness

temperature

air pressure

surface finish

condition of mould/surface

product integrity and general conformance to specification

the potential effects of variations in raw materials, conditions and equipment in relation to quality of product

typical process and product problems, as listed below, including early warning signs, possible causes and corrective actions:

wrinkles and pinholes

slow gel coat cure

sagging of the gel coat

porosity

separation of colour

discolouration

spray blockage, build-up, wear or breakage

mixing problems

temperature variation

catalysation

incomplete cure

organisational procedures relevant to the work environment or job role

hazards that may arise in the job or work environment, and:

their possible causes

potential consequences

appropriate risk controls.

# Assessment Conditions

Skills must have been demonstrated in the workplace or in a simulated environment that reflects workplace conditions and contingencies.

Assessors must satisfy the NVR/AQTF mandatory competency requirements for assessors.

# Links

Companion Volume implementation guides are found in VETNet – - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=932aacef-7947-4c80-acc6-593719fe4090