# PMBPROD283E Demould polymer products

Release: 1

PMBPROD283E Demould polymer products

# Modification History

Release 1. Unit code and title changed. Application changed. Elements and Performance Criteria changed. Range of Conditions removed. Assessment Requirements changed. Supersedes and is equivalent to PMBPROD283 Demould product.

# Application

This unit describes the skills and knowledge required to remove polymer products from moulds typically used in the rotational moulding, urethane foam, thermoforming and composites sectors.

This unit applies to an individual working alone or as part of a team.

The unit principally refers to customised demoulding operations where there may be significant manual handling of moulds and products, which requires attention to safe working and care of the mould surfaces. It is not applicable to injection moulding.

No licensing or certification requirements exist at the time of publication. Relevant legislation, industry standards and codes of practice within Australia must be applied.

# Pre-requisite Unit

Nil

# Competency Field

Production – Polyurethane, Rotational Moulding, Rubber Processing, Thermoforming

# Elements and Performance Criteria

| Elements Elements describe the essential outcomes. | Performance Criteria Performance criteria describe the performance needed to demonstrate achievement of the element. |
| --- | --- |
| 1. Check work requirements | 1.1 Identify requirements from job specifications 1.2 Perform housekeeping tasks to prepare for work to be undertaken 1.3 Perform pre-operational checks including checking for and acting on hazards |
| 2. Demould product | 2.1 Open mould according to mould instructions 2.2 Remove product according to mould instructions 2.3 Clean surface of mould and apply release agent in readiness for next operation 2.4 Monitor and maintain supply of materials to support ongoing operation of the mould 2.5 Accurately complete logs and records 2.6 Apply waste management strategies to collect and segregate scrap, trim and other materials according to workplace procedures 2.7 Clean equipment and work area according to equipment specifications and workplace procedures |
| 3. Respond to process control variances | 3.1 Recognise and log process control variances and their causes 3.2 Determine, prioritise and implement remedial and preventative actions based on identified issues and causes 3.3 Accurately maintain records and logbooks |

# Foundation Skills

This section describes those language, literacy, numeracy and employment skills that are essential to performance.

Numeracy skills to interpret technical information.

Reading skills to interpret specifications and workplace procedures.

Writing and oral communication skills to record and report on polymer demoulding operations.

Other foundation skills essential to performance are explicit in the performance criteria of this unit of competency.

# Unit Mapping Information

Supersedes and is equivalent to PMBPROD283 Demould product.

# Links

Companion Volume implementation guides are found in VETNet – - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=932aacef-7947-4c80-acc6-593719fe4090

Assessment Requirements for PMBPROD283E Demould polymer products

# Modification History

Release 1. Unit code and title changed. Application changed. Elements and Performance Criteria changed. Range of Conditions removed. Assessment Requirements changed. Supersedes and is equivalent to PMBPROD283 Demould product.

# Performance Evidence

There must be evidence the candidate has completed the tasks outlined in the elements and performance criteria of this unit, and demonstrated the ability to:

demould polymer products at least once to meet job specifications.

# Knowledge Evidence

There must be evidence the candidate has knowledge of:

types and application of mould parts and assemblies and attachments

types and role of releasing agents that are being used in the process

typical tools and equipment used in the demoulding process, including:

applicators

dispensers

measuring devices

cutters

cleaning utensils

powered equipment

specialised demoulding techniques to suit the actual production technology

the criteria for overall process or equipment readiness for the process or equipment being used

production workflow sequences

key variables that may affect product quality and production output

typical process and product problems, including early warning signs, possible causes and corrective actions

organisational procedures relevant to the work environment or job role

hazards that may arise in the job or work environment, and:

their possible causes

potential consequences

appropriate risk controls.

# Assessment Conditions

Skills must have been demonstrated in the workplace or in a simulated environment that reflects workplace conditions and contingencies.

Assessors must satisfy the NVR/AQTF mandatory competency requirements for assessors.

# Links

Companion Volume implementation guides are found in VETNet – - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=932aacef-7947-4c80-acc6-593719fe4090