# Assessment Requirements for PMBPROD287E Weld thermoplastics materials

Release: 1

Assessment Requirements for PMBPROD287E Weld thermoplastics materials

# Modification History

Release 1. Unit code and title changed. Application changed. Elements and Performance Criteria changed. Range of Conditions removed. Assessment Requirements changed. Supersedes and is equivalent to PMBPROD287 Weld plastics materials.

# Performance Evidence

There must be evidence the candidate has completed the tasks outlined in the elements and performance criteria of this unit, and demonstrated the ability to:

weld thermoplastic materials at least once to meet job specifications.

# Knowledge Evidence

There must be evidence the candidate has knowledge of:

types of plastics materials commonly used in the sector or work environment and their behaviour when prepared and welded

types and functions of welding equipment, and their related procedures, including:

butt fusion welding

electro fusion welding

extrusion welding

heated wedge welding

high speed nozzle

hot gas welding

infrared welding

socket fusion welding

suitability of use of welding equipment for different materials

types of welds and how to achieve them

production workflow sequences and materials demand

quality standards at each stage of production

welding symbols

key variables that may affect product quality and production output, including:

weld temperature

feed rate of filler rod

variations in materials being welded

thickness of materials being welded

typical process and product problems, including early warning signs, possible causes and corrective actions:

incorrect patterns or inaccurate assembly or clamping

incorrect filler rod selection

poor surface preparation

incorrect selection of welding nozzle or attachments

introduction of moisture to welding devices

sequencing problems

bonding problems

incomplete weld deposit

plastication not achieved

organisational procedures relevant to the work environment or job role, including procedures for the emergency shutdown of equipment

hazards that may arise in the job or work environment, and:

their possible causes

potential consequences

appropriate risk controls.

# Assessment Conditions

Skills must have been demonstrated in the workplace or in a simulated environment that reflects workplace conditions and contingencies.

Assessors must satisfy the NVR/AQTF mandatory competency requirements for assessors.

# Links

Companion Volume implementation guides are found in VETNet – - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=932aacef-7947-4c80-acc6-593719fe4090