# PMBWELD310 Design PVC plastic pressure pipelines

Release: 1

PMBWELD310 Design PVC plastic pressure pipelines

# Modification History

Release 1. Supersedes and is equivalent to PMBWELD310B Design PVC plastic pressure pipelines

# Application

This unit of competency covers the skills and knowledge required to design polyvinylchloride (PVC) plastics pressure pipes and pipeline components under industrial conditions in the field. Pipelines may be used for transmitting gas and liquids.

This unit of competency applies to experienced operators who are required to select suitable materials, components and designs for specific pressure applications of PVC pipes and pipelines to quality assurance requirements, including specifying appropriate testing and commissioning procedures.

This unit of competency applies to an experienced operator demonstrating theoretical and technical knowledge and well developed skills in situations that require some discretion and judgement. The operator may work alone or as a member of a team or group and will work in liaison with other shift team members, team leader and supervisor, as appropriate.

No licensing, legislative or certification requirements apply to this unit at the time of publication.

# Pre-requisite Unit

| PMBWELD308 | Install PVC plastic pressure pipelines |
| --- | --- |

# Competency Field

Welding

# Unit Sector

Not applicable

# Elements and Performance Criteria

| Elements describe the essential outcomes. |  | Performance criteria describe the performance needed to demonstrate achievement of the element. |  |
| --- | --- | --- | --- |
| 1 | Identify PVC pipe materials as being suitable for specific pressure applications | 1.1 | Identify suitable materials as PVC grades and variations from national standards, job specifications and work site instructions |
|  |  | 1.2 | Determine PVC materials and properties from national standards and material data sheets |
|  |  | 1.3 | Identify job needs from work site instructions and specifications |
| 2 | Determine appropriate products from national standards | 2.1 | Identify material options from national standards, supplier data sheets, and government codes and regulations |
|  |  | 2.2 | Identify performance limitations of material options |
|  |  | 2.3 | Identify pipe and component options from national standards, supplier data sheets, and government codes and regulations |
|  |  | 2.4 | Identify performance limitations pipe and component options |
|  |  | 2.5 | Identify suitable jointing methods for pipes and fittings |
|  |  | 2.6 | Identify operational conditions |
| 3 | Determine pipeline requirements | 3.1 | Identify length of pipe required |
|  |  | 3.2 | Identify fittings and number of fittings required |
|  |  | 3.3 | Select pipe and components considering specific job needs and product properties |
|  |  | 3.4 | Identify required jointing equipment or materials |
|  |  | 3.5 | Prepare field operational sheets |
| 4 | Determine appropriate testing and commissioning procedures | 4.1 | Determine specific installation test requirements |
|  |  | 4.2 | Identify alternative test procedures to meet requirements |
|  |  | 4.3 | Maintain quality records |

# Foundation Skills

This section describes those required skills (language, literacy and numeracy) that are essential to performance.

Foundation skills essential to performance are explicit in the performance criteria of this unit of competency.

# Range of Conditions

| This field allows for different work environments and conditions that may affect performance. Essential operating conditions that may be present (depending on the work situation, needs of the candidate, accessibility of the item, and local industry and regional contexts) are included. |  |
| --- | --- |
| Regulatory framework | The latest version of all legislation, regulations, industry codes of practice and Australian/international standards, or the version specified by the local regulatory authority, must be used. Applicable legislation, regulations, standards and codes of practice include: health, safety and environmental (HSE) legislation, regulations and codes of practice relevant to the workplace, manual handling and hazardous materials Australian/international standards relevant to the materials being used and products being made, including one or more of: AS/NZS 1477:2006 PVC pipes and fittings for pressure applications or its replacement AS/NZS 4441:2008 Oriented PVC (PVC-O) pipes for pressure applications or its replacement AS/NZS 4765:2007 Modified PVC (PVC-M) pipes for pressure applications or its replacement AS/NZS 2566.2:2002 Buried flexible pipelines - Installation or its replacement AS/NZS 2032:2006 Installation of PVC pipe systems or its replacement any relevant licence and certification requirements. All operations to which this unit applies are subject to stringent HSE requirements, which may be imposed through state/territory or federal legislation, and these must not be compromised at any time. Where there is an apparent conflict between performance criteria and such requirements the legislative requirements take precedence. |
| PVC pressure pipeline materials | PVC pressure pipeline materials include one or more of the following: unplasticised polyvinylchloride (UPVC) modified polyvinylchloride (MPVC) oriented polyvinylchloride (OPVC). |
| Joining methods | Joining methods for installers include one or more of: mechanical chemical bond thermal fusion. |
| Installation | Installation can be one or more of: above ground below ground. |
| Procedures | All operations must be performed in accordance with relevant procedures. Procedures are written, verbal, visual, computer-based or in some other form, and include one or any combination of: emergency procedures work instructions standard operating procedures (SOPs) safe work method statements (SWMS) formulas/recipes batch sheets temporary instructions any similar instructions provided for the smooth running of the plant. |
| Problems | Problems include one or more of: limitations on the available PE materials, pipes and components variable field site conditions limitations in available data on specific job requirements emergency situations |

# Unit Mapping Information

Release 1. Supersedes and is equivalent to PMBWELD310B Design PVC plastic pressure pipelines

# Links

MSA Training Package Implementation Guides - http://mskills.org.au/training-packages/info/