# PMC552092 Prepare for and install mouldable refractory materials

Release: 1

PMC552092 Prepare for and install mouldable refractory materials

# Modification History

Release 1. Supersedes and is equivalent to PMC552092B Prepare for and install mouldable refractory materials

# Application

This unit of competency covers the skills and knowledge required to prepare for and install moulded refractory. It applies to installation with moulded refractory alone and with moulded refractory that needs to join to brick/block or cast refractory.

This unit of competency applies to operators who are required to prepare for and carry out installation and repairs to procedures and specifications, mix and apply refractory mortars, correct any deficiencies in the quality of work and identify and take action on any problems.

This unit of competency applies to an individual working alone or as part of a team or group and working in liaison with other shift team members, team leader and supervisor, as appropriate.

No licensing, legislative or certification requirements apply to this unit at the time of publication.

# Pre-requisite Unit

Nil

# Competency Field

Operations

# Unit Sector

Not applicable

# Elements and Performance Criteria

| Elements describe the essential outcomes. |  | Performance criteria describe the performance needed to demonstrate achievement of the element. |  |
| --- | --- | --- | --- |
| 1 | Establish the suitability of resources | 1.1 | Check all information conforms with resources and pneumatic ramming equipment |
| 2 | Prepare for and install mouldable refractory materials | 2.1 | Prepare backgrounds, such as cutting out, demolition and keying |
|  |  | 2.2 | Organise work to meet work deadlines |
|  |  | 2.3 | Install refractory to appropriate procedures |
|  |  | 2.4 | Carry out remedial work to meet the specifications |
|  |  | 2.5 | Inform the person in charge that installation and any remedial work have been completed |
|  |  | 2.6 | Store tools and equipment following organisational requirements for current legislation and official guidance |
| 3 | Contribute to controlling hazards in work area | 3.1 | Identify hazards in work area |
|  |  | 3.2 | Assess risks arising from those hazards |
|  |  | 3.3 | Take appropriate action to control risks to procedures and duty of care |
| 4 | Respond to problems | 4.1 | Identify possible problems in equipment or process |
|  |  | 4.2 | Determine problems needing action |
|  |  | 4.3 | Determine possible fault causes |
|  |  | 4.4 | Rectify problem using appropriate solution within area of responsibility |
|  |  | 4.5 | Follow through items initiated until final resolution has occurred |
|  |  | 4.6 | Report problems outside area of responsibility to designated person |

# Foundation Skills

This section describes those required skills (language, literacy and numeracy) that are essential to performance.

Foundation skills essential to performance are explicit in the performance criteria of this unit of competency.

# Range of Conditions

| This field allows for different work environments and conditions that may affect performance. Essential operating conditions that may be present (depending on the work situation, needs of the candidate, accessibility of the item, and local industry and regional contexts) are included. |  |
| --- | --- |
| Regulatory framework | The latest version of all legislation, regulations, industry codes of practice and Australian/international standards, or the version specified by the local regulatory authority, must be used. Applicable legislation, regulations, standards and codes of practice include: health, safety and environmental (HSE) legislation, regulations and codes of practice relevant to the workplace, equipment and production processes and hazardous materials Australian/international standards relevant to the materials being used and products being made any relevant licence and certification requirements. All operations to which this unit applies are subject to stringent HSE requirements, which may be imposed through state/territory or federal legislation, and these must not be compromised at any time. Where there is an apparent conflict between performance criteria and such requirements the legislative requirements take precedence. |
| Procedures | All operations must be performed in accordance with relevant procedures. Procedures are written, verbal, visual, computer-based or in some other form, and include one or any combination of: manufacturer’s technical information job cards drawings emergency procedures work instructions standard operating procedures (SOPs) safe work method statements (SWMS) formulas/recipes batch sheets temporary instructions any similar instructions provided for the smooth running of the plant. |
| Problems | Routine problems must be resolved by applying known solutions. Routine problems are predictable and include one or more of: damaged/worn tools/equipment incorrect tools/equipment for job incorrect/incomplete information for job variations in materials preparation problems. Known solutions are drawn from one or more of: procedures training remembered experience. Non-routine problems must be reported according to according to relevant procedures. |
| Tools and equipment | Tools and equipment include: moulding tools refractory mixing equipment lifting and placing equipment equipment allowing access. |
| Hazards | Hazards must be identified and controlled. Identifying hazards requires consideration of: heat, smoke, dust, vapours or other atmospheric hazards weight, shape, volume of materials to be handled hazardous products and materials sharp edges, protrusions or obstructions slippery surfaces, spills or leaks electricity gas gases and liquids under pressure noise rotational equipment or vibration plant services (steam, condensate, cooling water, etc) structural hazards equipment failures machinery, equipment and product mass limited head spaces or overhangs working at heights working in restricted or confined spaces other hazards that might arise. |
| Records and reports | Records include one or more of: log books/sheets electronic records job/work sheets other records used for the smooth running of the plant. Reports include one or more of: paper or electronic-based logs and reports verbal/radio reports reporting items found which require action. |

# Unit Mapping Information

Release 1. Supersedes and is equivalent to PMC552092B Prepare for and install mouldable refractory materials

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=45a7f1d5-61a5-447a-9688-7abbd7e1a5c7

Assessment Requirements for PMC552092 Prepare for and install mouldable refractory materials

# Modification History

Release 1. Supersedes and is equivalent to PMC552092B Prepare for and install mouldable refractory materials

# Performance Evidence

Evidence required to demonstrate competence in this unit must be relevant to and satisfy the requirements of the elements and performance criteria and demonstrate the ability to:

read and interpret manufacturer’s technical information, job sheets, procedures, material labels and safety information

confirm information/procedures are available and conform with resources

identify and select materials, tools and equipment

use and maintain tools and equipment

prepare and install mouldable refractory to achieve required specifications

carry out remedial work to meet the specifications

monitor key variables, including:

moulding to shape

correct thickness/cover

conforming to required curves and angles while maintain cover

tying mouldable to structure

keying/bonding to existing refractory

cutting requirements

support requirements

monitor installation integrity and conformance to specification and make adjustments to meet specifications

recognise routine and non-routine problems

identify when the operator is able to rectify problems, when assistance is required and who is the appropriate source for assistance

take action to resolve problems or report them to appropriate personnel

distinguish between possible causes of routine problems, including:

raw material variations

equipment faults/abnormalities

process/procedure errors

identify hazards and apply relevant hazard controls

apply safety procedures

communicate effectively with team/work group and supervisors

complete workplace records

do basic arithmetical manipulations, including additions, subtractions, divisions, fractions and percentages.

# Knowledge Evidence

Must provide evidence that demonstrates knowledge relevant to their job sufficient to operate under routine only supervision and to solve routine problems, including knowledge of:

types of materials used in mouldable refractory installation and/or repair and:

their application

preparation methods

finishing methods

types and characteristics of background surfaces and their preparation methods

types, uses and operation of tools and equipment

methods of compacting mouldable materials

effects of under- or over-ramming mouldable materials

reasons for anchors and methods of ensuring compaction of mouldable refractory around the anchors

types and methods of finishing the surface of the installed mouldables

types and uses of temporary support/formwork

methods of protecting work during installation

reasons for, and methods of, providing test panels

methods of cutting

safeguards to take during reinstatement work

methods of removing damaged refractory

methods of keying and bonding new to existing refractories

reasons for, and methods of, obtaining seals between new and existing refractory

routine problems, their probable causes and relevant corrective actions

organisation procedures relevant to the work environment/job role

hierarchy of control

hazards that may arise in the job/work environment and:

their possible causes

potential consequences

appropriate risk controls.

# Assessment Conditions

The unit should be assessed holistically and the judgement of competence shall be based on a holistic assessment of the evidence.

The collection of performance evidence:

should occur over a range of situations which include typical disruptions to normal, smooth operation of an operating plant

will typically include a supervisor/third-party report or other evidence, focusing on consistent performance and problem recognition and solving. A supervisor/third-party report must be prepared by someone who has a direct, relevant, current relationship with the person being assessed and who is in a position to form a judgement on workplace performance relevant to the unit of competency

must include the use of an appropriate industrial item/s of equipment requiring demonstration of operation, start-up and shutdown procedures and responding to problems

may use industry-based simulation for all or part of the unit particularly where safety, lack of opportunity or significant cost is an issue.

Assessment should occur in operational workplace situations. Where this is not possible or where personal safety or environmental damage are limiting factors assessment must occur in a sufficiently rigorous simulated environment that reflects realistic operational workplace conditions. This must cover all aspects of workplace performance, including environment, task skills, task management skills, contingency management skills and job role environment skills.

Assessment in a simulated environment should use evidence collected from demonstration of skills and one or more of:

walk-throughs

pilot plant operation

industry-based case studies/scenarios

‘what ifs’.

Knowledge evidence may be collected concurrently with performance evidence (provided a record is kept) or through an independent process such as workbooks, written assessments or interviews (provided a record is kept).

Assessment processes and techniques must be appropriate to the language, literacy and numeracy requirements of the work being performed and the needs of the candidate.

Conditions for assessment must include access to all tools, equipment, materials and documentation required, including relevant workplace procedures, product and manufacturing specifications associated with this unit.

The regulatory framework will be reflected in workplace policies and procedures and is not required to be independently assessed.

Foundation skills are integral to competent performance of the unit and should not be assessed separately.

Assessors must satisfy the assessor competency requirements that are in place at the time of the assessment as set by the VET regulator.

In addition the assessor or anyone acting in subject matter expert role in assessment shall demonstrate both technical competency and currency. If the assessor cannot demonstrate technical competency and currency they shall assess with a subject matter expert who does meet these requirements.

Technical competence can be demonstrated through one or more of:

relevant VET or other qualification/Statement of Attainment

appropriate workplace experience undertaking the type of work being assessed under routine and non-routine conditions

appropriate workplace experience supervising/evaluating the type of work being assessed under routine and non-routine conditions

Currency can be demonstrated through one or more of:

being currently employed undertaking the type of work being assessed

being employed by the organisation undertaking the type of work being assessed and having maintained currency in accordance with that organisation’s policies and procedures

having consulted/had contact with an organisation undertaking the type of work being assessed within the last twelve months, the consultation/contact being related to assessment

conducting on the job training/assessments of the type of work being assessed

being an active member of a relevant professional body and participating in activities relevant to the assessment of this type of work.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=45a7f1d5-61a5-447a-9688-7abbd7e1a5c7