# Assessment Requirements for PMC552094 Prepare for and apply shotcrete for installation

Release: 1

Assessment Requirements for PMC552094 Prepare for and apply shotcrete for installation

# Modification History

Release 1. Supersedes and is equivalent to PMC552094B Prepare for and apply shotcrete for installation

# Performance Evidence

Evidence required to demonstrate competence in this unit must be relevant to and satisfy the requirements of the elements and performance criteria and demonstrate the ability to:

read and interpret manufacturer's technical information, job sheets, procedures, material labels and safety information

confirm information/procedures are available and conform with resources

identify and select materials, components, tools and equipment

use and maintain tools and equipment

mix and apply shotcrete to achieve required specifications

monitor key variables, including:

uniformity and thickness of course

consistency of mix

correct compaction

conforming to required curves and angles

adhesion to surface

tying to structure

keying to existing refractory

cutting requirements

support requirements

monitor installation integrity and conformance to specification and make adjustments to meet specifications

recognise routine and non-routine problems

identify when the operator is able to rectify problems, when assistance is required and who is the appropriate source for assistance

take action to resolve problems or report them to appropriate personnel

distinguish between possible causes of routine problems, including:

raw material variations

equipment faults/abnormalities

process/procedure errors

identify hazards and apply relevant hazard controls

apply safety procedures

communicate effectively with team/work group and supervisors

complete workplace records

do basic arithmetical manipulations, including additions, subtractions, divisions, fractions and percentages.

# Knowledge Evidence

Must provide evidence that demonstrates knowledge relevant to their job sufficient to operate under routine only supervision and to solve routine problems, including knowledge of:

types, uses and operation of shotcreting equipment

types and characteristics of shotcreting materials

methods of mixing and applying shotcreting materials

methods of avoiding lamination of applied shotcreting materials

methods of protecting applied shotcreting material during application and curing

types and methods of finishing the surface of the shotcrete

methods of curing shotcrete

types and characteristics of background surfaces and their preparation methods

types, uses and operation of tools and equipment

methods of preparing background surfaces

types, uses and operation of tools and equipment

reasons for, and timing of, dedicated curing and heat-up programs and the consequences of the installed concrete if the program is not correctly followed

types and uses of temporary support/formwork

methods of protecting work during installation

reasons for, and methods of, providing test panels

methods of cutting

safeguards to take during reinstatement

methods of removing damaged refractory

methods of keying and bonding new to existing refractories

reasons for, and methods of, obtaining seals between new and existing refractory

routine problems, their probable causes and relevant corrective actions

organisation procedures relevant to the work environment/job role

hierarchy of control

hazards that may arise in the job/work environment and:

their possible causes

potential consequences

appropriate risk controls.

# Assessment Conditions

The unit should be assessed holistically and the judgement of competence shall be based on a holistic assessment of the evidence.

The collection of performance evidence:

should occur over a range of situations which include typical disruptions to normal, smooth operation of an operating plant

will typically include a supervisor/third-party report or other evidence, focusing on consistent performance and problem recognition and solving. A supervisor/third-party report must be prepared by someone who has a direct, relevant, current relationship with the person being assessed and who is in a position to form a judgement on workplace performance relevant to the unit of competency

must include the use of an appropriate industrial item/s of equipment requiring demonstration of operation, start-up and shutdown procedures and responding to problems

may use industry-based simulation for all or part of the unit particularly where safety, lack of opportunity or significant cost is an issue.

Assessment should occur in operational workplace situations. Where this is not possible or where personal safety or environmental damage are limiting factors assessment must occur in a sufficiently rigorous simulated environment that reflects realistic operational workplace conditions. This must cover all aspects of workplace performance, including environment, task skills, task management skills, contingency management skills and job role environment skills.

Assessment in a simulated environment should use evidence collected from demonstration of skills and one or more of:

walk-throughs

pilot plant operation

industry-based case studies/scenarios

‘what ifs’.

Knowledge evidence may be collected concurrently with performance evidence (provided a record is kept) or through an independent process such as workbooks, written assessments or interviews (provided a record is kept).

Assessment processes and techniques must be appropriate to the language, literacy and numeracy requirements of the work being performed and the needs of the candidate.

Conditions for assessment must include access to all tools, equipment, materials and documentation required, including relevant workplace procedures, product and manufacturing specifications associated with this unit.

The regulatory framework will be reflected in workplace policies and procedures and is not required to be independently assessed.

Foundation skills are integral to competent performance of the unit and should not be assessed separately.

Assessors must satisfy the assessor competency requirements that are in place at the time of the assessment as set by the VET regulator.

In addition the assessor or anyone acting in subject matter expert role in assessment shall demonstrate both technical competency and currency. If the assessor cannot demonstrate technical competency and currency they shall assess with a subject matter expert who does meet these requirements.

Technical competence can be demonstrated through one or more of:

relevant VET or other qualification/Statement of Attainment

appropriate workplace experience undertaking the type of work being assessed under routine and non-routine conditions

appropriate workplace experience supervising/evaluating the type of work being assessed under routine and non-routine conditions

Currency can be demonstrated through one or more of:

being currently employed undertaking the type of work being assessed

being employed by the organisation undertaking the type of work being assessed and having maintained currency in accordance with that organisation’s policies and procedures

having consulted/had contact with an organisation undertaking the type of work being assessed within the last twelve months, the consultation/contact being related to assessment

conducting on the job training/assessments of the type of work being assessed

being an active member of a relevant professional body and participating in activities relevant to the assessment of this type of work.

# Links

MSA Training Package Implementation Guides - http://mskills.org.au/training-packages/info/