# PPMPRV301 Co-ordinate and direct clothing changes

Release: 1

PPMPRV301 Co-ordinate and direct clothing changes

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with PPM Pulp and Paper Manufacturing Training Package Version 3.0. |

# Application

This unit of competency describes the outcomes required to co-ordinate and direct clothing changes to papermaking machinery to meet safety, quality and productivity requirements.

The unit applies to production operators who work in a paper manufacturing facility and are responsible for the replacement of scheduled changes or used clothing and filter fabrics for papermaking machinery.

No licensing, legislative or certification requirements apply to this unit at the time of publication.

# Pre-requisite Unit

Nil

# Unit Sector

Production Support (PRV)

# Elements and Performance Criteria

| Elements | Performance Criteria |
| --- | --- |
| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. |
| 1. Prepare machine and equipment for clothing change | 1.1 Identify machinery clothing requirements according to scheduled change or production issues caused by clothing problems 1.2 Identify workplace health and safety hazards, assess risks and determine control measures 1.3 Obtain new clothing to be installed 1.4 Communicate clothing preparation tasks and safety requirements to operators 1.5 Prepare equipment for isolation, and shut down equipment according to workplace health and safety and operating procedures |
| 2. Remove and install machine clothing | 2.1 Prepare and remove damaged or exhausted machine clothing according to workplace health and safety and operating procedures 2.2 Dismantle and remove machine parts 2.3 Install new machine clothing according to manufacturer specifications 2.4 Reassemble machine parts and replace as required 2.5 Remove isolations and establish and confirm operational settings to required specification |
| 3. Prepare machine section for production | 3.1 Inspect clothing section for potential hazards, and assess and control associated risks 3.2 Tension clothing to specification and to meet operational requirements 3.3 Check and adjust clothing guiding system to operational specification 3.4 Record and report clothing change process to appropriate personnel |

# Foundation Skills

This section describes those language, literacy, numeracy and employment skills that are essential for performance in this unit of competency but are not explicit in the performance criteria.

| Skill | Description |
| --- | --- |
| Reading | Interpret information from workplace procedures and documentation and manufacturer specifications |
| Writing | Complete records accurately and legibly using clear language and industry terminology |
| Oral communication | Provide clear, unambiguous instructions about requirements for clothing changes to operators |
| Numeracy | Interpret basic numerical settings on instruments and gauges, and set to operational specification |

# Unit Mapping Information

| Code and title current version | Code and title previous version | Comments | Equivalence status |
| --- | --- | --- | --- |
| PPMPRV301 Co-ordinate and direct clothing changes | PPMPRV320 Co-ordinate and direct clothing changes | Changes to performance criteria, foundation skills, performance evidence and knowledge evidence Assessment conditions updated | Not equivalent |

# Links

Companion Volumes, including Implementation Guides, are available at VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=12998f8d-d0ac-40bc-a69e-72a600d4fd93

Assessment Requirements for PPMPRV301 Co-ordinate and direct clothing changes

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with PPM Pulp and Paper Manufacturing Training Package Version 3.0. |

# Performance Evidence

An individual demonstrating competency must satisfy all of the elements and performance criteria in this unit.

There must be evidence that the individual has co-ordinated and directed at least two clothing changes for paper manufacturing equipment, one of which is based on fault change.

In completing the above, there must also be evidence that the individual has for each clothing change:

installed machine clothing according to manufacturer specifications

communicated effectively and worked safely with others when undertaking clothing changes

completed accurate basic records for clothing change processes.

# Knowledge Evidence

An individual must be able to demonstrate the knowledge required to perform the tasks outlined in the elements and performance criteria of this unit. This includes knowledge of:

methods used to assess clothing problems and associated production issues

typical faults with machinery clothing and effects on machine and process productivity:

end of life

score mark

holes and tears

creases

narrow

blind

scold marks

compacted

scorched

process stages and key control points undertaking clothing changes

operational specifications for:

tensioning clothing

clothing guiding systems

safe manual handling techniques for shifting machinery clothing

workplace procedures relevant to clothing changes:

standard operating procedures

workplace health and safety, equipment lock-out, manual handling and use of personal protective equipment

communication reporting lines

recording and reporting clothing change processes.

# Assessment Conditions

Assessment of the skills in this unit of competency must take place under the following conditions:

physical conditions:

skills must be demonstrated in a paper manufacturing facility or an environment that accurately represents workplace conditions

resources, equipment and materials:

papermaking machinery

equipment used to remove and install clothing and dismantle and reassemble machinery

replacement clothing

personal protective equipment suitable for completing clothing changes

proforma or recording system for recording clothing change processes

specifications:

workplace and standard operating procedures relating to machinery clothing changes, including health and safety, risks and hazards identification, plant isolation, quality, housekeeping and environmental requirements

maintenance schedule for machinery clothing changes

relationships:

relevant personnel for the purposes of communicating information.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volumes, including Implementation Guides, are available at VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=12998f8d-d0ac-40bc-a69e-72a600d4fd93