# RIIBHD303E Conduct long hole drilling

Release: 1

RIIBHD303E Conduct long hole drilling

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version first released with RII Resources and Infrastructure Industry Training Package Version 5.0. |

# Application

This unit describes the skills and knowledge required to conduct long hole drilling in metalliferous mining. It includes conducting drilling and monitoring operations and setting up and clearing drill sites.

It applies to those working in driller roles. They generally work under minimal supervision to undertake a broad range of skilled applications in varied work contexts, using some discretion and judgement in selecting equipment, services or contingency measures.

No licensing, legislative or certification requirements apply to this unit at the time of publication.

# Unit Sector

Metalliferous mining

# Elements and Performance Criteria

| ELEMENT | PERFORMANCE CRITERIA |
| --- | --- |
| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. |
| 1. Plan and prepare for long hole drilling | 1.1 Obtain, interpret, confirm and clarify shift change over details and review historical information 1.2 Access, interpret and apply drilling documentation and confirm work activity is compliant 1.3 Select and wear personal protective equipment according to site requirements 1.4 Inspect and assess site conditions, determine if scaling is required and respond according to site requirements 1.5 Install barricades and safety provisions according to site requirements 1.6 Check work site and identify and control misfires according to site requirements 1.7 Apply dust suppression and extraction methods according to site requirements 1.8 Conduct equipment prestart checks according to manufacturer specifications and site requirements 1.9 Transport, position, stabilise and connect equipment and attachments according to manufacturer specifications, drill plan and site requirements |
| 2. Drill and monitor progression | 2.1 Identify, manage and report hazards and potential risks according to site requirements 2.2 Carry out start-up procedures according to manufacturer specifications and site requirements 2.3 Drill holes and realign equipment according to manufacturer specifications, drill plan and site requirements 2.4 Monitor drilling operations and site conditions and adjust operations as required according to manufacturer specifications and site requirements 2.5 Remove drill cuttings and sludge 2.6 Mark misdirected holes according to site requirements 2.7 Carry out shutdown procedures according to manufacturer specifications and site requirements |
| 3. Carry out operator maintenance and housekeeping | 3.1 Carry out shutdown procedures and de-rig equipment according to manufacturer specifications and site requirements 3.2 Clear site and remove auxiliary services according to site requirements 3.3 Inspect and clean equipment and reports faults according to manufacturer specifications and site requirements 3.4 Complete documentation according to site requirements 3.5 Pass on end of shift information to oncoming shift according to site requirements |

# Foundation Skills

This section describes those language, literacy, numeracy and employment skills that are essential to performance but not explicit in the performance criteria.

| SKILL | DESCRIPTION |
| --- | --- |
| Numeracy | Calculates measurements, masses and dimensions |
| Reading | Identifies and interprets information from workplace documentation |
| Self-management | Monitors and minimises own exposure to worksite risks and hazards during activities |

# Unit Mapping Information

Supersedes and is equivalent to RIIBHD303D Conduct long hole drilling.

# Links

Companion Volume Implementation Guide is found on VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=88a61002-9a21-4386-aaf8-69c76e675272

Assessment Requirements for RIIBHD303E Conduct long hole drilling

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version first released with RII Resources and Infrastructure Industry Training Package Version 5.0. |

# Performance Evidence

The candidate must demonstrate the ability to complete the tasks outlined in the elements, performance criteria and foundation skills of this unit, including evidence of the ability to:

conduct long hole drilling on at least two occasions, including:

operating, maintaining, cleaning and storing auxiliary equipment

preparing site and equipment for drilling

interpreting drill plans, drilling holes and realigning equipment

carrying out shutdown procedures and de-rigging equipment.

During the above, the candidate must:

locate and apply relevant documentation and procedures and confirm work activity is compliant

works effectively with others to conduct long hole drilling that meets required outcomes, including:

organising work activities to meet task requirements

communicating clearly and concisely with others to receive and pass on changing of shift information.

# Knowledge Evidence

The candidate must be able to demonstrate knowledge to complete the tasks outlined in the elements, performance criteria and foundation skills of this unit, including knowledge of:

key procedures and documentation required to conduct long hole drilling, including:

auxiliary services procedures

drilling plan

drill system manufacturer specifications

equipment safety requirements

characteristics, technical capabilities and limitations of long role drill rigs, accessories and associated equipment, including:

prestart, start-up and shutdown procedures

routine operator service, maintenance and housekeeping procedures

techniques for dust suppression and extraction

techniques for interpreting and applying basic geological and technical data

principles and techniques for identifying and responding to relevant hazards and emergencies

techniques for coordinating job activities with others.

# Assessment Conditions

Mandatory conditions for assessment of this unit are stipulated below. The assessment must:

include access to:

personal protective equipment

equipment required to conduct long hole drilling

be conducted in a safe environment; and,

be assessed in the context of this sector’s work environment; and,

be assessed in compliance with relevant legislation/regulation and using policies, procedures and processes directly related to the industry sector for which it is being assessed; and,

confirm consistent performance can be applied in a range of relevant workplace circumstances.

Where personal safety or environmental damage are limiting factors, assessment may occur in a simulated work environment* provided it is realistic and sufficiently rigorous to cover all aspects of this sector’s workplace performance, including environment, task skills, task management skills, contingency management skills and job role environment skills.

Assessor requirements

Assessors must be able to clearly demonstrate current and relevant industry knowledge and experience to satisfy the mandatory regulatory standards as set out in the Standards for Registered Training Organisations (RTOs) 2015/Australian Quality Training Framework mandatory requirements for assessors current at the time of assessment and any relevant licensing and certification requirements. This includes:

vocational competencies at least to the level being delivered and assessed

current industry skills directly relevant to the training and assessment being provided

current knowledge and skills in vocational training and learning that informs their training and assessment

formal relevant qualifications in training and assessment

having knowledge of and/or experience using the latest techniques and processes

possessing the required level of RII training product knowledge

having an understanding and knowledge of legislation and regulations relevant to the industry and to employment and workplaces

demonstrating the performance evidence, and knowledge evidence outlined in this unit of competency, and

the minimum years of current** work experience after competency has been obtained as specified below in an industry sector relevant to the outcomes of the unit.

It is also acceptable for the appropriately qualified assessor to work with an industry expert to conduct assessment together and for the industry expert to be involved in the assessment judgement. The industry expert must have current industry skills directly relevant to the training and assessment being provided. This means the industry subject matter expert must demonstrate skills and knowledge from the minimum years of current work experience after competency has been obtained as specified below, including time spent in roles related to the unit being assessed:

| Industry sector | AQF indicator level*** | Required assessor or industry subject matter expert experience |
| --- | --- | --- |
| Drilling, Metalliferous Mining, Coal Mining, Extractive (Quarrying) and Civil Infrastructure | 1 | 1 year |
|  | 2 | 2 years |
| Drilling, Coal Mining, Extractive (Quarrying), Metalliferous Mining and Civil Infrastructure | 3-6 | 3 years |
| Other sectors | Where this unit is being assessed outside of the resources and infrastructure sectors assessor and/or industry subject matter expert experience should be in-line with industry standards for the sector in which it is being assessed and where no industry standard is specified should comply with any relevant regulation. |  |

*Guidance on simulated environments has been stipulated in the RII implementation guide located on VETNet.

**Assessors can demonstrate current work experience through employment within industry in a role relevant to the outcomes of the unit; or, for external assessors this can be demonstrated through exposure to industry by conducting a minimum number of site assessments as determined by the relevant industry sector, across various locations.

***While a unit of competency does not have an AQF level, where a unit is being delivered outside of a qualification the first numeric character in the unit code should be considered as the AQF indicator level for assessment purposes.

# Links

Companion Volume Implementation Guide is found on VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=88a61002-9a21-4386-aaf8-69c76e675272