# Assessment Requirements for RIIBLA202F Support underground shotfiring operations

Release: 1

Assessment Requirements for RIIBLA202F Support underground shotfiring operations

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version first released with RII Resources and Infrastructure Industry Training Package Version 3.0. |

# Performance Evidence

The candidate must demonstrate the ability to complete the tasks outlined in the elements and performance criteria of this unit.

The candidate must demonstrate completion of supporting underground shotfiring at a blast site that safely, effectively and efficiently follows legislative requirements and workplace procedures to meet required outcomes on at least two occasions. This includes:

following preparation procedures to support shotfiring, including ventilation, barring down (scaling), and air or water services

securing and isolating blast site

identifying that drill holes are drilled to authorised design and blast plan

flushing or cleaning, and testing drill holes

following precautionary measures relevant to ground conditions, including identifying reactive ores and using explosion inhibitor as required

handling and transporting explosives

loading and stemming explosives

removing equipment and facilities not required from blast site

performing final clean-up

assisting with securing blast site.

In the course of the above work, the candidate must also:

locate and apply required documentation, policies and procedures

select and wear personal protective equipment required for work activities

plan for support shotfiring activities

identify and report deteriorated explosives and accessories, applying safe work practices

identify and report potential health, safety and environmental hazards, and implement control measures.

# Knowledge Evidence

The candidate must demonstrate the following knowledge to effectively complete the tasks outlined in the elements and performance criteria of this unit.

Key policies, procedures, relevant standards, and legislative requirements for the underground shotfiring operations described in the performance evidence above, including those relating to:

site personnel and operational safety requirements

accessing explosives

transporting explosives

blast site isolation

working safely at heights

emergencies at blast site

safety data sheets

security of explosives

hazard identification and implementation of control measures

environmental requirements and constraints, including:

blast monitoring systems

gas monitoring methods and limits

flammable dust limits and controls

clean-up activities.

Key site information, including:

basic geological and technical data

site and ground conditions.

Key features associated with shotfiring performance, including:

types and characteristics of explosives, and protection measures

physical and technical quality of explosives

equipment characteristics, technical capabilities, limitations, and site conditions.

# Assessment Conditions

Mandatory conditions for assessment of this unit are stipulated below.

The assessment must:

include access to:

explosives and shotfiring equipment

blast plans

personal protective equipment required for the activities described in the performance evidence

be conducted in a safe environment

be assessed in the context of this sector’s work environment

be assessed in compliance with relevant legislation and regulations; and using policies, procedures, processes and operational manuals directly related to the industry sector for which it is being assessed

confirm consistent performance can be applied in a range of relevant workplace circumstances.

### Assessor requirements

Assessors must be able to clearly demonstrate current and relevant industry knowledge and experience to satisfy the mandatory competency standards as set out in the Standards for Registered Training Organisations (RTOs) current at the time of assessment. This includes:

having knowledge of and/or experience using the latest techniques and processes

possessing a high level of RII training product knowledge

having understanding and knowledge of legislation and regulations relevant to the industry and to employment and workplaces

possessing formal industry and training qualifications to the level being assessed

demonstrating the performance evidence and knowledge evidence outlined in this unit of competency, and through the minimum years of current* work experience specified below in an industry sector relevant to the outcomes of the unit.

Where the assessor does not meet experience requirements, a co-assessment or partnership arrangement must exist between the qualified assessor and an industry subject matter expert. The industry subject matter expert should hold the unit being assessed (or an equivalent unit), and must also demonstrate skills and knowledge from the minimum years of current work experience specified below, including time spent in roles related to the unit being assessed.

| #### Industry sector | #### AQF** level | #### Required assessor or industry subject matter expert experience |
| --- | --- | --- |
| Drilling, Metalliferous Mining, Coal Mining, Extractive (Quarrying) and Civil Construction | 1 | 1 Year |
|  | 2 | 2 Years |
| Drilling, Coal Mining and Extractive (Quarrying) | 3-6 | 3 Years |
| Metalliferous Mining and Civil Construction | 3-6 | 5 Years |
| Other sectors | Where this unit is being assessed outside of the resources and infrastructure sectors, assessor and/or industry subject matter expert experience should be in line with industry standards for the sector in which it is being assessed and, where no industry standard is specified, should comply with any relevant regulation. |  |

*Assessors can demonstrate current work experience through employment within industry in a role relevant to the outcomes of the unit; or, for external assessors this can be demonstrated through exposure to industry by conducting frequent site assessments across various locations.

**Where a unit is being delivered outside of a qualification, the first numeric character in the unit code should be considered to indicate the AQF level.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=88a61002-9a21-4386-aaf8-69c76e675272