# RIIBLA205D Store, handle and transport explosives

Release: 2

RIIBLA205D Store, handle and transport explosives

# Modification History

| #### Release | #### Comment |
| --- | --- |
| 1 | This unit replaces RIIBLA205A Store, handle and transport explosives |
| 2 | Required frequency and volume of evidence amended in Performance evidence. Substantial amendments made in Assessment Conditions field, including: references to Industry Sectors, assessor and subject matter expert experience requirements, how assessment should be conducted and what it should confirm. |

# Application

This unit describes a participant’s skills and knowledge required to store, handle and transport explosives in the Resources and Infrastructure Industries.

This unit is appropriate for those working in operational roles.

No licensing, legislation or certification requirements apply to this unit at the time of publication.

# Elements and Performance Criteria

| 1 Plan and prepare for storage, handling and transport of explosives | 1.1 Access, interpret and apply explosives documentation and ensure the work activity is compliant 1.2 Obtain, confirm and apply work instructions before proceeding 1.3 Identify, manage and report all potential hazards risks and environmental issues 1.4 Select and wear personal protective equipment appropriate to the work activities 1.5 Obtain and interpret emergency procedures and be prepared for fire/accident/emergency 1.6 Resolve coordination requirements with others prior to commencing and during work activities 1.7 Identify the explosives and accessories used for different applications 1.8 Classify explosives according to statutory criteria 1.9 Apply safe handling procedures and precautions |
| --- | --- |
| 2 Access and manage explosives storage | 2.1 Gain access to magazine as authorised person 2.2 Maintain compliance requirements for signage, construction, safety and security of magazines, storage limits 2.3 Receive and despatch explosives and record transfers indicating type and quantity together with identity of recipient 2.4 Segregate blasting agents and explosives according to type in magazines and to prevent deterioration, spoilage and spillage 2.5 Ensure rotation of explosives in magazine 2.6 Restrict access to store to authorised persons 2.7 Maintain written stock records and report discrepancies 2.8 Conduct housekeeping of magazine 2.9 Complete all required written documentation and reports clearly, concisely and on time 2.10 Pass on information regarding explosives stock and storage |
| 3 Prepare to transport explosives | 3.1 Receive orders and confirm authority of recipient to possess explosives 3.2 Select sufficient quantities of explosives and associated materials according to blast plan, for safe transport 3.3 Conduct and record results of pre-start check on explosive transport vehicle and confirm presence and operability of relevant safety equipment. 3.4 Carry out vehicle start-up procedures 3.5 Display and ensure legibility of relevant signs on vehicles 3.6 Load vehicle in accordance with separation and segregation requirements 3.7 Adhere to emergency procedures in case of fire and/or accident |
| 4 Transport explosives | 4.1 Communicate with other equipment operators and other persons using appropriate communication methods to advise of explosive movements 4.2 Transport explosives and accessories separately in approved and secured containers 4.3 Transport explosives and accessories to designated location using designated route 4.4 Apply secure and safe driving conventions 4.5 Check that delivery site is suited to explosives storage and that consignment is not left unattended 4.6 Implement emergency procedures to ensure safety of personnel and site 4.7 Return surplus explosives to magazine 4.8 Complete required documentation and reports promptly |

# Foundation Skills

Foundation skills essential to performance are explicit in the performance criteria of this unit. Further information is available in the Resources and Infrastructure Industry Training Package Companion Volume.

# Unit Mapping Information

RIIBLA205A Store, handle and transport explosives

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=88a61002-9a21-4386-aaf8-69c76e675272

Assessment Requirements for RIIBLA205D Store, handle and transport explosives

# Modification History

| #### Release | #### Comment |
| --- | --- |
| 1 | This unit replaces RIIBLA205A Store, handle and transport explosives |
| 2 | Required frequency and volume of evidence amended in Performance evidence. Substantial amendments made in Assessment Conditions field, including: references to Industry Sectors, assessor and subject matter expert experience requirements, how assessment should be conducted and what it should confirm. |

# Performance Evidence

Evidence is required to be collected that demonstrates a candidate’s competency in this unit. Evidence must be relevant to the roles within this sector’s work operations and satisfy all of the requirements of the performance criteria of this unit and include evidence that the candidate:

locates and applies relevant documentation, policies and procedures

demonstrates completion of storing, handling and transporting explosives that safely, effectively and efficiently meets all of the required outcomes on more than one (1) occasion including:

planning and preparing to handle and transport explosives

receiving and despatching explosives

segregating explosives and accessories

completing rotation of explosives in magazine

maintaining stock records and report discrepancies

selecting sufficient quantities of explosives and accessories according to shot plan

storing; handling and transporting explosives

accessing and managing explosives storage

implementing required signage

# Knowledge Evidence

The candidate must demonstrate knowledge of storing, handling and transporting explosives through:

blasting

hazardous goods requirements

WHS requirements and procedures

emergency procedures

site operational systems, procedures and checks

manufacturer’s instructions

types, characteristics and applications of explosives and accessories

equipment operational procedures, technical capability and limitations

# Assessment Conditions

An assessor of this unit must satisfy the requirements of the NVR/AQTF or their successors; and Industry regulations for certification and licensing; and,

this unit is best assessed in the context of this sector’s work environment;

where personal safety or environmental damage are limiting factors, assessment may occur in a simulated environment provided it is realistic and sufficiently rigorous to cover all aspects of this sector’s workplace performance, including environment, task skills, task management skills, contingency management skills and job role environment skills; and,

this unit must be assessed in compliance with relevant legislation/regulation and using policies, procedures, processes and operational manuals directly related to the industry sector for which it is being assessed; and,

assessment may be conducted in conjunction with the assessment of other Units of Competency; and,

assessment must confirm consistent performance can be applied in a range of relevant workplace circumstances; and,

assessors must demonstrate the performance evidence, and knowledge evidence as outlined in this Unit of Competency, and through the minimum years of current* work experience specified below in an Industry sector relevant to the outcomes of the unit; or,

where the assessor does not meet experience requirements a co-assessment or partnership arrangement must exist between the qualified assessor and an Industry subject matter expert. The Industry subject matter expert should hold the unit being assessed (or an equivalent unit) and/or demonstrate equivalence of skills and knowledge at the unit level. An Industry technical expert must also demonstrate skills and knowledge from the minimum years of current work experience specified below in the Industry sector, including time spent in roles related to the unit being assessed; and,

assessor and Industry subject matter expert requirements differ depending on the Australian Qualifications Framework Level (AQF) of the qualification being assessed and/or Industry Sector as follows:

| #### Industry sector | #### AQF** Level | #### Required assessor or Industry subject matter expert experience |
| --- | --- | --- |
| Drilling, Metalliferous Mining, Coal Mining, Extractive (Quarrying) and Civil Construction | 1 | 1 Year |
|  | 2 | 2 Years |
| Drilling, Coal Mining and Extractive (Quarrying) | 3-6 | 3 Years |
| Metalliferous Mining and Civil Construction | 3-6 | 5 Years |
| Other sectors | Where this Unit is being assessed outside of the Resources and Infrastructure Sectors assessor and/or Industry subject matter expert experience should be in-line with industry standards for the sector in which it is being assessed and where no Industry standard is specified should comply with any relevant regulation. |  |

*Assessors can demonstrate current work experience through employment within Industry in a role relevant to the outcomes of the Unit; or, for external assessors this can be demonstrated through exposure to Industry by conducting frequent site assessments across various locations.

**Where a unit is being delivered outside of a Qualification the first numeric character in the Unit code should be considered to indicate the AQF level

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=88a61002-9a21-4386-aaf8-69c76e675272