# RIICBS303E Conduct materials transfer vehicle operations

Release: 1

RIICBS303E Conduct materials transfer vehicle operations

# Modification History

This unit replaces RIICBS303D Conduct materials transfer vehicle operations. Minor endorseable amendments have been made to Elements, Performance Criteria, Foundation Skills, Performance Evidence and Knowledge Evidence to better reflect current industry practices and clarify training outcomes.

# Application

This unit describes the skills and knowledge required to conduct materials transfer vehicle operations in civil construction.

It applies to those working in operational roles. The individual is generally a skilled operator and may provide theoretical advice and support a team.

Licensing, legislative, regulatory and certification requirements that apply to this unit can vary between states, territories, and industry sectors. Relevant information must be sourced prior to application of the unit.

# Unit Sector

Civil construction

# Elements and Performance Criteria

| ELEMENT | PERFORMANCE CRITERIA |
| --- | --- |
| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. |
| 1. Prepare to transfer materials | 1.1 Obtain, interpret and confirm work requirements 1.2 Access, interpret and apply documentation required for conducting materials transfer vehicle operations and confirm work activity is compliant 1.3 Identify environmental issues and potential hazards, and assess and address risks within scope of own role and according to workplace procedures 1.4 Select and wear personal protective equipment required for work activities 1.5 Confirm and comply with environmental protection, safety and traffic management signage requirements 1.6 Select plant, tools and equipment required for tasks and check for faults within scope of own role 1.7 Obtain and interpret emergency procedures and identify steps required in case of fire, accident and other emergencies |
| 2. Transfer materials | 2.1 Establish communication with other plant operators and personnel according to job requirements and workplace procedures 2.2 Fit feeder bin according to job requirements and workplace procedures 2.3 Perform start-up, park, shutdown and secure equipment procedures 2.4 Test vehicle controls and functions before commencing operations 2.5 Position transfer vehicle for asphalt delivery to paver without spillage 2.6 Direct delivery truck into position with the materials transfer vehicle according to job requirements and workplace procedures 2.7 Receive asphalt from delivery truck into the materials transfer vehicle hoppers and monitor 2.8 Deliver asphalt to paver smoothly according to job requirements and without impact that would cause bounce in the paver 2.9 Maintain speed as directed by paver operator during transfer operations |
| 3. Carry out operator maintenance | 3.1 Park safely, shut down and prepare the material transfer vehicle for maintenance 3.2 Conduct inspection and fault finding 3.3 Remove and replace defective parts 3.4 Carry out regular programmed maintenance tasks according to workplace procedures 3.5 Clean out equipment between changeover of product and at the end of the work period as required |
| 4. Clean up | 4.1 Clear work area and dispose of and recycle materials as required according to workplace procedures and environmental requirements 4.2 Clean, check, perform maintenance on in scope of own role, and store plant, tools and equipment 4.3 Carry out housekeeping to workplace requirements |

# Foundation Skills

This section describes those language, literacy, numeracy and employment skills that are essential to performance but not explicit in the performance criteria.

| Skill | Description |
| --- | --- |
| Reading | Identifies and interprets information from workplace procedures, documentation, legislation and regulations |
| Self-management | Monitors and minimises own exposure to worksite hazards during activities |
| Oral communication | Uses a range of communication techniques and systems to communicate with others |

# Unit Mapping Information

Supersedes and is equivalent to RIICBS303D Conduct materials transfer vehicle operations.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=88a61002-9a21-4386-aaf8-69c76e675272

Assessment Requirements for RIICBS303E Conduct materials transfer vehicle operations

# Modification History

This unit replaces RIICBS303D Conduct materials transfer vehicle operations. Minor endorseable amendments have been made to Elements, Performance Criteria, Foundation Skills, Performance Evidence and Knowledge Evidence to better reflect current industry practices and clarify training outcomes.

# Performance Evidence

The candidate must demonstrate the ability to complete the tasks outlined in the elements, performance criteria and foundation skills of this unit; including evidence of the ability to:

conduct materials transfer vehicle operations on at least two occasions.

During the above, the candidate must:

locate and apply legislation, documentation, policies and procedures required for conducting materials transfer vehicle operations

work with others to undertake and complete materials transfer vehicle operations that meets the required outcomes, including:

organising work activities to meet all task requirements

communicating with others to receive and clarify work instructions

using a range of communication techniques and systems

using signage to advise others of work activity and exclusion zone

communicating with paver operator to maintain speed during transfer operations

identify, report and record hazards and risks

clean, check, perform maintenance on, relocate and store equipment

complete housekeeping activities.

# Knowledge Evidence

The candidate must be able to demonstrate knowledge to complete the tasks outlined in the elements, performance criteria and foundation skills of this unit, including knowledge of:

on site processes and responsibilities relating to site isolation and traffic control

processes for calculating material requirements, application rates and travel speed

safety data sheet (SDS) compliance processes

key components of job safety analyses (JSAs)/job safety environmental analyses (JSEAs)/safe work method statements (SWMs)

key legislation for conducting materials transfer vehicle operations

key policies, procedures and established requirements for conducting materials transfer vehicle operations, including those for:

site and equipment safety

work quality

environmental protection, including waste disposal and recycling

positioning vehicle for transfer

communicating with other plant operators and personnel

delivering asphalt to paver

directing delivery truck

operating and maintaining vehicle and equipment

safe operating techniques in all terrain

housekeeping

key factors affecting work activities described in performance evidence above, including:

asphalt paving techniques

material transfer vehicle equipment types, characteristics, technical capabilities and limitations.

# Assessment Conditions

Mandatory conditions for assessment of this unit are stipulated below. The assessment must:

include access to:

materials transfer vehicle

other materials transfer equipment

personal protective equipment

be conducted in a safe environment; and,

be assessed in context of this sector’s work environment; and,

be assessed in compliance with legislation/regulation and using policies, procedures, processes and operational manuals directly related to the industry sector for which it is being assessed; and,

confirm consistent performance can be applied in a range of relevant workplace circumstances.

Where personal safety or environmental damage are limiting factors, assessment may occur in a simulated work environment* provided it is realistic and sufficiently rigorous to cover all aspects of this sectors workplace performance, including environment, task skills, task management skills, contingency management skills and job role environment skills.

# Assessor Requirements

Assessors must be able to clearly demonstrate current and relevant industry knowledge and experience to satisfy the mandatory competency standards as set out in the Standards for Registered Training Organisations (RTOs) 2015/AQTF mandatory requirements for assessors current at the time of assessment and any relevant licensing and certification requirements. This includes:

vocational competencies at least to the level being delivered and assessed

current industry skills directly relevant to the training and assessment being provided

current knowledge and skills in vocational training and learning that informs their training and assessment

formal relevant qualifications in training and assessment

having knowledge of and/or experience using the latest techniques and processes

possessing a high level of RII training product knowledge

having an understanding and knowledge of legislations and regulations relevant to the industry and to employment and workplaces

demonstrating the performance evidence, and knowledge evidence as outlined in this unit of competency, and

the minimum years of current** work experience after competency has been obtained as specified below in an industry sector relevant to the outcomes of the unit.

It is also acceptable for the appropriately qualified assessor to work with an industry expert to conduct assessment together and for the industry expert to be involved in the assessment judgement. The industry expert must hold the relevant vocational competencies and have current industry skills directly relevant to the training and assessment being provided and must work alongside a trainer and/or assessor to conduct the assessment. This means the industry subject matter expert should hold the unit being assessed (or an equivalent unit), and must also demonstrate skills and knowledge from the minimum years of current work experience after competency has been obtained as specified below, including time spent in roles related to the unit being assessed:

| #### Industry sector | #### AQF** level | #### Required assessor or industry subject matter expert experience |
| --- | --- | --- |
| Drilling, Metalliferous Mining, Coal Mining, Extractive (Quarrying) and Civil Construction | 1 | 1 Year |
|  | 2 | 2 Years |
| Drilling, Coal Mining and Extractive (Quarrying) | 3-6 | 3 Years |
| Metalliferous Mining and Civil Construction | 3-6 | 5 Years |
| Other sectors | Where this unit of competency is being assessed outside of the resources and infrastructure sectors assessor and/or industry subject matter expert experience should be in-line with industry standards for the sector in which it is being assessed and where no industry standard is specified should comply with any relevant regulation. |  |

*Guidance on simulated environments has been stipulated in the RII Companion Volume Implementation Guide located on VETNet.

**Assessors can demonstrate current work experience through employment within industry in a role relevant to the outcomes of the unit; or, for external assessors this can be demonstrated through exposure to industry by conducting a minimum number of site assessments as determined by the relevant industry sector, across various locations.

*** While a unit of competency does not have an AQF level, where a unit is being delivered outside of a qualification the first numeric character in the unit code should be considered as the AQF indicator level for assessment purposes.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=88a61002-9a21-4386-aaf8-69c76e675272