# RIICCM205F Carry out manual excavation

Release: 2

RIICCM205F Carry out manual excavation

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version first released with RII Resources and Infrastructure Industry Training Package Version 6.0. |

# Application

This unit describes the skills and knowledge required to carry out manual excavation in civil construction, including confirming the location of underground services, manually excavating, placing barricades and isolating the excavation.

It applies to those working in operational roles. They generally work under supervision to undertake a prescribed range of functions involving known routines and procedures and take some responsibility for the quality of work outcomes.

Licensing, legislative and certification requirements that apply to this unit can vary between states, territories, and industry sectors. Users must check requirements with relevant body before applying the unit.

# Unit Sector

Civil construction

# Elements and Performance Criteria

| ELEMENT | PERFORMANCE CRITERIA |
| --- | --- |
| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. |
| 1. Plan and prepare for manual excavation | 1.1 Obtain, interpret and confirm work requirements 1.2 Access, interpret and apply documentation required to carry out manual excavation 1.3 Obtain and interpret emergency procedures and identify steps required to respond to emergencies 1.4 Select and wear personal protective equipment according to site requirements 1.5 Confirm and identify requirements for complying with environmental protection, safety and traffic management signage requirements 1.6 Identify, address and report environmental issues, potential hazards and risks 1.7 Select plant, tools and equipment required to carry out tasks and check for faults 1.8 Identify, confirm, and locate materials required for work application |
| 2. Initiate manual excavation procedures | 2.1 Confirm location and specifications of intended excavation 2.2 Identify service markers and taped areas 2.3 Determine and confirm location of underground services and avoid damage and interference 2.4 Use hand tools and dig post holes, small pits and trenches to required dimensions 2.5 Undertake trench collapse prevention procedures where excavation is in unstable ground 2.6 Place barricades around the excavation according to job requirements |
| 3. Complete and isolate the excavation | 3.1 Clean loose material out of excavation using hand tools 3.2 Confirm excavation complies with specification and job requirements 3.3 Clear loose material away from edge of excavation |
| 4. Conduct housekeeping activities | 4.1 Clear work area and dispose of or recycle materials according to site requirements 4.2 Clean and maintain condition of equipment, confirm suitability for use, and address and report issues as required within scope of own role 4.3 Manage and report hazards, and maintain a safe working environment 4.4 Process records according to site requirements |

# Foundation Skills

This section describes those language, literacy, numeracy and employment skills that are essential to performance but not explicit in the performance criteria.

| Skill | Description |
| --- | --- |
| Numeracy | Calculates basic measurements including mass and dimensions |
| Self-management | Monitors and minimises own exposure to worksite risks and hazards during work activities |

# Unit Mapping Information

Supersedes and is equivalent to RIICCM205E Carry out manual excavation.

# Links

Companion Volume Implementation Guide is found on VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=88a61002-9a21-4386-aaf8-69c76e675272

Assessment Requirements for RIICCM205F Carry out manual excavation

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version first released with RII Resources and Infrastructure Industry Training Package Version 6.0. |

# Performance Evidence

The candidate must demonstrate the ability to complete the tasks outlined in the elements, performance criteria and foundation skills of this unit, including evidence of the ability to:

carry out manual excavation on at least two occasions, including:

identifying the location of underground services

digging post holes, small pits and trenches to the required dimensions

applying trench collapse prevention procedures in unstable ground.

During the above, the candidate must:

locate and apply relevant documentation, policies and procedures

identify, report and record hazards and risks

implement the requirements, procedures and techniques for carrying out manual excavation, including using manual excavation tools

work with others to carry out manual excavation to meet required outcomes, including:

communicating with others to receive and clarify work instructions

using communications techniques and equipment to convey information to others, including signage to advise others of work activity and exclusion zones

complying with reporting requirements and procedures.

# Knowledge Evidence

The candidate must be able to demonstrate knowledge to complete the tasks outlined in the elements, performance criteria and foundation skills of this unit, including knowledge of:

legislation required to carry out manual excavation

policies, procedures and documentation required to carry out manual excavation, including those relating to:

hazard and risk management

work health and safety

emergency response

environmental protection

waste management

communication techniques and equipment

workplace recording and reporting

site isolation and traffic control responsibilities

operational maintenance and basic diagnostic procedures

project quality requirements

job plans and specifications

housekeeping activities, including:

cleaning work area

cleaning and maintaining condition of equipment

waste management

processing records

principles and techniques required to carry out manual excavation, including those relating to:

organising work activities

soil technology

trench collapse prevention techniques, including benching and battering

site conditions

underground services

excavation methods

rock types, including:

metamorphic

igneous

sedimentary

types, characteristics, technical capabilities and limitations of plant and equipment required to carry out manual excavation, including:

picks

shovels

hand augers

string lines

jack hammers

scabblers

materials safety data sheet (SDS) compliance processes

components of job safety analyses (JSAs), job safety environmental analyses (JSEAs), and safe work method statements (SWMs)

civil construction terminology related to carrying out manual excavation.

# Assessment Conditions

Mandatory conditions for assessment of this unit are stipulated below. The assessment must:

include access to:

personal protective equipment

equipment required to carry out manual excavation

be conducted in a safe environment; and,

be assessed in the context of this sector’s work environment; and,

be assessed in compliance with relevant legislation/regulation and using policies, procedures and processes directly related to the industry sector for which it is being assessed; and,

confirm consistent performance can be applied in a range of relevant workplace circumstances.

Where personal safety or environmental damage are limiting factors, assessment may occur in a simulated work environment* provided it is realistic and sufficiently rigorous to cover all aspects of this sector’s workplace performance, including environment, task skills, task management skills, contingency management skills and job role environment skills.

Assessor requirements

Assessors must be able to clearly demonstrate current and relevant industry knowledge and experience to satisfy the mandatory regulatory standards as set out in the Standards for Registered Training Organisations 2015/Australian Quality Training Framework mandatory requirements for assessors current at the time of assessment and any relevant licensing and certification requirements. This includes:

vocational competencies at least to the level being delivered and assessed

current industry skills directly relevant to the training and assessment being provided

current knowledge and skills in vocational training and learning that informs their training and assessment

formal relevant qualifications in training and assessment

having knowledge of and/or experience using the latest techniques and processes

possessing the required level of RII training product knowledge

having an understanding and knowledge of legislation and regulations relevant to the industry and to employment and workplaces

demonstrating the performance evidence, and knowledge evidence outlined in this unit of competency, and

the minimum years of current** work experience after competency has been obtained as specified below in an industry sector relevant to the outcomes of the unit.

It is also acceptable for the appropriately qualified assessor to work with an industry expert to conduct assessment together and for the industry expert to be involved in the assessment judgement. The industry expert must have current industry skills directly relevant to the training and assessment being provided. This means the industry subject matter expert must demonstrate skills and knowledge from the minimum years of current work experience after competency has been obtained as specified below, including time spent in roles related to the unit being assessed:

| #### Industry sector | #### AQF indicator level*** | #### Required assessor or industry subject matter expert experience |
| --- | --- | --- |
| Drilling, Metalliferous Mining, Coal Mining, Extractive (Quarrying) and Civil Infrastructure | 1 | 1 year |
|  | 2 | 2 years |
| Drilling, Coal Mining, Extractive (Quarrying), Metalliferous Mining and Civil Infrastructure | 3-6 | 3 years |
| Other sectors | Where this unit is being assessed outside of the resources and infrastructure sectors assessor and/or industry subject matter expert experience should be in-line with industry standards for the sector in which it is being assessed and where no industry standard is specified should comply with any relevant regulation. |  |

*Guidance on simulated environments has been stipulated in the RII Companion Volume Implementation Guide located on VETNet.

**Assessors can demonstrate current work experience through employment within industry in a role relevant to the outcomes of the unit; or, for external assessors this can be demonstrated through exposure to industry by conducting a minimum number of site assessments as determined by the relevant industry sector, across various locations.

*** While a unit of competency does not have an AQF level, where a unit is being delivered outside of a qualification the first numeric character in the unit code should be considered as the AQF indicator level for assessment purposes.

# Links

# Links

Companion Volume Implementation Guide is found on VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=88a61002-9a21-4386-aaf8-69c76e675272