# Assessment Requirements for RIICFW305E Conduct cast in-situ pilings operations

Release: 1

Assessment Requirements for RIICFW305E Conduct cast in-situ pilings operations

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version first released with RII Resources and Infrastructure Industry Training Package Version 6.0. |

# Performance Evidence

The candidate must demonstrate the ability to complete the tasks outlined in the elements, performance criteria and foundation skills of this unit, including evidence of the ability to:

conduct cast in-situ piling operations on at least two occasions, including:

boring a hole to install at least one caisson, including reinforcement construction and placing concrete into caisson

setting up, relocating and removing boring rig.

During the above, the candidate must:

locate and apply required legislation, documentation, policies and procedures

identify, report and record hazards and risks

work with others to undertake and complete cast in-situ pilings operations that meets required outcomes, including:

communicating with others to receive and clarify work instructions

using communication techniques and equipment, including signage to advise others of work activity and exclusion zones.

# Knowledge Evidence

The candidate must be able to demonstrate knowledge to complete the tasks outlined in the elements, performance criteria and foundation skills of this unit, including knowledge of:

legislation required to conduct cast in-situ pilings operations

policies, procedures and documentation required to conduct cast in-situ pilings operations, including those relating to:

hazard and risk management

work health and safety

emergency response

site and equipment safety

environmental protection

waste management

communication techniques and equipment

site isolation and traffic control responsibilities

operational, maintenance and basic diagnostic procedures

project quality requirements job plans and specifications

site requirements

manufacturer specifications for required plant and equipment

workplace recording and reporting

principles and techniques required to conduct cast in-situ pilings operations, including those relating to:

foundation work

water erosion

construction work

bored cast in-situ piles characteristics and installation

scaffolding and work platform installation

installing piles in water

pile location

soil types and characteristics, including:

sand

rock

clay

shale

gravel

silt

vibration methods

characteristics of reinforced concrete

concreting

bore types

boring rigs

caisson types and characteristics

interpreting engineering drawings

types, characteristics, technical capabilities and limitations of plant and equipment required to conduct cast in-situ pilings operations, including:

augers

spirit levels

plumb bobs

compressors

concrete vibrators

high pressure hoses

scaffolding

materials safety data sheet (SDS) compliance processes

components of job safety analyses (JSAs), job safety environmental analyses (JSEAs), and safe work method statements (SWMs)

civil construction terminology related to conducting cast in-situ pilings operations.

# Assessment Conditions

Mandatory conditions for assessment of this unit are stipulated below. The assessment must:

include access to:

personal protective equipment

equipment required to conduct in-situ pilings operations

be conducted in a safe environment; and,

be assessed in the context of this sector’s work environment; and,

be assessed in compliance with required legislation/regulation and using policies, procedures and processes directly related to the industry sector for which it is being assessed; and,

confirm consistent performance can be applied in a range of relevant workplace circumstances.

Where personal safety or environmental damage are limiting factors, assessment may occur in a simulated work environment* provided it is realistic and sufficiently rigorous to cover all aspects of this sector’s workplace performance, including environment, task skills, task management skills, contingency management skills and job role environment skills.

Assessor requirements

Assessors must be able to clearly demonstrate current and relevant industry knowledge and experience to satisfy the mandatory regulatory standards as set out in the Standards for Registered Training Organisations 2015/Australian Quality Training Framework mandatory requirements for assessors current at the time of assessment and any relevant licensing and certification requirements. This includes:

vocational competencies at least to the level being delivered and assessed

current industry skills directly relevant to the training and assessment being provided

current knowledge and skills in vocational training and learning that informs their training and assessment

formal relevant qualifications in training and assessment

having knowledge of and/or experience using the latest techniques and processes

possessing the required level of RII training product knowledge

having an understanding and knowledge of legislation and regulations relevant to the industry and to employment and workplaces

demonstrating the performance evidence, and knowledge evidence outlined in this unit of competency, and

the minimum years of current** work experience after competency has been obtained as specified below in an industry sector relevant to the outcomes of the unit.

It is also acceptable for the appropriately qualified assessor to work with an industry expert to conduct assessment together and for the industry expert to be involved in the assessment judgement. The industry expert must have current industry skills directly relevant to the training and assessment being provided. This means the industry subject matter expert must demonstrate skills and knowledge from the minimum years of current work experience after competency has been obtained as specified below, including time spent in roles related to the unit being assessed:

| #### Industry sector | #### AQF indicator level*** | #### Required assessor or industry subject matter expert experience |
| --- | --- | --- |
| Drilling, Metalliferous Mining, Coal Mining, Extractive (Quarrying) and Civil Infrastructure | 1 | 1 year |
|  | 2 | 2 years |
| Drilling, Coal Mining, Extractive (Quarrying), Metalliferous Mining and Civil Infrastructure | 3-6 | 3 years |
| Other sectors | Where this unit is being assessed outside of the resources and infrastructure sectors assessor and/or industry subject matter expert experience should be in-line with industry standards for the sector in which it is being assessed and where no industry standard is specified should comply with any relevant regulation. |  |

*Guidance on simulated environments has been stipulated in the RII implementation guide located on VETNet.

**Assessors can demonstrate current work experience through employment within industry in a role relevant to the outcomes of the unit; or, for external assessors this can be demonstrated through exposure to industry by conducting a minimum number of site assessments as determined by the relevant industry sector, across various locations.

***While a unit of competency does not have an AQF level, where a unit is being delivered outside of a qualification the first numeric character in the unit code should be considered as the AQF indicator level for assessment purposes.

# Links

Companion Volume Implementation Guide is found on VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=88a61002-9a21-4386-aaf8-69c76e675272