# RIICFW306E Direct pile driving operations

Release: 1

RIICFW306E Direct pile driving operations

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version first released with RII Resources and Infrastructure Industry Training Package Version 6.0. |

# Application

This unit describes the skills and knowledge required to direct pile driving operations in civil construction. This includes establishing recovery pegs, providing guidance for lifting, manoeuvring and setting up piling rigs, installing timber and pre-cast concrete piles, and conducting pile hammering.

It applies to those working in operational roles. They generally work under minimal supervision to undertake a broad range of tasks in varied work contexts, using some discretion and judgement in selecting equipment, services or contingency measures.

Licensing, legislative and certification requirements that apply to this unit can vary between states, territories and industry sectors. Users must check requirements with relevant body before applying the unit.

# Unit Sector

Civil construction

# Elements and Performance Criteria

| ELEMENT | PERFORMANCE CRITERIA |
| --- | --- |
| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. |
| 1. Plan and prepare to direct pile driving operations | 1.1 Obtain, interpret and confirm work instructions 1.2 Access, interpret and apply documentation required to direct pile driving operations and confirm work activity is compliant 1.3 Obtain and interpret emergency procedures and identify steps required to respond to emergencies 1.4 Select and wear personal protective equipment according to site requirements 1.5 Confirm and identify requirements for complying with environmental protection, safety and traffic management signage requirements 1.6 Identify potential hazards and environmental issues, implement control measures within scope of own role and escalate where required 1.7 Select plant, tools and equipment required to carry out tasks and check for faults 1.8 Identify, confirm, and locate materials required for work application |
| 2. Assist with locating pile position and establishing piling rig | 2.1 Establish location for piles from reference points and set out according to job requirements 2.2 Establish recovery pegs and conduct on-going measurements to confirm correct location according to job requirements 2.3 Direct plant and related equipment in position according to job requirements and conduct pre-start checks 2.4 Prepare and confirm conformity of pile according to job requirements 2.5 Identify and protect area for pile driving and surrounding working space according to job requirements |
| 3. Support piling process | 3.1 Provide guidance for lifting and manoeuvring piles into position according to job requirements 3.2 Provide guidance for setting up piling rig and driving pile into position according to job requirements 3.3 Direct splicing according to job requirements |
| 4. Carry out post-operational requirements | 4.1 Assist with dismantling of plant and related equipment according to manufacturer specifications and job requirements 4.2 Provide direction for relocation of piling rig according to job and site requirements 4.3 Clear work area and dispose of or recycle materials according to site requirements 4.4 Clean, check, perform maintenance on and store plant, tools and equipment according to manufacturer specifications and site requirements |

# Foundation Skills

This section describes those language, literacy, numeracy and employment skills that are essential to performance but not explicit in the performance criteria.

| SKILL | DESCRIPTION |
| --- | --- |
| Initiative and enterprise | Identifies and executes tasks independently |
| Numeracy | Calculates basic measurements including mass and dimensions |
| Oral communication | Identifies and interprets information from verbal instructions Conveys information and instructions to others verbally |
| Self-management | Monitors and minimises own exposure to worksite risks and hazards during work activities |

# Unit Mapping Information

Supersedes and is equivalent to RIICFW306D Direct pile driving operations.

# Links

Companion Volume Implementation Guide is found on VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=88a61002-9a21-4386-aaf8-69c76e675272

Assessment Requirements for RIICFW306E Direct pile driving operations

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version first released with RII Resources and Infrastructure Industry Training Package Version 6.0. |

# Performance Evidence

The candidate must demonstrate the ability to complete the tasks outlined in the elements, performance criteria and foundation skills of this unit, including evidence of the ability to:

direct pile driving operations on at least two occasions, including:

providing guidance for setting up of piling rigs

installing at least two timber piles, two pre-cast concrete piles, or two sheet piles

demonstrating at least one of the three pile hammering methods.

During the above, the candidate must:

locate and apply required legislation, documentation, policies and procedures

identify, report and record hazards and risks

work with others to direct, undertake and complete pile driving operations tasks that meets required outcomes, including:

communicating with others to resolve coordination requirements and receive and clarify work instructions

using communication techniques and equipment, including signage to advise others of work activity and exclusion zones.

# Knowledge Evidence

The candidate must be able to demonstrate knowledge to complete the tasks outlined in the elements, performance criteria and foundation skills of this unit, including knowledge of:

legislation required to direct pile driving operations

policies, procedures and documentation required to direct pile driving operations, including those relating to:

hazard and risk management

work health and safety

emergency response

site and equipment safety

environmental protection

environmentally sensitive fluids and materials disposal

waste management

communication techniques and equipment

site isolation and traffic control responsibilities

operational, maintenance and basic diagnostic procedures

project quality requirements

chemical and fuel safety

hazardous goods handling

site heritage requirements

work instructions

inspecting and preparing work areas

pre-start checks

job plans and specifications

site requirements

manufacturer specifications for required plant and equipment

workplace recording and reporting

principles and techniques required to direct pile driving operations, including those relating to:

off-set pegging and recovery pegs

pile locating

plumbing and pile driving

pile splicing and jointing

maintenance requirements

water erosion

foundation work

scaffolding and work platform installation

driven piles and installation

techniques for installing piles in water

lifting, manoeuvring and setting up pile rigs

pile hammering

soil types and characteristics, including:

sand

rock

clay

shale

gravel

silt

personnel coordination

machine operation, including characteristics, technical capability and limitations

interpreting engineering drawings

types, characteristics, technical capabilities and limitations of plant and equipment required to direct pile driving operations, including:

pile hammers

shovels

crow bars

measuring tapes

spirit levels

plumb bob

materials safety data sheet (SDS) compliance processes

components of job safety analyses (JSAs), job safety environmental analyses (JSEAs), and safe work method statements (SWMs)

civil construction terminology related to directing pile driving operations.

# Assessment Conditions

Mandatory conditions for assessment of this unit are stipulated below. The assessment must:

include access to:

personal protective equipment

equipment required to direct pile driving operations

be conducted in a safe environment; and,

be assessed in the context of this sector’s work environment; and,

be assessed in compliance with required legislation/regulation and using policies, procedures and processes directly related to the industry sector for which it is being assessed; and,

confirm consistent performance can be applied in a range of relevant workplace circumstances.

Where personal safety or environmental damage are limiting factors, assessment may occur in a simulated work environment* provided it is realistic and sufficiently rigorous to cover all aspects of this sector’s workplace performance, including environment, task skills, task management skills, contingency management skills and job role environment skills.

Assessor requirements

Assessors must be able to clearly demonstrate current and relevant industry knowledge and experience to satisfy the mandatory regulatory standards as set out in the Standards for Registered Training Organisations 2015/Australian Quality Training Framework mandatory requirements for assessors current at the time of assessment and any relevant licensing and certification requirements. This includes:

vocational competencies at least to the level being delivered and assessed

current industry skills directly relevant to the training and assessment being provided

current knowledge and skills in vocational training and learning that informs their training and assessment

formal relevant qualifications in training and assessment

having knowledge of and/or experience using the latest techniques and processes

possessing the required level of RII training product knowledge

having an understanding and knowledge of legislation and regulations relevant to the industry and to employment and workplaces

demonstrating the performance evidence, and knowledge evidence outlined in this unit of competency, and

the minimum years of current** work experience after competency has been obtained as specified below in an industry sector relevant to the outcomes of the unit.

It is also acceptable for the appropriately qualified assessor to work with an industry expert to conduct assessment together and for the industry expert to be involved in the assessment judgement. The industry expert must have current industry skills directly relevant to the training and assessment being provided. This means the industry subject matter expert must demonstrate skills and knowledge from the minimum years of current work experience after competency has been obtained as specified below, including time spent in roles related to the unit being assessed:

| #### Industry sector | #### AQF indicator level*** | #### Required assessor or industry subject matter expert experience |
| --- | --- | --- |
| Drilling, Metalliferous Mining, Coal Mining, Extractive (Quarrying) and Civil Infrastructure | 1 | 1 year |
|  | 2 | 2 years |
| Drilling, Coal Mining, Extractive (Quarrying), Metalliferous Mining and Civil Infrastructure | 3-6 | 3 years |
| Other sectors | Where this unit is being assessed outside of the resources and infrastructure sectors assessor and/or industry subject matter expert experience should be in-line with industry standards for the sector in which it is being assessed and where no industry standard is specified should comply with any relevant regulation. |  |

*Guidance on simulated environments has been stipulated in the RII implementation guide located on VETNet.

**Assessors can demonstrate current work experience through employment within industry in a role relevant to the outcomes of the unit; or, for external assessors this can be demonstrated through exposure to industry by conducting a minimum number of site assessments as determined by the relevant industry sector, across various locations.

***While a unit of competency does not have an AQF level, where a unit is being delivered outside of a qualification the first numeric character in the unit code should be considered as the AQF indicator level for assessment purposes.

# Links

Companion Volume Implementation Guide is found on VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=88a61002-9a21-4386-aaf8-69c76e675272