# RIICRC207D Install noise barriers

Release: 2

RIICRC207D Install noise barriers

# Modification History

| Release | Comment |
| --- | --- |
| 1 | This unit replaces RIICRC207A Install noise barriers |
| 2 | Required frequency and volume of evidence amended in Performance evidence. Substantial amendments made in Assessment Conditions field, including: references to Industry Sectors, assessor and subject matter expert experience requirements, how assessment should be conducted and what it should confirm. |

# Application

This unit describes a participant’s skills and knowledge required to install noise barriers in Civil Construction.

This unit is appropriate for those working in operational roles.

Licensing, legislative, regulatory and certification requirements that apply to this unit can vary between states, territories, and Industry sectors. Relevant information must be sourced prior to application of the unit.

# Unit Sector

Civil construction

# Elements and Performance Criteria

| 1. Plan and prepare for installing noise barriers | 1.1 Access, interpret and apply noise barrier installation documentation and ensure the work activity is compliant 1.2 Obtain, read, interpret, clarify and confirm work requirements 1.3 Identify and address potential risks, hazards and environmental issues, and implement control measures 1.4 Select and wear correct personal protective equipment appropriate for work activities 1.5 Identify, obtain, confirm and implement signage requirements 1.6 Select and check for faults all plant, tools and equipment to carry out tasks 1.7 Survey work area for loading and unloading and stock-piling materials 1.8 Assess plant and crane capabilities as required |
| --- | --- |
| 2. Erect noise barriers | 2.1 Set out position of noise barrier line and post holes, and excavate to specifications 2.2 Install base section 2.3 Install uprights plumb and to alignment 2.4 Fit and fix rails, cladding or panels |
| 3. Clean up | 3.1 Clear work area and recycle or dispose of materials 3.2 Store and stack unused materials 3.3 Clean, check, maintain and store plant, tools and equipment |

# Foundation Skills

Foundation skills essential to performance are explicit in the performance criteria of this unit. Further information is available in the Resources and Infrastructure Industry Training Package Companion Volume.

# Unit Mapping Information

RIICRC207A Install noise barriers

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=88a61002-9a21-4386-aaf8-69c76e675272

Assessment Requirements for RIICRC207D Install noise barriers

# Modification History

| Release | Comment |
| --- | --- |
| 1 | This unit replaces RIICRC207A Install noise barriers |
| 2 | Required frequency and volume of evidence amended in Performance evidence. Substantial amendments made in Assessment Conditions field, including: references to Industry Sectors, assessor and subject matter expert experience requirements, how assessment should be conducted and what it should confirm. |

# Performance Evidence

Evidence is required to be collected that demonstrates a candidate’s competency in this unit. Evidence must be relevant to the roles within this sector’s work operations and satisfy all of the requirements of the performance criteria of this unit and include evidence that the candidate:

locates and applies applicable documentation, policies and procedures

implements the requirements, procedures and techniques for the safe, effective and efficient completion of noise barrier installation including:

accessing, interpreting and applying documentation relevant to the installation of noise barriers

identifying and applying relevant drawings and job specifications to all work activities

identifying, addressing and reporting potential hazards and risks

identifying, obtaining, confirming and implementing signage requirements

selecting and checking for faults all plant, tools and equipment to carry out tasks

identifying, addressing and reporting environmental issues and ground conditions

surveying work area for loading and unloading and stock-piling materials

correct use of plant and crane as required

works effectively with others to undertake and complete the installation of noise barriers in a way that meets all requirements including:

preparing for and organising work activities to meet all task requirements

communicating clearly and concisely with others to receive and clarify work instructions

using a range of communication techniques and aids to advise others of work activity and exclusion zones

monitoring hazards and identifying and communicating changes to the work environment during the work process

demonstrates completion of installing noise barriers that safely, effectively and efficiently meets all of the required outcomes on more than one (1) occasion including:

erecting noise barriers to meet drawings and job specifications requirements

clearing work area and disposing of, recycling or storing materials

as an individual or as a member of a two-three (2-3) person team, undertake and complete the installation of fifty (50) metres of noise barriers

# Knowledge Evidence

The candidate must demonstrate knowledge in installing noise barriers through:

accessing, interpreting and applying, organisation and site requirements and procedures for:

identifying and managing risks

identifying, addressing and reporting environmental issues and ground conditions

setting up work activity

site isolation and traffic control responsibilities

work health and safety

managing waste

selecting and checking for faults, tools and equipment to carry out tasks

confirming plant and crane capabilities as required

carrying out work to drawings and specifications

applying effective installation techniques

# Assessment Conditions

An assessor of this unit must satisfy the requirements of the NVR/AQTF or their successors; and Industry regulations for certification and licensing; and,

this unit must be assessed in the context of this sector’s work environment; and,

this unit must be assessed in compliance with relevant legislation/regulation and using policies, procedures, processes and operational manuals directly related to the industry sector for which it is being assessed; and,

assessment may be conducted in conjunction with the assessment of other Units of Competency; and,

assessment must confirm consistent performance can be applied in a range of relevant workplace circumstances; and,

assessors must demonstrate the performance evidence, and knowledge evidence as outlined in this Unit of Competency, and through the minimum years of current* work experience specified below in an Industry sector relevant to the outcomes of the unit; or,

where the assessor does not meet experience requirements a co-assessment or partnership arrangement must exist between the qualified assessor and an Industry subject matter expert. The Industry subject matter expert should hold the unit being assessed (or an equivalent unit) and/or demonstrate equivalence of skills and knowledge at the unit level. An Industry technical expert must also demonstrate skills and knowledge from the minimum years of current work experience specified below in the Industry sector, including time spent in roles related to the unit being assessed; and,

assessor and Industry subject matter expert requirements differ depending on the Australian Qualifications Framework Level (AQF) of the qualification being assessed and/or Industry Sector as follows:

| Industry sector | AQF** Level | Required assessor or Industry subject matter expert experience |
| --- | --- | --- |
| Drilling, Metalliferous Mining, Coal Mining, Extractive (Quarrying) and Civil Construction | 1 | 1 Year |
|  | 2 | 2 Years |
| Drilling, Coal Mining and Extractive (Quarrying) | 3-6 | 3 Years |
| Metalliferous Mining and Civil Construction | 3-6 | 5 Years |
| Other sectors | Where this Unit is being assessed outside of the Resources and Infrastructure Sectors assessor and/or Industry subject matter expert experience should be in-line with industry standards for the sector in which it is being assessed and where no Industry standard is specified should comply with any relevant regulation. |  |

*Assessors can demonstrate current work experience through employment within Industry in a role relevant to the outcomes of the Unit; or, for external assessors this can be demonstrated through exposure to Industry by conducting frequent site assessments across various locations.

**Where a unit is being delivered outside of a Qualification the first numeric character in the Unit code should be considered to indicate the AQF level

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=88a61002-9a21-4386-aaf8-69c76e675272