# Assessment Requirements for RIICRC302D Place and form concrete kerb, channel and fixtures

Release: 2

Assessment Requirements for RIICRC302D Place and form concrete kerb, channel and fixtures

# Modification History

| Release | Comment |
| --- | --- |
| 1 | This unit replaces RIICRC302A Place and form concrete kerb, channel and fixtures |
| 2 | Required frequency and volume of evidence amended in Performance evidence. Substantial amendments made in Assessment Conditions field, including: references to Industry Sectors, assessor and subject matter expert experience requirements, how assessment should be conducted and what it should confirm. |

# Performance Evidence

Evidence is required to be collected that demonstrates a candidate’s competency in this unit. Evidence must be relevant to the roles within this sector’s work operations and satisfy all of the requirements of the performance criteria of this unit and include evidence that the candidate:

locates and applies applicable documentation, policies and procedures

implements the requirements, procedures and techniques for the safe, effective and efficient completion of placing and forming concrete kerbs, channels and fixtures including:

accessing, interpreting and applying documentation relevant to the placing and forming concrete kerb, channel and fixtures

identifying and applying relevant drawings and job specifications to all work activities

identifying, addressing and reporting potential hazards and risks

identifying, obtaining, confirming and implementing signage requirements

selecting and checking for faults all plant, tools and equipment to carry out tasks

identifying, addressing and reporting environmental issues and ground conditions

works effectively with others to undertake and complete the placing and forming of concrete kerbs, channels and fixtures that meets all of the required outcomes including:

preparing for and organising work activities to meet all task requirements

communicating clearly and concisely with others to receive and clarify work instructions

using a range of communication techniques and aids to advise others of work activity and exclusion zones

monitoring hazards and identifying and communicating changes to the work environment during the work process

demonstrates completion of place and form concrete kerb, channel and fixtures that safely, effectively and efficiently meets all of the required outcomes on more than one (1) occasion including:

setting out and preparing for construction/installation in compliance with drawings and specifications requirements

forming kerb and barrier strips by placing concrete and applying mortar to specifications requirements

pouring concrete into installed formwork

finishing kerbs, channels and fixtures

installing pre-cast concrete units and finishing to specifications

repairing kerb, gutters and median, barrier strips

clearing, backfilling and finishing work areas

clearing work area and disposing of, recycling or storing materials

# Knowledge Evidence

The candidate must demonstrate knowledge in placing and forming concrete kerb, channel and fixtures through:

applying organisation and site requirements and procedures for:

identifying and managing risks

identifying, addressing and reporting environmental issues and ground conditions

setting up work activity

site isolation and traffic control responsibilities

work health and safety

managing waste

selecting and checking for faults, tools and equipment to carry out tasks

carrying out work to drawings and specifications

applying effective place and form concrete kerb, channel and fixture techniques

# Assessment Conditions

An assessor of this unit must satisfy the requirements of the NVR/AQTF or their successors; and Industry regulations for certification and licensing; and,

this unit must be assessed in the context of this sector’s work environment; and,

this unit must be assessed in compliance with relevant legislation/regulation and using policies, procedures, processes and operational manuals directly related to the industry sector for which it is being assessed; and,

assessment may be conducted in conjunction with the assessment of other Units of Competency; and,

assessment must confirm consistent performance can be applied in a range of relevant workplace circumstances; and,

assessors must demonstrate the performance evidence, and knowledge evidence as outlined in this Unit of Competency, and through the minimum years of current* work experience specified below in an Industry sector relevant to the outcomes of the unit; or,

where the assessor does not meet experience requirements a co-assessment or partnership arrangement must exist between the qualified assessor and an Industry subject matter expert. The Industry subject matter expert should hold the unit being assessed (or an equivalent unit) and/or demonstrate equivalence of skills and knowledge at the unit level. An Industry technical expert must also demonstrate skills and knowledge from the minimum years of current work experience specified below in the Industry sector, including time spent in roles related to the unit being assessed; and,

assessor and Industry subject matter expert requirements differ depending on the Australian Qualifications Framework Level (AQF) of the qualification being assessed and/or Industry Sector as follows:

| Industry sector | AQF** Level | Required assessor or Industry subject matter expert experience |
| --- | --- | --- |
| Drilling, Metalliferous Mining, Coal Mining, Extractive (Quarrying) and Civil Construction | 1 | 1 Year |
|  | 2 | 2 Years |
| Drilling, Coal Mining and Extractive (Quarrying) | 3-6 | 3 Years |
| Metalliferous Mining and Civil Construction | 3-6 | 5 Years |
| Other sectors | Where this Unit is being assessed outside of the Resources and Infrastructure Sectors assessor and/or Industry subject matter expert experience should be in-line with industry standards for the sector in which it is being assessed and where no Industry standard is specified should comply with any relevant regulation. |  |

*Assessors can demonstrate current work experience through employment within Industry in a role relevant to the outcomes of the Unit; or, for external assessors this can be demonstrated through exposure to Industry by conducting frequent site assessments across various locations.

**Where a unit is being delivered outside of a Qualification the first numeric character in the Unit code should be considered to indicate the AQF level

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=88a61002-9a21-4386-aaf8-69c76e675272