# Assessment Requirements for RIIERR305E Control underground fires

Release: 2

Assessment Requirements for RIIERR305E Control underground fires

# Modification History

| #### Release | #### Comments |
| --- | --- |
| 1 | This unit replaces RIIERR305D Control underground fires. Minor endorseable amendments have been made to Elements, Performance Criteria, Foundation Skills, Performance Evidence and Knowledge Evidence to better reflect current industry practices and clarify training outcomes. |
| 2 | Editorial corrections. |

# Performance Evidence

The candidate must demonstrate the ability to complete the tasks outlined in the elements, performance criteria and foundation skills of this unit, including evidence of the ability to:

control underground fires in a manner that safely, effectively and efficiently meets all of the required outcomes on at least two occasions, including:

locating firefighting equipment at the site

navigating in an underground site

reading maps and interpreting symbols

locating plant and fire suppression systems on a site layout plan

locating and operating fixed and portable fire suppression equipment

selecting and using appropriate extinguishing media

using and troubleshooting water supply systems and identifying alternative systems

determining personnel requirements

determining fire-fighting strategies and procedures, including evacuation and post-fire activities.

During the above, the candidate must:

locate and apply relevant documentation, policies and procedures

work effectively with others to undertake and complete the control of underground fires that meets all the required outcomes, including:

complying with written and verbal reporting requirements and procedures

communicating clearly and concisely with others to receive and clarify work instructions

communicating clearly and concisely with others to coordinate work activities

operating emergency communications systems.

# Knowledge Evidence

The candidate must be able to demonstrate knowledge to complete the tasks outlined in the elements, performance criteria and foundation skills of this unit, including knowledge of:

types and causes of hazards and incidents in underground sites

effects of hot and humid atmospheres in underground sites

location and distribution of caches

trigger points to initiate emergency response to alarms

locations and purposes of places of safety

guidance system and marker uses

key components and requirements of site emergency plans

locations of escape routes and alternative escape routes

functions of underground ventilation systems

types and effects of harmful gases in underground sites

rescue team structure, roles and responsibilities

operation and limitations of breathing apparatus

standby/emergency procedures for incident control

types of fire and associated risks and control measures

ventilation management techniques

properties of extinguishing media and the selection of those appropriate to the situation

fire-ground management procedures

sector-specific signals required for work activity

techniques for refurbishing fire-fighting equipment and systems.

# Assessment Conditions

Mandatory conditions for assessment of this unit are stipulated below.

The assessment must:

include access to:

site plans

breathing apparatus

personal protective equipment required for the activities described in the performance evidence

be conducted in a safe environment

be assessed in context of this sector’s work environment

be assessed in compliance with relevant legislation and regulations and using policies, procedures, processes and operational manuals directly related to the industry sector for which it is being assessed

confirm consistent performance can be applied in a range of relevant workplace circumstances.

Where personal safety or environmental damage are limiting factors, assessment may occur in a simulated work environment* provided it is realistic and sufficiently rigorous to cover all aspects of this sector’s workplace performance, including environment, task skills, task management skills, contingency management skills and job role environment skills.

Assessor Requirements

Assessors must be able to clearly demonstrate current and relevant industry knowledge and experience to satisfy the mandatory regulatory standards as set out in the Standards for Registered Training Organisations (RTOs) 2015/Australian Quality Training Framework mandatory requirements for assessors current at the time of assessment and any relevant licensing and certification requirements. This includes:

vocational competencies at least to the level being delivered and assessed

current industry skills directly relevant to the training and assessment being provided

current knowledge and skills in vocational training and learning that informs their training and assessment

formal relevant qualifications in training and assessment

having knowledge of and/or experience using the latest techniques and processes

possessing a high level of RII training product knowledge

having an understanding and knowledge of legislation and regulations relevant to the industry and to employment and workplaces

demonstrating the performance evidence, and knowledge evidence outlined in this unit of competency, and

the minimum years of current** work experience after competency has been obtained as specified below in an industry sector relevant to the outcomes of the unit.

It is also acceptable for the appropriately qualified assessor to work with an industry expert to conduct assessment together and for the industry expert to be involved in the assessment judgement. The industry expert must hold the relevant vocational competencies and have current industry skills directly relevant to the training and assessment being provided and must work alongside a trainer and/or assessor to conduct the assessment. This means the industry subject matter expert should hold the unit being assessed (or an equivalent unit), and must also demonstrate skills and knowledge from the minimum years of current work experience after competency has been obtained as specified below, including time spent in roles related to the unit being assessed:

| #### Industry sector | #### AQF indicator level*** | #### Required assessor or industry subject matter expert experience |
| --- | --- | --- |
| Drilling, Metalliferous Mining, Coal Mining, Extractive (Quarrying) and Civil Infrastructure | 1 | 1 year |
|  | 2 | 2 years |
| Drilling, Coal Mining, Extractive (Quarrying), Metalliferous Mining and Civil Infrastructure | 3-6 | 3 years |
| Other sectors | Where this unit is being assessed outside of the resources and infrastructure sectors assessor and/or industry subject matter expert experience should be in-line with industry standards for the sector in which it is being assessed and where no industry standard is specified should comply with any relevant regulation. |  |

*Guidance on simulated environments has been stipulated in the RII Companion Volume Implementation Guide located on VETNet.

**Assessors can demonstrate current work experience through employment within industry in a role relevant to the outcomes of the unit; or, for external assessors this can be demonstrated through exposure to industry by conducting a minimum number of site assessments as determined by the relevant industry sector, across various locations.

*** While a unit of competency does not have an AQF level, where a unit is being delivered outside of a qualification the first numeric character in the unit code should be considered as the AQF indicator level for assessment purposes.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=88a61002-9a21-4386-aaf8-69c76e675272