# Assessment Requirements for RIIMCU408D Apply the spontaneous combustion management plan

Release: 3

Assessment Requirements for RIIMCU408D Apply the spontaneous combustion management plan

# Modification History

| Release | Comment |
| --- | --- |
| 1 | This unit replaces RIIMCU408A Apply the spontaneous combustion management plan |
| 2 | Plan and Prepare element added to Performance Criteria; Inserted Modification History |
| 3 | Required frequency and volume of evidence amended in Performance evidence. Substantial amendments made in Assessment Conditions field, including: references to Industry Sectors, assessor and subject matter expert experience requirements, how assessment should be conducted and what it should confirm. |

# Performance Evidence

Evidence is required to be collected that demonstrates a candidate’s competency in this unit. Evidence must be relevant to the roles within this sector’s work operations and satisfy all of the requirements of the performance criteria of this unit and include evidence that the candidate:

locates and applies relevant legislation, documentation, policies and procedures

implements the requirements, procedures and techniques for the safe, effective and efficient completion of the application of the spontaneous combustion management plan including:

accessing, reading, interpreting, communicating and applying:

technical information

records and reports

briefing and handover details

planning and coordinating work

conducting mine atmosphere monitoring

conducting inspections related to spontaneous combustion

interpreting and recording data related to spontaneous combustion

interpreting and responding to spontaneous combustion indicators and ratios

identifying and investigating incidents and potential hazards associated with spontaneous combustion

identifying training needs, and preparing and implementing training plans

applying spontaneous combustion control procedures or methods

works effectively with others to plan, prepare and conduct the application of the spontaneous combustion management plan that meets all of the required outcomes including:

organising work activities to meet all task requirements

communicating clearly and concisely with others to receive and clarify work instructions

complying with written and verbal reporting requirements and procedures

resolving coordination requirements throughout work activities

demonstrates completion of applying the spontaneous combustion management plan that safely, effectively and efficiently meets all of the required outcomes on more than one (1) occasion including:

applying spontaneous combustion management plan procedures such as:

mine atmosphere monitoring

reporting requirements

auditing

ventilation systems and usage

inertisation techniques

mine plan

action plans

response plans

emergency procedures

individual group responsibilities

training and education procedures

applying mine atmosphere monitoring such as:

continuous monitoring

portable (hand) monitoring

collection of bag samples

gas chromatography

ventilation measurements from all areas of the mine including sealed areas

waste workings monitoring

identifying spontaneous combustion indicators such as:

smoke

haze

sweating

smell

heat

production of carbon monoxide

hydrogen

carbon dioxide

methane

higher hydrocarbons

Or use of indicator ratios such as:

CO make

Graham’s ratio

other ratios included in the site safety management system

# Knowledge Evidence

The candidate must demonstrate knowledge of applying the spontaneous combustion management plan through:

accessing, interpreting and applying legislative, organization and site requirements and procedures for:

applying risk assessment and management processes

identifying and managing environmental issues, hazards and risks, and identifying environmental monitoring systems

identifying training systems

identifying the impact of coal seam characteristics, depositional factors and geological conditions on spontaneous combustion

identifying and applying the fundamentals of:

mine design and plan relating to spontaneous combustion

mine gases

mine and goaf ventilation systems

spontaneous combustion causes and hazards

methods of control of spontaneous combustion

spontaneous combustion indicators and trigger points

sealing procedures

identifying and applying emergency response and evacuation/disaster planning processes and techniques

# Assessment Conditions

An assessor of this unit must satisfy the requirements of the NVR/AQTF or their successors; and Industry regulations for certification and licensing; and,

this unit must be assessed in the context of this sector’s work environment; and,

this unit must be assessed in compliance with relevant legislation/regulation and using policies, procedures, processes and operational manuals directly related to the industry sector for which it is being assessed; and,

assessment may be conducted in conjunction with the assessment of other Units of Competency; and,

assessment must confirm consistent performance can be applied in a range of relevant workplace circumstances; and,

assessors must demonstrate the performance evidence, and knowledge evidence as outlined in this Unit of Competency, and through the minimum years of current* work experience specified below in an Industry sector relevant to the outcomes of the unit; or,

where the assessor does not meet experience requirements a co-assessment or partnership arrangement must exist between the qualified assessor and an Industry subject matter expert. The Industry subject matter expert should hold the unit being assessed (or an equivalent unit) and/or demonstrate equivalence of skills and knowledge at the unit level. An Industry technical expert must also demonstrate skills and knowledge from the minimum years of current work experience specified below in the Industry sector, including time spent in roles related to the unit being assessed; and,

assessor and Industry subject matter expert requirements differ depending on the Australian Qualifications Framework Level (AQF) of the qualification being assessed and/or Industry Sector as follows:

| Industry sector | AQF** Level | Required assessor or Industry subject matter expert experience |
| --- | --- | --- |
| Drilling, Metalliferous Mining, Coal Mining, Extractive (Quarrying) and Civil Construction | 1 | 1 Year |
|  | 2 | 2 Years |
| Drilling, Coal Mining and Extractive (Quarrying) | 3-6 | 3 Years |
| Metalliferous Mining and Civil Construction | 3-6 | 5 Years |
| Other sectors | Where this Unit is being assessed outside of the Resources and Infrastructure Sectors assessor and/or Industry subject matter expert experience should be in-line with industry standards for the sector in which it is being assessed and where no Industry standard is specified should comply with any relevant regulation. |  |

*Assessors can demonstrate current work experience through employment within Industry in a role relevant to the outcomes of the Unit; or, for external assessors this can be demonstrated through exposure to Industry by conducting frequent site assessments across various locations.

**Where a unit is being delivered outside of a Qualification the first numeric character in the Unit code should be considered to indicate the AQF level

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=88a61002-9a21-4386-aaf8-69c76e675272