# RIIMPO338E Conduct rigid haul truck operations

Release: 1

RIIMPO338E Conduct rigid haul truck operations

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version first released with RII Resources and Infrastructure Industry Training Package Version 3.0. |

# Application

This unit describes the skills and knowledge required to operate, load, haul and dump materials using a rigid haul truck.

This unit applies to those who work in operational roles.

Licensing, legislative, regulatory and certification requirements that apply to this unit can vary between states, territories, and industry sectors, and must be sourced from state jurisdictions prior to applying this unit.

# Unit Sector

Civil infrastructure

Coal mining

Extractive

Metalliferous mining

# Elements and Performance Criteria

| ELEMENT | PERFORMANCE CRITERIA |
| --- | --- |
| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. |
| 1. Plan and prepare for rigid haul truck operations | 1.1 Access, interpret and apply conducting rigid haul truck operations documentation 1.2 Obtain, interpret, clarify and confirm work requirements 1.3 Identify hazards and environmental issues, assess risk and implement control measures 1.4 Select and wear personal protective equipment required for the work activities 1.5 Access, interpret and apply required geological and survey data 1.6 Coordinate and communicate planned activities with others at the site prior to commencement of work activity |
| 2. Operate rigid haul truck in line with established requirements | 2.1 Carry out prestart and start-up checks in line with workplace procedures 2.2 Identify faults or defects and rectify or report within scope of own responsibility and according to workplace procedures 2.3 Drive and operate rigid haul truck using techniques suited to equipment capabilities, site and work conditions, and according to workplace procedures 2.4 Monitor and manage equipment performance using indicators and alarms 2.5 Monitor hazards and risks during operation, and ensure safety to self, others, plant and equipment |
| 3. Load, haul and dump materials to complete work activity | 3.1 Position rigid haul truck under loading equipment and load vehicle 3.2 Carry out haulage operations in line with workplace procedures 3.3 Carry out approved dumping operations 3.4 Park up, shut down, secure and carry out post operational inspection of equipment in line with workplace procedures |
| 4. Conduct housekeeping activities | 4.1 Clear work area in line with workplace procedures 4.2 Manage and/or report hazards to maintain a safe working environment 4.3 Complete and file or distribute documentation |

# Foundation Skills

This section describes those language, literacy, numeracy and employment skills that are essential to performance.

| Skill | Description |
| --- | --- |
| Reading | Identifies and interprets information from workplace procedures, documentation, legislation and regulations |
| Numeracy | Uses equipment operating capacity schedule to confirm safe weight load limits |

# Unit Mapping Information

| #### Code and title current version | #### Code and title previous version | #### Comments | #### Equivalence status |
| --- | --- | --- | --- |
| RIIMPO338E Conduct rigid haul truck operations (Release 1) | RIIMPO338D Conduct rigid haul truck operations (Release 4) | Minor updates to all sections to reflect changes to operator maintenance | Equivalent unit |

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=88a61002-9a21-4386-aaf8-69c76e675272