# RIINHB205E Assist continuous flight auger drilling

Release: 1

RIINHB205E Assist continuous flight auger drilling

# Modification History

| #### Release | #### Comment |
| --- | --- |
| 1 | This unit replaces RIINHB205D Assist continuous flight auger drilling. Element related to sampling removed and corresponding amendments made to performance and knowledge evidence. |

# Application

This unit describes a participant’s skills and knowledge required to assist continuous flight auger drilling in the Resources and Infrastructure Industry.

This unit is appropriate for those working in drillers assistant roles.

No licensing, legislative or certification requirements apply to this unit at the time of publication.

# Elements and Performance Criteria

| 1. Plan and prepare for assisting with continuous flight auger drilling | 1.1 Access, interpret and apply assist continuous flight auger drilling documentation and ensure the work activity is compliant 1.2 Obtain, read, interpret, clarify and confirm work requirements 1.3 Identify and address potential risks, hazards and environmental issues and implement control measures 1.4 Select and wear appropriate personal protective equipment and protective clothing 1.5 Obtain and interpret emergency procedures, and be prepared for fire/accident/emergency 1.6 Communicate and coordinate activities with others throughout the work activity 1.7 Load, unload, move, handle, use and store continuous flight augers and all associated tools, sampling devices and connecting equipment 1.8 Set up and stabilise auger racks |
| --- | --- |
| 2. Assist with augering process | 2.1 Fit and remove bits to and from the lead auger 2.2 Lay out auger string in readiness for auger process 2.3 Insert and remove augers from the drill string and assist with auger to auger connections 2.4 Regularly maintain cleanliness around the hole collar by removing spoil only when rotation is stopped 2.5 Observe housekeeping and site safety measures while conducting auger assistance and sampling duties |
| 3. Clean equipment | 3.1 Clean and reassemble sampling devices and associated equipment 3.2 Use high pressure cleaners and/or steam cleaning equipment to clean augers and equipment 3.3 Avoid or confine the spread of contamination from auger and equipment cleaning processes. 3.4 Follow safe work practices for use of cleaning equipment, chemicals and materials. 3.5 Bag, remove and dispose of or neutralise excess spoil from site operations |
| 4. Carry out water sampling | 4.1 Bail or pump holes in preparation for collection of water sample 4.2 Prepare, obtain and handle water sample bottles 4.3 Obtain a water sample from a bore hole 4.4 Fill, seal, label, store and transport correct volume water samples relevant for the analytical purpose or tests required using appropriate type containers 4.5 Remove or dispose of or neutralise excess water generated by sampling or cleaning processes that may be contaminated or harmful to the environment, plants, native animals, domestic stock or people |
| 5. Carry out basic maintenance of tools and equipment | 5.1 Perform inspection and checks on serviceability of augers including condition of flights, threads, socket connectors, D clips and bits 5.2 Inspect serviceability of pressure cleaning equipment, water sampling pumps, sample tubes, SPT equipment 5.3 Maintain all auger and sampling equipment in serviceable condition 5.4 Conduct housekeeping activities |

# Foundation Skills

Foundation skills essential to performance are explicit in the performance criteria of this unit. Further information is available in the Resources and Infrastructure Industry Training Package Companion Volume.

# Unit Mapping Information

| #### Code and title #### current version | #### Code and title #### previous version | #### Comments | #### Equivalence status |
| --- | --- | --- | --- |
| RIINHB205E Assist continuous flight auger drilling | RIINHB205D Assist continuous flight auger drilling | Element removed. Performance and knowledge evidence amended | Not equivalent |

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=88a61002-9a21-4386-aaf8-69c76e675272

Assessment Requirements for RIINHB205E Assist continuous flight auger drilling

# Modification History

| #### Release | #### Comment |
| --- | --- |
| 1 | This unit replaces RIINHB205D Assist continuous flight auger drilling. Element related to sampling removed and corresponding amendments made to performance and knowledge evidence. |

# Performance Evidence

Evidence is required to be collected that demonstrates a candidate’s competency in this unit. Evidence must be relevant to the roles within this sector’s work operations and satisfy all of the requirements of the performance criteria of this unit and include evidence that the candidate:

locates and applies relevant documentation, policies and procedures

implements the requirements, procedures and techniques for the safe, effective and efficient assisting with continuous flight auger drilling including:

operating ancillary equipment

applying basic maintenance and servicing of auxiliary equipment

measuring and identifying flight auger components

identifying thread types in use on site

identifying bits in use and how to measure them

applying safe storage of tools

using load handling equipment on site

assisting the driller in the removal and adding of drill rods to the line string

refuelling vehicles, drill rigs and ancillary equipment

identifying and applying correct lubricants

conducting housekeeping

installing restraining devices to pressure and delivery hoses

identification of bits to suit differing ground conditions

works effectively with others to undertake and complete the continuous flight auger drilling tasks that meets all of the required outcomes including:

complying with written and verbal reporting requirements and procedures

communicating clearly and concisely with others to receive and clarify work instructions

communicating clearly and concisely with others to resolve coordination requirements throughout work activities

demonstrates completion of assisting continuous flight auger drilling that safely, effectively and efficiently meets all of the required outcomes on more than one (1) occasion including:

fitting and removing bits to and from the lead auger

laying out auger string in readiness for auger process

inserting removing augers from the drill string and assisting with auger to auger connections

# Knowledge Evidence

The candidate must demonstrate knowledge of assisting continuous flight auger drilling through:

organisation and site requirements and procedures for:

work, health and safety

environment issues

test bore and well abandonment

basic soil description methods

calculation of volume in cylinders, tanks

# Assessment Conditions

Assessors must satisfy the Standards for Registered Training Organisations (RTOs) 2015/AQTF mandatory competency requirements for assessors; and industry regulations for certification and licensing; and,

this unit must be assessed in the context of this sector’s work environment; and,

this unit must be assessed in compliance with relevant legislation/regulation and using policies, procedures, processes and operational manuals directly related to the industry sector for which it is being assessed; and,

assessment may be conducted in conjunction with the assessment of other units of competency; and,

assessment must confirm consistent performance can be applied in a range of relevant workplace circumstances; and,

assessors must demonstrate the performance evidence, and knowledge evidence as outlined in this unit of competency, and through the minimum years of current* work experience specified below in an industry sector relevant to the outcomes of the unit; or,

where the assessor does not meet experience requirements a co-assessment or partnership arrangement must exist between the qualified assessor and an industry subject matter expert. The industry subject matter expert should hold the unit being assessed (or an equivalent unit) and/or demonstrate equivalence of skills and knowledge at the unit level. An industry technical expert must also demonstrate skills and knowledge from the minimum years of current work experience specified below in the industry sector, including time spent in roles related to the unit being assessed; and,

assessor and industry subject matter expert requirements differ depending on the Australian Qualifications Framework Level (AQF) of the qualification being assessed and/or industry sector as follows:

| #### Industry sector | #### AQF** Level | #### Required assessor or industry subject matter expert experience |
| --- | --- | --- |
| Drilling, Metalliferous Mining, Coal Mining, Extractive (Quarrying) and Civil Construction | 1 | 1 Year |
|  | 2 | 2 Years |
| Drilling, Coal Mining and Extractive (Quarrying) | 3-6 | 3 Years |
| Metalliferous Mining and Civil Construction | 3-6 | 5 Years |
| Other sectors | Where this unit is being assessed outside of the Resources and Infrastructure Sectors, assessor and/or industry subject matter expert experience should be in-line with industry standards for the sector in which it is being assessed and, where no industry standard is specified, should comply with any relevant regulation. |  |

*Assessors can demonstrate current work experience through employment within industry in a role relevant to the outcomes of the unit; or, for external assessors this can be demonstrated through exposure to industry by conducting frequent site assessments across various locations.

**Where a unit is being delivered outside of a qualification the first numeric character in the unit code should be considered to indicate the AQF level.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=88a61002-9a21-4386-aaf8-69c76e675272