# RIIRAI611D Establish and maintain a surface mine mechanical plant management system

Release: 3

RIIRAI611D Establish and maintain a surface mine mechanical plant management system

# Modification History

| #### Release | #### Comment |
| --- | --- |
| 1 | This unit replaces RIIRIS611A Establish and maintain a surface mine mechanical plant management system |
| 2 | Plan and Prepare element added; removed Install element on the basis that this is an Establish and Maintain unit. |
| 3 | Required frequency and volume of evidence amended in Performance evidence. Substantial amendments made in Assessment Conditions field, including: references to Industry Sectors, assessor and subject matter expert experience requirements, how assessment should be conducted and what it should confirm. |

# Application

This unit describes a participant’s skills and knowledge required to establish and maintain a surface mine mechanical plant management system in Coal mining.

This unit is appropriate for those working in management or technical specialist roles.

No licensing, legislative or certification requirements apply to this unit at the time of publication.

# Unit Sector

Coal mining

# Elements and Performance Criteria

| 1. Plan and prepare surface mine mechanical plant management system | 1.1 Access, interpret and apply surface mine mechanical plant management system documentation and ensure the work activity is compliant 1.2 Obtain, read, interpret, clarify and confirm work requirements 1.3 Identify and address potential risks, hazards and environmental issues and implement control measures 1.4 Identify design/selection specifications and critical aspects of the surface mine mechanical plant management system from a comprehensive analysis of operating requirements 1.5 Identify systems options from an analysis of all relevant technical, operational and financial information 1.6 Select the preferred systems options on the basis of performance against specification requirements 1.7 Develop, in consultation, document and have approved policies and system procedures 1.8 Define, document and have approved, roles and responsibilities of all relevant personnel 1.9 Identify, source and provide the resources for the operation of the system |
| --- | --- |
| 2. Identify, analyse and evaluate hazards and risks associated with the use of mechanical equipment in a surface coal mine | 2.1 Identify, evaluate and clarify mine conditions which contribute to hazardous environments 2.2 Identify, analyse and evaluate hazards and risks relating to mechanical plant 2.3 Identify, analyse and evaluate hazards and risks relating to fire initiate and/or supported by mechanical equipment 2.4 Identify, analyse and evaluate hazards and risks relating to storage of dangerous goods |
| 3. Identify, analyse and evaluate hazards and risk control options and measures | 3.1 Identify, analyse and evaluate hazardous environment control techniques 3.2 Identify, analyse and evaluate fire protection and prevention methods and systems on mechanical equipment 3.3 Identify, analyse and evaluate methods and systems to control noise, dust, and other emissions initiated from or created by mechanical equipment 3.4 Identify, analyse and evaluate methods for working at heights 3.5 Develop systems to maintain the integrity of safety critical functions of mechanical equipment 3.6 Develop systems to control hazards in lifting and towing systems for mechanical equipment 3.7 Develop systems to control hazards associated with transport equipment and other plant 3.8 Develop systems to protect people in their interrelationship with mechanical equipment in the surface environment 3.9 Evaluate the uses of mine monitoring and information systems to control hazards and risks on mechanical equipment 3.10 Develop systems to control hazards for energy dissipation and isolation systems on mechanical equipment 3.11 Identify, analyse and evaluate methods and systems to control hazardous substances initiated from or used by mechanical equipment 3.12 Develop systems for the interaction and/or failure of transport and production equipment |
| 4. Establish the mine mechanical plant management system | 4.1 Establish mechanical plant objectives, systems, descriptions and responsibilities and incorporate into the mine mechanical plant management system 4.2 Consult with relevant stakeholders in the development of the mine mechanical plant management system 4.3 Design mechanical plant management systems to satisfy the operational systems and conditions of the mine 4.4 Incorporate maintenance and inspections systems to ensure the equipment is safe to operate 4.5 Incorporate systems to monitor and test the performance of safety critical functions on mechanical plant and equipment 4.6 Incorporate systems to control noise, dust, other emissions and other hazardous substances 4.7 Incorporate systems to develop safe work procedure 4.8 Incorporate systems for the management of mechanical contractors 4.9 Develop and establish mechanical engineering standards 4.10 Develop and establish mechanical equipment information recording and reporting procedures, including defect reporting and rectification 4.11 Establish a program, including systems and procedures, to satisfy mechanical equipment training requirements 4.12 Incorporate audit and review requirements and update procedures into the mine mechanical plant management system |
| 5. Audit and review the mechanical plant management system | 5.1 Audit mine mechanical engineering standards for compliance with statutory and mine site requirements 5.2 Audit the maintenance program and procedures for compliance with the mine mechanical plant management system and statutory requirements 5.3 Audit recording and reporting systems for compliance with the mine mechanical plant management system 5.4 Audit mechanical plant management training program for currency, relevance and compliance with the requirements of the mine mechanical plant management system 5.5 Establish procedures for non-compliances revealed by audit and modify the management plan where appropriate 5.6 Review the mine mechanical plant management system |

# Foundation Skills

Foundation skills essential to performance are explicit in the performance criteria of this unit. Further information is available in the Resources and Infrastructure Industry Training Package Companion Volume.

# Unit Mapping Information

No equivalent unit

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=88a61002-9a21-4386-aaf8-69c76e675272

Assessment Requirements for RIIRAI611D Establish and maintain a surface mine mechanical plant management system

# Modification History

| #### Release | #### Comment |
| --- | --- |
| 1 | This unit replaces RIIRIS611A Establish and maintain a surface mine mechanical plant management system |
| 2 | Plan and Prepare element added; removed Install element on the basis that this is an Establish and Maintain unit. |
| 3 | Required frequency and volume of evidence amended in Performance evidence. Substantial amendments made in Assessment Conditions field, including: references to Industry Sectors, assessor and subject matter expert experience requirements, how assessment should be conducted and what it should confirm. |

# Performance Evidence

Evidence is required to be collected that demonstrates a candidate’s competency in this unit. Evidence must be relevant to the roles within this sector’s work operations and satisfy all of the requirements of the performance criteria of this unit and include evidence that the candidate:

locates and applies relevant legislation, documentation, policies and procedures

implements procedures and techniques for the safe, effective and efficient completion of the establishing and maintaining of a surface mine mechanical plant management system including:

accessing, reading, interpreting and applying procedures for;

managing technical information

managing site/legislative requirements

establishing recording and reporting systems

conducting briefings and handover details

applying the principles of mine design

hazard identification and risk management

work planning and coordination

training needs analysis

using manufacturers’ instructions

using effective communication

works effectively with others to establish and maintain a surface mine mechanical plant management system including:

establishing training to support, maintain and improve activity

managing people and processes

resolving conflict

coordinating human, financial and physical resources

developing, initiating and administering work plans

interpreting and applying operational performance data

engaging internal and external stakeholders

demonstrates completion of establishing and maintaining a surface mine mechanical plant management system that safely, effectively and efficiently meets all of the required outcomes on more than one (1) occasion including:

identifying the relevant information and scope of the work

identifying the full range of specification requirements

identifying all aspects of the surface mine mechanical plant

selecting the preferred management system options

establishing and implementing the mine mechanical plant management system which establishes procedures for maintaining safe equipment including:

hazard identification and quantification

risk management

consultation

authority and responsibility

controls established to manage identified risks

reporting and communication

document control

audit and review

identifying options to control hazards and risks relating to mine mechanical plant and equipment which are analysed and clarified in the mechanical plan management system including:

explosion initiate from mechanical equipment

fire initiate and/or supported by mechanical equipment

hazardous substances used for mechanical equipment

toxic substances generated by mechanical equipment

the release of uncontrolled kinetic energy (unplanned movements) on mechanical equipment

the interaction and/or failure of transport and production equipment

the use and storage of high pressure fluids

moving and rotating parts on mechanical equipment

fluid injection incidents

dangerous goods, e.g. LPG, flammable liquids

lifting and towing systems

hazards associated with transport of equipment and other plant

interrelationship between people and mechanical equipment

auditing and reviewing the mechanical plant management system

# Knowledge Evidence

The candidate must demonstrate knowledge in establishing and maintaining a surface mine mechanical plant management system through:

legislative, organisation and site requirements and procedures for;

mine operation

environmental monitoring

risk management

emergency response and evacuation

mine plans

firefighting systems

transport rules

maintenance operations

standard operating procedures

training

mine design relating to production and transport systems and equipment

production and transport systems and equipment management requirements

production and transport equipment and systems (types, uses, characteristics and limitations appropriate for safe operation at the mine site)

energy sources, including protection and reticulation systems for electrical, hydraulic, compressed air, diesel

information on mine fires; the types, sources of ignition and possible impacts on plant and equipment

accessing, reading, interpreting and applying safety design features of production and transport systems

using stores system

specification design criteria, including noise, dust, lighting, ergonomics, remote control, physical clearance, confined space, visibility, seating vibration and machine equipment and personal protection

training programs

computer based systems relating to production and transport systems

# Assessment Conditions

An assessor of this unit must satisfy the requirements of the NVR/AQTF or their successors; and Industry regulations for certification and licensing; and,

this unit must be assessed in the context of this sector’s work environment; and,

this unit must be assessed in compliance with relevant legislation/regulation and using policies, procedures, processes and operational manuals directly related to the industry sector for which it is being assessed; and,

assessment may be conducted in conjunction with the assessment of other Units of Competency; and,

assessment must confirm consistent performance can be applied in a range of relevant workplace circumstances; and,

assessors must demonstrate the performance evidence, and knowledge evidence as outlined in this Unit of Competency, and through the minimum years of current* work experience specified below in an Industry sector relevant to the outcomes of the unit; or,

where the assessor does not meet experience requirements a co-assessment or partnership arrangement must exist between the qualified assessor and an Industry subject matter expert. The Industry subject matter expert should hold the unit being assessed (or an equivalent unit) and/or demonstrate equivalence of skills and knowledge at the unit level. An Industry technical expert must also demonstrate skills and knowledge from the minimum years of current work experience specified below in the Industry sector, including time spent in roles related to the unit being assessed; and,

assessor and Industry subject matter expert requirements differ depending on the Australian Qualifications Framework Level (AQF) of the qualification being assessed and/or Industry Sector as follows:

| Industry sector | AQF** Level | Required assessor or Industry subject matter expert experience |
| --- | --- | --- |
| Drilling, Metalliferous Mining, Coal Mining, Extractive (Quarrying) and Civil Construction | 1 | 1 Year |
|  | 2 | 2 Years |
| Drilling, Coal Mining and Extractive (Quarrying) | 3-6 | 3 Years |
| Metalliferous Mining and Civil Construction | 3-6 | 5 Years |
| Other sectors | Where this Unit is being assessed outside of the Resources and Infrastructure Sectors assessor and/or Industry subject matter expert experience should be in-line with industry standards for the sector in which it is being assessed and where no Industry standard is specified should comply with any relevant regulation. |  |

*Assessors can demonstrate current work experience through employment within Industry in a role relevant to the outcomes of the Unit; or, for external assessors this can be demonstrated through exposure to Industry by conducting frequent site assessments across various locations.

**Where a unit is being delivered outside of a Qualification the first numeric character in the Unit code should be considered to indicate the AQF level

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=88a61002-9a21-4386-aaf8-69c76e675272