# RIIRTM301E Operate truck or trailer mounted attenuators

Release: 1

RIIRTM301E Operate truck or trailer mounted attenuators

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version first released with RII Resources and Infrastructure Industry Training Package Version 6.0. |

# Application

This unit describes the skills and knowledge required to operate truck or trailer mounted attenuators when carrying out traffic management activities in civil construction. It includes planning and preparing, using radio communication, operating and positioning truck or trailer mounted attenuator, using traffic controls, and cleaning up.

It applies to those working in operational roles. They generally work under minimal supervision to undertake a broad range of tasks in varied work contexts, using some discretion and judgement in selecting equipment, services or contingency measures.

Licensing, legislative and certification requirements that apply to this unit can vary between states, territories, and industry sectors. Users must check requirements with relevant body before applying the unit.

# Unit Sector

Civil construction

# Elements and Performance Criteria

| ELEMENT | PERFORMANCE CRITERIA |
| --- | --- |
| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. |
| 1. Plan and prepare to operate attenuator | 1.1 Obtain, interpret, clarify and confirm safety requirements from site safety plan, vehicle movement plan and organisational policies and procedures and apply to the allotted task 1.2 Access, interpret and apply documentation required to operate truck or trailer mounted attenuators and confirm work activity is compliant 1.3 Identify, obtain and implement signage according to job requirements 1.4 Select vehicles, plant, tools and equipment required for work activities, confirm serviceability and rectify and report faults as required 1.5 Identify, confirm, clarify and apply environmental protection requirements 1.6 Obtain and interpret emergency procedures and identify steps required to respond to emergencies |
| 2. Conduct attenuator pre-operational checks | 2.1 Check attenuator connection with host vehicle, tyres and brakes according to trailer mounted attenuator pre-start check 2.2 Carry out start-up, park and shutdown procedures according to manufacturer specifications and site requirements 2.3 Check truck and attenuator controls and functions for serviceability and report and rectify any faults as required |
| 3. Operate communication devices | 3.1 Adjust communication device such as radio controls for transmission and reception 3.2 Test and confirm radio contact between all vehicles and handheld radios 3.3 Transmit radio messages according to job requirements 3.4 Confirm communications contact after nominated period of non-contact 3.5 Confirm radio communication is maintained between vehicles and confirm correct positioning of vehicle |
| 4. Operate mounted attenuator | 4.1 Identify site hazards associated with truck or trailer mounted attenuator operations 4.2 Identify and apply safe operating techniques for truck or trailer mounted attenuator according to national truck or trailer mounted attenuator guidelines 4.3 Operate truck or trailer mounted attenuator according to job requirements |
| 5. Position traffic controls and truck or trailer mounted attenuator | 5.1 Position and confirm traffic control signs and display on vehicles 5.2 Display and confirm required vehicle warning lights 5.3 Position vehicle according to traffic management plan, and traffic and site safety regulations 5.4 Confirm traffic is controlled and work crew protected according to job requirements 5.5 Move truck or trailer mounted attenuator between worksites, observing road rules and traffic management requirements 5.6 Confirm incidents are recorded and reported as required |
| 6. Carry out operator maintenance | 6.1 Conduct inspection and fault finding according to manufacturer specifications and site requirements 6.2 Complete required documentation according to site requirements |
| 7. Conduct clean-up activities | 7.1 Confirm work area is clear and dispose of or recycle materials according to site requirements 7.2 Confirm tools and equipment are cleaned, checked, maintained and stored according to manufacturer specifications and site requirements |

# Foundation Skills

This section describes those language, literacy, numeracy and employment skills that are essential to performance but not explicit in the performance criteria.

| SKILL | DESCRIPTION |
| --- | --- |
| Oral communication | Conveys information and instructions to other verbally |
| Self-management | Monitors and minimises own exposure to worksite risks and hazards during work activities |

# Unit Mapping Information

Supersedes and is equivalent to RIIRTM301D Operate a truck or trailer mounted attenuator.

# Links

Companion Volume Implementation Guide is found on VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=88a61002-9a21-4386-aaf8-69c76e675272

Assessment Requirements for RIIRTM301E Operate truck or trailer mounted attenuators

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version first released with RII Resources and Infrastructure Industry Training Package Version 6.0. |

# Performance Evidence

The candidate must demonstrate the ability to complete the tasks outlined in the elements, performance criteria and foundation skills of this unit, including evidence of the ability to:

operate a truck or trailer mounted attenuator on at least two occasions at three different road sites, at least one site which will include working in a mobile or progressively moving work situation.

During the above, the candidate must:

locate and apply required legislation, documentation, policies and procedures, including National Guidelines and Standards

implement the requirements, procedures and techniques required to operate a truck or trailer mounted attenuator, including:

positioning other traffic controls and signs as required in the traffic management plan

conducting truck and attenuator pre-operational checks, including any attachments and reporting any faults

closing lanes

work with others to operate a truck or trailer mounted attenuator that meets required outcomes, including:

using communication techniques and equipment to convey information to others

participating in pre-work safety meetings, toolbox meetings or similar

completing written and verbal reporting requirements

organising work activities to meet task requirements

communicating with others to receive and clarify work instructions.

# Knowledge Evidence

The candidate must be able to demonstrate knowledge to complete the tasks outlined in the elements, performance criteria and foundation skills of this unit, including knowledge of:

legislation to operate a truck or trailer mounted attenuator

policies, procedures and documentation required to operate a truck or trailer mounted attenuator, including those relating to:

National Guidelines and Standards

work health and safety

risk assessment and management

communication techniques and equipment

manufacturer specifications for required plant and equipment

workplace recording and reporting

site requirements

site traffic management and safety plans

vehicle movement plan

site and equipment safety

emergency response

environmental protection

response to and contingency plans for impact incidents

quality management

vehicle warning lights

workplace documentation

Job Safety Analyses (JSA)s/Safe Work Method Statements (SWMS)

checking truck and attenuator controls and functions for serviceability, including:

raising and lowering attenuator

video cameras and other attachments

brakes and manoeuvrability

procedures for accessing and interpreting project traffic management plans

procedures for closing lanes

displaying and checking vehicle warning lights including arrow boards and rotating beacon warning lights

mobile traffic control and general traffic control responsibilities and authorities

types, characteristics, technical capabilities and limitations of plant and equipment required to operate truck or trailer mounted attenuators

processes for calculating travel speed

civil infrastructure terminology relevant to operating a truck or trailer mounted attenuator.

# Assessment Conditions

Mandatory conditions for assessment of this unit are stipulated below. The assessment must:

include access to:

personal protective equipment

equipment required to operate a truck or trailer mounted attenuator

be conducted in a safe environment; and,

be assessed in the context of this sector’s work environment; and,

be assessed in compliance with relevant legislation/regulation and using policies, procedures and processes directly related to the industry sector for which it is being assessed; and,

confirm consistent performance can be applied in a range of relevant workplace circumstances.

Where personal safety or environmental damage are limiting factors, assessment may occur in a simulated work environment* provided it is realistic and sufficiently rigorous to cover all aspects of this sector’s workplace performance, including environment, task skills, task management skills, contingency management skills and job role environment skills.

Assessor requirements

Assessors must be able to clearly demonstrate current and relevant industry knowledge and experience to satisfy the mandatory regulatory standards as set out in the Standards for Registered Training Organisations 2015/Australian Quality Training Framework mandatory requirements for assessors current at the time of assessment and any relevant licensing and certification requirements. This includes:

vocational competencies at least to the level being delivered and assessed

current industry skills directly relevant to the training and assessment being provided

current knowledge and skills in vocational training and learning that informs their training and assessment

formal relevant qualifications in training and assessment

having knowledge of and/or experience using the latest techniques and processes

possessing the required level of RII training product knowledge

having an understanding and knowledge of legislation and regulations relevant to the industry and to employment and workplaces

demonstrating the performance evidence, and knowledge evidence outlined in this unit of competency, and

the minimum years of current** work experience after competency has been obtained as specified below in an industry sector relevant to the outcomes of the unit.

It is also acceptable for the appropriately qualified assessor to work with an industry expert to conduct assessment together and for the industry expert to be involved in the assessment judgement. The industry expert must have current industry skills directly relevant to the training and assessment being provided. This means the industry subject matter expert must demonstrate skills and knowledge from the minimum years of current work experience after competency has been obtained as specified below, including time spent in roles related to the unit being assessed:

| #### Industry sector | #### AQF indicator level*** | #### Required assessor or industry subject matter expert experience |
| --- | --- | --- |
| Drilling, Metalliferous Mining, Coal Mining, Extractive (Quarrying) and Civil Infrastructure | 1 | 1 year |
|  | 2 | 2 years |
| Drilling, Coal Mining, Extractive (Quarrying), Metalliferous Mining and Civil Infrastructure | 3-6 | 3 years |
| Other sectors | Where this unit is being assessed outside of the resources and infrastructure sectors assessor and/or industry subject matter expert experience should be in-line with industry standards for the sector in which it is being assessed and where no industry standard is specified should comply with any relevant regulation. |  |

*Guidance on simulated environments has been stipulated in the RII implementation guide located on VETNet.

**Assessors can demonstrate current work experience through employment within industry in a role relevant to the outcomes of the unit; or, for external assessors this can be demonstrated through exposure to industry by conducting a minimum number of site assessments as determined by the relevant industry sector, across various locations.

***While a unit of competency does not have an AQF level, where a unit is being delivered outside of a qualification the first numeric character in the unit code should be considered as the AQF indicator level for assessment purposes.

# Links

Companion Volume Implementation Guide is found on VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=88a61002-9a21-4386-aaf8-69c76e675272