# Assessment Requirements for RIIWHS207 Control traffic on high volume roads

Release: 1

Assessment Requirements for RIIWHS207 Control traffic on high volume roads

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version first released with RII Resources and Infrastructure Industry Training Package Version 6.0. |

# Performance Evidence

The candidate must demonstrate the ability to complete the tasks outlined in the elements, performance criteria and foundation skills of this unit, including evidence of the ability to:

control traffic on high volume roads and signalised intersections on at least two occasions, including:

completing at least one of the following:

controlling traffic on roads with posted speed limits greater than or equal to 60km/h including:

assessing traffic flow and the interaction between all road users, including the impact of queues in traffic, traffic speeds, traffic volume, traffic delays and merging behaviour of drivers

identifying and reporting issues with traffic flow and interaction between road users to the traffic management implementer

controlling traffic at signalised intersections where traffic signals have been turned off

interpreting traffic control elements within the traffic guidance schemes for works on high volume roads and signalised intersections

completing at least one of the following:

working with a pilot vehicle, including:

managing the speed and travel path of a driver through the work site

identifying and reporting when part of the length of the work site is out of view of the supervisor, work gang and traffic controller

directing traffic on high volume, multilane roads with one traffic controller per lane

directing pedestrian traffic, including at least one of the following road users:

pedestrians with mobility issues

pedestrians with prams

cyclists

using approved communication devices to transmit message and report traffic offenders, including the use of at least one of the following devices:

hand held radio

mobile phone.

During the above, the candidate must:

locate and apply required documentation, policies and procedures

participate in risk assessment and management processes

identify risks, hazards and required controls

complete at least one incident report from the below list and seek feedback on the outcome:

client

public

traffic control.

# Knowledge Evidence

The candidate must be able to demonstrate knowledge to complete the tasks outlined in the elements, performance criteria and foundation skills of this unit, including knowledge of:

legislation required to control traffic on high volume roads and signalised intersections

policies, procedures and documentation required to control traffic on high volume roads and signalised intersections, including those relating to:

Austroads Guide to Temporary Traffic Management

risk assessment processes and hierarchy of control

work instructions and procedures for planning work

work health and safety

environmental protection requirements

site incident and emergency procedures

site and equipment safety requirements

operational and maintenance procedures for equipment

site isolation and traffic control responsibilities and authorities

safe work method statements (SWMS)

communication techniques and equipment

site requirements

manufacturer specifications for required plant and equipment

traffic management plans

traffic controller resourcing

workplace recording and reporting

traffic control requirements and procedures for directing traffic:

on roads with speed limits greater than or equal to 60km/h and greater than 3,000 vehicles per day (Average Annual Daily Traffic (AADT))

on roads with any speed limit and greater than 10,000 vehicles per day (AADT)

at signalised intersections, including where traffic signals have been turned off

on high volume, multilane roads

for vulnerable road users, including:

pedestrians with mobility issues

pedestrians with prams

cyclists

traffic guidance scheme and traffic management plan compliance relating to traffic controllers

communication device operations, including:

hand held radios

mobile phones

traffic control equipment types, characteristics, technical capabilities and limitations

the effects of travel speed and vehicle mass on stopping distances.

# Assessment Conditions

Mandatory conditions for assessment of this unit are stipulated below. The assessment must:

include access to:

stop-slow bat

personal protective equipment

handheld radio or mobile phone when permitted

be conducted in a safe environment; and

be assessed in the context of this sector’s work environment on a real live road setting under supervision, including evidence in at least one of the forms that this has occurred on at least three different temporary traffic management set ups:

dash cam footage

photos

verbal interviews

logbooks

completed job packs

be assessed in compliance with relevant legislation/regulation and using policies, procedures, processes and operational manuals directly related to the industry sector for which it is being assessed; and

confirm consistent performance can be applied in a range of relevant workplace circumstances

Where personal safety or environmental damage are limiting factors, assessment may occur in a simulated work environment* provided it is realistic and sufficiently rigorous to cover all aspects of this sector’s workplace performance, including environment, task skills, task management skills, contingency management skills and job role environment skills.

Assessor requirements

Assessors must be able to clearly demonstrate current and relevant industry knowledge and experience to satisfy the mandatory competency standards as set out in the Standards for Registered Training Organisations (RTOs) 2015/Australian Quality Training Framework mandatory requirements for assessors current at the time of assessment and any relevant licensing and certification requirements. This includes:

vocational competencies at least to the level being delivered and assessed

current industry skills directly relevant to the training and assessment being provided

current knowledge and skills in vocational training and learning that informs their training and assessment

formal relevant qualifications in training and assessment

having knowledge of and/or experience using the latest techniques and processes

possessing a high level of RII training product knowledge

having an understanding and knowledge of legislations and regulations relevant to the industry and to employment and workplaces

demonstrating the performance evidence, and knowledge evidence as outlined in this unit of competency, and

the minimum years of current** work experience after competency has been obtained as specified below in an industry sector relevant to the outcomes of the unit.

It is also acceptable for the appropriately qualified assessor to work with an industry expert to conduct assessment together and for the industry expert to be involved in the assessment judgement. The industry expert must hold the relevant vocational competencies and have current industry skills directly relevant to the training and assessment being provided and must work alongside a trainer and/or assessor to conduct the assessment. This means the industry subject matter expert should hold the unit being assessed (or an equivalent unit), and must also demonstrate skills and knowledge from the minimum years of current work experience after competency has been obtained as specified below, including time spent in roles related to the unit being assessed:

| #### Industry sector | #### AQF indicator level*** | #### Required assessor or industry subject matter expert experience |
| --- | --- | --- |
| Drilling, Metalliferous Mining, Coal Mining, Extractive (Quarrying) and Civil Construction | 1 | 1 year |
|  | 2 | 2 years |
| Drilling, Coal Mining and Extractive (Quarrying), Metalliferous Mining and Civil Construction | 3-6 | 3 years |
| Other sectors | Where this unit is being assessed outside of the resources and infrastructure sectors assessor and/or industry subject matter expert experience should be in-line with industry standards for the sector in which it is being assessed and where no industry standard is specified should comply with any relevant regulation. |  |

*Guidance on simulated environments has been stipulated in the RII Companion Volume Implementation Guide located on VETNet.

**Assessors can demonstrate current work experience through employment within industry in a role relevant to the outcomes of the unit; or, for external assessors this can be demonstrated through exposure to industry by conducting a minimum number of site assessments as determined by the relevant industry sector, across various locations.

*** While a unit of competency does not have an AQF level, where a unit is being delivered outside of a qualification the first numeric character in the unit code should be considered as the AQF indicator level for assessment purposes.

# Links

Companion Volume Implementation Guide is found on VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=88a61002-9a21-4386-aaf8-69c76e675272