# SFIDIV306 Perform underwater work in the aquaculture sector

Release: 1

SFIDIV306 Perform underwater work in the aquaculture sector

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with SFI Seafood Industry Training Package Version 1.0. |

# Application

This unit of competency describes the skills and knowledge required to perform underwater dive work in aquaculture environments, including entering and leaving the water safely, undertaking maintenance on culture, holding or other structures and vessels, using basic hand tools and auxiliary devices, restraining and lifting loads, and conducting underwater searches.

This unit applies to individuals who work as occupational divers in the aquaculture sector of the seafood industry.

Occupational licensing, legislative and certification requirements apply to this unit but vary according to state/territory requirements. Occupational diving is regulated independently by each state and territory work health and safety authority. Users are advised to check with the relevant authority to confirm current requirements.

# Pre-requisite Unit

Nil

# Unit Sector

Diving (DIV)

# Elements and Performance Criteria

| Elements | Performance Criteria |
| --- | --- |
| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. |
| 1. Enter and leave the water | 1.1 Consult crew and consider prevailing weather and water conditions when assessing the safety of diving operations, and make a judgement on whether to continue the dive 1.2 Ensure that procedures during the dive, and when entering or leaving the water from the dive platform meet guidelines and are according to normal dive practices 1.3 Use signals to the dive platform before descent and following ascent according to normal dive practices |
| 2. Maintain structures, vessels and equipment | 2.1 Inspect and assess the condition of culture or holding structures and report any required action to the supervisor 2.2 Inspect and assess the condition of associated vessels, structures, plant and equipment and report any required action to the supervisor 2.3 Undertake appropriate maintenance according to industry or licensing body instructions and health and safety procedures |
| 3. Assist in lifting and handling | 3.1 Recognise the limitations and suitability of winches with hydraulic and air motors for use on the surface and underwater when undertaking winching operations 3.2 Use correct signs and signals during winch operations to communicate effectively with crew 3.3 Select and rig lifting equipment and ropes of the correct type and safe working load using the correct mechanical advantage for the load being lifted 3.4 Use and secure lifting bags to raise loads 3.5 Restrain the load so that ascent is controlled 3.6 Inspect and maintain slings, ropes and lifting bags and carry out pre- and post-dive checks on lifting devices |
| 4. Use basic hand tools and auxiliary devices underwater | 4.1 Use a range of hand tools to complete simple tasks underwater 4.2 Use auxiliary devices in underwater operations safely 4.3 Inspect tools and devices for defects, maintain and store after use or set aside for repair |
| 5. Conduct underwater searches | 5.1 Use a range of underwater search and survey techniques to locate targeted items 5.2 Maintain signals while conducting underwater search |

# Foundation Skills

This section describes those language, literacy, numeracy and employment skills that are essential for performance in this unit of competency but are not explicit in the performance criteria.

| Skill | Description |
| --- | --- |
| Numeracy | Interprets and follows dive tables Reads diving instruments Performs calculations and measurements for diving operations and underwater search and survey techniques |
| Reading | Interprets required information from diving-related instructions, procedures, regulatory requirements and guidelines |
| Navigate the world of work | Takes responsibility for decisions about when and how to complete tasks, and coordinates with others |
| Interact with others | Uses appropriate terminology and signals specific to diving, lifting and handling operations |
| Get the work done | Uses diver communication systems |

# Unit Mapping Information

| Code and title current version | Code and title previous version | Comments | Equivalence status |
| --- | --- | --- | --- |
| SFIDIV306 Perform underwater work in the aquaculture sector | SFIDIVE315A Perform underwater work in the aquaculture sector | Updated to meet Standards for Training Packages Prerequisite removed Minor amendments to elements and performance criteria for clarity | Equivalent unit |

# Links

Companion Volumes, including Implementation Guides, are available at VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=e31d8c6b-1608-4d77-9f71-9ee749456273

Assessment Requirements for SFIDIV306 Perform underwater work in the aquaculture sector

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with SFI Seafood Industry Training Package Version 1.0. |

# Performance Evidence

An individual demonstrating competency must satisfy all of the elements and performance criteria in this unit.

There must be evidence that the individual has performed underwater dive work in an aquaculture environment on at least one occasion, including:

making an assessment of the conditions before undertaking a dive

entering and leaving the water according to standard dive procedures

assisting with winch operations

undertaking the required maintenance tasks on the aquaculture site

operating and maintaining the appropriate tools and auxiliary devices to complete the required underwater tasks

using appropriate dive signals when communicating with crew

using appropriate search techniques to trace items.

# Knowledge Evidence

An individual must be able to demonstrate the knowledge required to perform the tasks outlined in the elements and performance criteria of this unit. This includes knowledge of:

local weather and sea conditions that impact on diving operations

diving signals used in underwater work, including when entering and leaving water and carrying out lifting and handling activities and underwater searches

aquaculture apparatus and associated equipment, including:

auxiliary devices

lifting equipment, including its suitability and limitations

basic hand tools

diver communication systems

methods for entering and leaving the water from a range of dive platforms

maintenance procedures that require underwater work on an aquaculture site

safe working loads and mechanical advantages for winching operations

underwater search and survey techniques used for underwater work on an aquaculture site.

# Assessment Conditions

Assessment of skills must take place under the following conditions:

physical conditions:

skills must be demonstrated in a diving environment in an aquaculture setting or an environment that accurately represents workplace conditions

resources, equipment and materials:

diving equipment

aquaculture apparatus and associated equipment

basic hand tools used in the aquaculture sector

underwater auxiliary devices

specifications:

workplace procedures for performing underwater work that include advice on health and safety requirements

manufacturer instructions for basic hand tools.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volumes, including Implementation Guides, are available at VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=e31d8c6b-1608-4d77-9f71-9ee749456273