# SHBBHRS007 Provide laser hair reduction treatments

Release: 1

SHBBHRS007 Provide laser hair reduction treatments

# Modification History

Not applicable.

# Application

This unit describes the performance outcomes, skills and knowledge required to safely provide laser hair reduction treatments to reduce unwanted hair on the face and body. It requires the ability to follow an agreed treatment program, monitor and modify treatments in response the client’s skin and hair reactions, and assess treatment results.

This unit applies to laser practitioners working in a diverse range of beauty industry sectors and business contexts who provide laser hair reduction treatments. These individuals operate independently, under limited supervision of others and within established organisational policies and procedures.

The unit reflects the safety guidelines, for a broad range of laser dermal applications, expressed in the following Australian and New Zealand Standards, current at the time of publication:

AS/NZS 4173:2018 Safe use of lasers and intense light sources in health care

AS/NZS IEC 60825.1:2014 Safety of laser products – Part 1: Equipment classifications and requirements

AS/NZS 1336:2014 Eye and face protection - Guidelines.

The use of laser is subject to legislation, regulation and licensing in some Australian States and Territories.

# Pre-requisite Unit

SHBBHRS005 Identify and control safety risks for intense pulsed light and laser hair reduction treatments

# Competency Field

Hair Reduction Services

# Unit Sector

Beauty

# Elements and Performance Criteria

| ELEMENTS | PERFORMANCE CRITERIA |
| --- | --- |
| Elements describe the essential outcomes | Performance criteria describe the performance needed to demonstrate achievement of the element. |
| 1. Prepare treatment environment and equipment. | 1.1. Access and review agreed client treatment program to determine requirements for individual treatment. 1.2. Check that treatment environment is correctly organised and free from safety risks and hazards. 1.3. Confirm completion of risk assessment and equipment checks and take any required corrective action. 1.4. Set up equipment and check to ensure correct functioning as per manufacturer instructions, and safe practice protocols. 1.5. Cleanse and prepare treatment area and sanitise equipment prior to use on client. 1.6. Complete workplace and pre-treatment documentation required for treatment environment and equipment preparation. |
| 2. Prepare self and client. | 2.1. Identify need for and receive signed medical approval to proceed with treatment from client. 2.2. Confirm treatment program with client. 2.3. Determine, through discussion and questioning, changes to client medical history and contraindications arising since time of treatment program development. 2.4. Adjust treatment program to reflect identified changes and obtain signed client consent for adjusted treatment program. 2.5. Explain treatment process and likely sensations to client to ensure client comfort. 2.6. Identify jewellery worn by self and client and remove, or where not removable, tape according to workplace procedures. 2.7. Fit and adjust personal protective equipment, including eye protection, to self, client and others who may be present during treatment. |
| 3. Provide laser treatment. | 3.1. Follow workplace policies and procedures, laser safety plan and legislative requirements to apply laser according to treatment program. 3.2. Identify requirement for coupling gel and apply ensuring minimum wastage. 3.3. Perform a test shot on treatment area, observe immediate skin reaction and level of client discomfort, and adjust equipment settings in preparation for treatment. 3.4. Commence treatment on areas of least pain sensitivity. 3.5. Work sequentially on treatment area avoiding repeat treatment on area immediately after first shot. 3.6. Monitor client reaction to treatment and pain tolerance continuously and adjust equipment settings to minimise client discomfort. 3.7. Identify potential and actual hazards and assess risks throughout treatment and take action to minimise risk to self and client. 3.8. Identify and respond immediately to adverse reactions by stopping treatment and explaining reason for stoppage to client. 3.9. Identify need for, and take and store photographs of hair reduction treatment area according to workplace policies and procedures. 3.10.Turn off equipment and remove coupling gel at completion of treatment. |
| 4. Provide post-treatment care and advice. | 4.1. Select and apply post-treatment skin cooling products according to treatment plan and client reaction to treatment. 4.2. Advise client on post-treatment and self-care and ask questions to confirm client understanding. 4.3. Advise client on client care and precautionary requirements for period leading up to next treatment 4.4. Record treatment and adjust treatment plan to reflect treatment and outcomes. 4.5. Complete post treatment documentation according to workplace procedures. 4.6. Initiate follow-up to check for post-treatment complications and adverse effects according to workplace procedures. |

# Foundation Skills

| Foundation skills essential to performance in this unit, but not explicit in the performance criteria are listed here, along with a brief context statement. |  |
| --- | --- |
| SKILLS | DESCRIPTION |
| Reading skills to: | interpret both familiar and unfamiliar, and sometimes complex documents: client treatment programs client records workplace policies and procedures non-ionising radiation safety protection plan materials describing industry standards and regulatory requirements of laser hair reduction manufacturer instructions for the safe use, maintenance and storage of laser equipment. |
| Writing skills to: | clearly and legibly make amendments to treatment plan. |
| Oral communication skills to: | consult with, inform and reassure clients throughout treatment process respond to client questions and provide information to client throughout treatment process. |
| Numeracy skills to: | calculate and modify equipment settings. |

# Unit Mapping Information

No equivalent unit

# Links

Companion Volume Implementation Guide - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=255d312b-db07-48f2-b6d6-1b0b06c42898

Assessment Requirements for SHBBHRS007 Provide laser hair reduction treatments

# Modification History

Not applicable.

# Performance Evidence

Evidence of the ability to complete tasks outlined in elements and performance criteria of this unit in the context of the job role, and:

safely provide laser treatments to at least three different clients, each with different Fitzpatrick skin types, that cumulatively cover the following body areas (18 treatments in total):

back and neck

Brazilian

chest

facial area

legs or arms

underarm

complete at least one of the above 18 treatments on a Fitzpatrick skin type 5 or 6

for each of the above laser treatments:

adjust and modify treatment programs in response to client reaction to treatment

correctly set-up, operate and pack down equipment

provide pre and post-treatment advice to clients

complete and store client records and workplace documentation

consult, advise and reassure clients at all stages in treatment

during treatments, consistently follow workplace policies and procedures, non-ionising radiation safety protection plan and relevant state or territory laws, codes and guidelines related to the safe use of laser for hair reduction.

# Knowledge Evidence

Demonstrated knowledge required to complete the tasks outlined in elements and performance criteria of this unit:

current local, state or territory health legislation, regulation and industry standards and guidelines relevant to laser hair reduction treatments:

non-ionising radiation protection

skin penetration

work health and safety

practitioner’s legal and insurance liabilities and responsibilities for laser treatment:

role and importance of indemnity insurance

potential risks, liabilities and consequences of non-coverage

responsibilities of practitioner and client related to safe and responsible laser hair reduction treatments

workplace policies and procedures related to laser hair reduction treatments, including those for treatment of minors

workplace non-ionising radiation safety protection plan

types of laser equipment commonly used within industry and accepted by industry standards, their recommended use and application

factors impacting client suitability for laser treatments and their associated risks, benefits and required action

typical features and skin tanning response of each of the six Fitzpatrick skin types and their relationship to laser hair reduction treatments

hair growth patterns and effect on laser hair reduction on hair growth

anatomy and physiology of the skin and skin structures as related to laser hair reduction treatments:

normal process of skin ageing and structural change

normal skin response to irritation and trauma

scars, including hypertrophic and keloid origin and evolution, and abnormal scar tissue

causes of skin damage

effects of laser treatments on physical structure of the skin

wound healing in different skin types and locations

tissue interaction with laser hair reduction treatments

chromophores

melanin absorption

contraindications and their relationship to laser hair reduction treatments:

pre-treatment sun exposure

artificially tanned skin

pregnancy

herbal remedies

active herpes simplex

active impetigo

active eczema

active acne

photo-sensitive medication

hypertrophic and keloid scars

artificial pigment in the treatment area

psoriasis in the treatment area

certain circulatory conditions

inappropriate hair type and colour

for the above listed contraindications:

appearance and identification

required action and treatment precautions

instances where treatment should not occur

instances where recommendation to seek advice of other professionals should be provided

medical contraindications, their relationship to laser hair reduction treatments, and required action and treatment precautions:

conditions already being treated by a general practitioner, dermatologist or skin therapist

medical oedema

recent surgery

diabetes

asthma

epilepsy

effects of laser hair reduction treatments and appropriate responses and control measures:

paradoxical increased hair growth

temporary heat

temporary localised pain

erythema

causes of, appearance and management of specific treatment reactions and adverse effects:

hypopigmentation

hyperpigmentation

blistering

scarring and textural changes

perifollicular oedema

perifollicular erythema

infections of the hair follicle

burns to skin

factors affecting treatments, their effect and appropriate responses:

test shot skin reaction

skin healing

skin reaction

follicle distribution

areas treated

extent of area to be treated

hypertrophic and keloid scarring

outcomes of previous treatments

pain tolerance

wound healing

post-treatment care procedures:

low-frequency laser

healing products

cooling products:

cold compress

cryogen sprays

cold gels

client post-treatment care requirements:

post-treatment care and advice for the treatment area

post treatment precautions related to:

tweezing

waxing

bleaching

sunbathing

solarium treatments

use of depilatory creams

electrolysis

use of self-tanning products

treatment evaluation strategies:

discussing and reviewing response to treatment

taking photographs of treatment area before and after each session, as required

reviewing achievement of treatment goals

monitoring time frame for achievement of treatment goals

key features, uses, benefits and limitations of the following laser equipment types:

long pulsed alexandrite

long pulsed Nd:Yag

diode

treatment settings and their application in laser hair reduction treatments:

wavelengths

pulse duration

energy settings (fluence)

treatment duration.

# Assessment Conditions

Skills must be demonstrated in a simulated laser hair reduction clinical workplace environment set up for the purposes of assessment.

Assessment must be directly observed, in person, for the total duration of each laser treatment, by assessors who meet the assessor requirements outlined below.

Assessment must ensure access to:

documentation relevant to providing laser hair reduction treatments:

workplace policies and procedures for laser use

non-ionising radiation safety protection plan

current safety and environmental standards, guidelines regarding laser safety

client content and consultation forms

treatment log book

post-care forms

laser equipment which:

when energised, can emit an amount of non-ionising radiation higher than accessible limit for a Class 3B laser for the relevant period stated in, and measured in accordance with, the laser standard AS2211 (a Class 4 laser)

which can treat Fitzpatrick skin types one to six

is registered for purpose on the Australian Register of Therapeutic Goods

complies with Safety of laser products equipment classification and requirements AS/NZS IEC 60825.1:2014

laser clinical treatment environment that includes:

as per relevant state or territory legislation either, or both, of the following:

a radiation warning sign

an illuminated light above door

non-flammable screens fitted inside windows to protect a person outside the window from non-ionising radiation levels greater than maximum permissible exposure from radiation

ventilation designed to ensure that airborne hazards are not passed downstream in air handling and exhaust system

fire extinguishing equipment

sufficient, clinical lighting

eye protection equipment for practitioner and clients compliant with AS/NZS 1336:2014

disposable masks

disposable gloves

skin grid pencils

handwashing basin

cooling post-treatment products

antibacterial cleaning products

client skin wipes

real people with different Fitzpatrick skin types seeking hair reduction treatments on different areas of the face and body

activities that reflect industry practice and allow the individual to work with commercial timing and productivity to complete client treatments within designated timeframes.

Assessors must satisfy the Standards for Registered Training Organisations requirements for assessors, and:

have at least three years’ experience in operating laser equipment.

# Links

Companion Volume Implementation Guide - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=255d312b-db07-48f2-b6d6-1b0b06c42898