# SISCAQU018 Operate self-contained breathing apparatus in aquatic facility emergencies

Release: 1

SISCAQU018 Operate self-contained breathing apparatus in aquatic facility emergencies

# Modification History

Not applicable.

# Application

This unit describes the performance outcomes, skills and knowledge required to operate a self-contained breathing apparatus (SCBA) under emergency conditions in an aquatic facility. It covers skills for completing periodic SCBA checks and minor repairs to ensure the apparatus is readily available when required. It requires the ability to respond to chlorine gas emergencies.

This unit applies to public aquatic facilities where chlorine gas is used. It applies to senior technical and operational staff who work independently or with limited guidance from others, taking responsibility for their own work functions and outputs.

The skills in this unit must be applied in accordance with Commonwealth and State or Territory legislation, Australian standards and industry codes of practice.

The Western Australian Code of Practice for the Design, Construction, Operation, Management and Maintenance of Aquatic Facilities requires technical operators working in Group 1 aquatic facilities to be certified in this unit of competency.

No other occupational licensing, certification or specific legislative requirements apply to this unit at the time of publication.

# Pre-requisite Unit

Nil

# Competency Field

Aquatics

# Unit Sector

Community Recreation

# Elements and Performance Criteria

| ELEMENTS | PERFORMANCE CRITERIA |
| --- | --- |
| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. |
| 1. Maintain self-contained breathing apparatus. | 1.1. Interpret manufacturer guidelines to determine repair and maintenance requirements for self-contained breathing apparatus. 1.2. Complete periodic safety and serviceability checks on apparatus according to manufacturer instructions. 1.3. Complete leak detection and withdrawal warning alarm tests according to manufacturer instructions. 1.4. Complete minor repairs to faulty equipment and replace defective parts according to manufacturer instructions. 1.5. Complete repair and maintenance records according to organisational procedures. |
| 2. Use self-contained breathing apparatus. | 2.1. Identify emergencies requiring use of self-contained breathing apparatus. 2.2. Assemble component parts of apparatus and promptly complete operational checks. 2.3. Fit and adjust apparatus to ensure comfort and safety. 2.4. Operate apparatus during emergency incidents according to manufacturer instructions and organisational work health and safety (WHS) procedures. 2.5. Monitor air supply by checking gauges regularly throughout emergency activities. |
| 3. Respond to chlorine gas emergency. | 3.1. Communicate initial assessment of emergency to appropriate personnel and determine required response. 3.2. Isolate chlorine gas equipment and notify emergency personnel according to organisational emergency response procedures. |
| 4. Maintain and store self-contained breathing apparatus after use. | 4.1. Dismantle apparatus and clean all components according to manufacturer instructions to maintain serviceable condition and prolonged lifespan. 4.2. Store apparatus to allow efficient access for next use. 4.3. Tag, isolate and report faulty apparatus according to organisational procedures. |
| 5. Debrief and complete reports. | 5.1. Participate in incident management debrief and identify future response improvements. 5.2. Complete required incident reports according to organisational procedures. |

# Foundation Skills

| Foundation skills essential to performance in this unit, but not explicit in the performance criteria are listed here, along with a brief context statement. |  |
| --- | --- |
| SKILLS | DESCRIPTION |
| Reading skills to: | interpret potentially unfamiliar and detailed manufacturer instructions for the use, repair and maintenance of self-contained breathing apparatus. |
| Writing skills to: | use fundamental sentence structure to complete basic repair and maintenance forms and reports that require factual information record detailed and accurate factual information about emergency responses. |
| Numeracy skills to: | interpret and record numerical information from gauges and meters involving volume and pressure. |
| Self-management skills to: | critically analyse all circumstances and implications to provide a prompt and considered emergency response. |

# Unit Mapping Information

Supersedes and is not equivalent to SISCAQU014 Operate self-contained breathing apparatus in an aquatic facility.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=1ca50016-24d2-4161-a044-d3faa200268b

Assessment Requirements for SISCAQU018 Operate self-contained breathing apparatus in aquatic facility emergencies

# Modification History

Not applicable.

# Performance Evidence

Evidence of the ability to complete tasks outlined in elements and performance criteria of this unit in the context of the job role, and:

follow manufacturer instructions to complete:

one periodic safety and serviceability check on self-contained breathing apparatus

one minor non-specialist repair on self-contained breathing apparatus

for each check and repair task above, complete accurate records

participate in two simulated emergency incidents involving the release of chlorine gas, following safety and emergency response procedures

for each incident above:

identify, assess and respond to the gas release circumstance

utilise a self-contained breathing apparatus

dismantle and clean apparatus

participate in a debrief and complete an incident report.

# Knowledge Evidence

Demonstrated knowledge required to complete the tasks outlined in elements and performance criteria of this unit:

organisational work health and safety (WHS) and emergency response procedures for chlorine gas release

for chlorine gas:

atmospheric conditions and how they can affect chlorine gas cylinders

causes of chlorine gas emergencies in aquatic facilities

properties of gas release, including visual appearance and odour

dangers to health, and symptoms of exposure to chlorine gas

emergency response actions, and communication protocols when gas is released

how to isolate chlorine gas equipment

for self-contained breathing apparatus:

the main components:

backpack and harness

air cylinder

regulator

face piece

operational features and functions of each component, and how to assemble, use and disassemble the apparatus

safety and serviceability checks completed on each component

schedules for periodic inspection and maintenance

common types of minor, non-specialist repairs, and tools and methods used to complete these

methods used to clean apparatus and types of cleaning agents used

formats and inclusions of:

manufacturer instructions for repair and maintenance of self-contained breathing apparatus

repair and maintenance records and fault tags

incident reports.

# Assessment Conditions

Skills must be demonstrated in a public aquatic facility with an operating pool and chlorine gas system. Facilities can include those operated by commercial, not-for-profit, community and government organisations.

The following resources must be available to replicate industry conditions of operation:

first aid equipment

communication equipment for emergency response.

Assessment of participation in emergency incidents can only be completed through simulated activities. Simulations must incorporate time critical requirements.

Assessment must ensure use of:

chlorine gas system equipment

self-contained breathing apparatus, inclusive of air cylinder and regulator

manufacturer instructions for repair and maintenance of self-contained breathing apparatus

template:

repair and maintenance records and fault tags

incident reports

organisational work health and safety and emergency response procedures for chlorine gas release.

Assessors must:

satisfy the Standards for Registered Training Organisations requirements for assessors, and

have workplace experience where they have applied the skills and knowledge covered in this unit of competency over a period of two years in industry.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=1ca50016-24d2-4161-a044-d3faa200268b