# Assessment Requirements for SISOCYT001 Set up, maintain and repair bicycles

Release: 1

Assessment Requirements for SISOCYT001 Set up, maintain and repair bicycles

# Modification History

Not applicable.

# Performance Evidence

Evidence of the ability to complete tasks outlined in elements and performance criteria of this unit in the context of the job role, and:

according to activity plans for three cycling activities operated in three different conditions:

select bicycles suited to conditions

assemble an in-field repair kit, including tools and spare parts, suited to conditions

adjust a total of three bicycles to suit the size of  different participants and collectively make the following adjustments:

saddle position and height

handlebar height and width

distance from saddle to handlebars

position of gears and brakes

complete pre-departure safety and serviceability checks on a total of two bicycles to include those for:

legally required accessories e.g. bells, reflectors, lights

handlebars and plugs

wheels, hubs and spokes

suspension

headsets

tyre condition and tyre pressure

chains and gears

brakes

bolts or quick release skewers

clean and service the following bicycle systems, collectively across two different bikes:

frame

drivetrain

steering

brakes

suspension

wheels and hubs

tyres, including two puncture repairs

accessories

across the services, collectively :

remove, adjust and refit a total of two components

identify a total of two worn or damaged components and replace

lubricate components

complete maintenance records for each bike serviced.

# Knowledge Evidence

Demonstrated knowledge required to complete the tasks outlined in elements and performance criteria of this unit:

organisational procedures for completing repairs, maintenance and safety and serviceability checks

different types of bikes used for recreational cycling activities, their differing construction features and how features are suited to different surfaces and terrain:

road bikes

mountain bikes

hybrid bikes

fixed gear

how these factors that affect the selection of bikes:

participant characteristics:

age, height, reach and weight

current cycling experience and skill level

fitness level and physical capabilities

injuries and medical conditions

emotional, behavioural and intellectual ability or disability

cycling surface:

sealed roads, formed bike paths or trails

unsealed surfaces - gravel roads, fire trails, single track, open or rocky ground

gradient of terrain

manufacturers’ specifications for bike use

bike sizing guidelines, and how to take measurements to suit rider size, for:

road bikes

mountain bikes

hybrid bikes

types of adjustments made to bikes to suit participant characteristics, surface and terrain and how to make these:

saddle position and height

handlebar height and width

distance from saddle to handlebars

position of gears and brakes

suspension

tyre pressure including range of suitable pressures

types of pre-departure safety and serviceability checks, how to complete these and what tools would be used, to include those for:

legally required accessories e.g. bells, reflectors, lights

handlebars and plugs

wheels, hubs and spokes

suspension

headsets

tyre condition and tyre pressure

chains and gears

brakes

components and key operational features of these bicycle systems:

frame

drivetrain

steering

brakes

suspension

wheels and hubs

tyres

accessories

for each of the above systems:

how to remove, adjust and refit components

how to identify worn or damaged components for replacement

cleaning requirements and techniques

lubrication requirements, as relevant

types of repairs referred to specialist technicians

for tyres, how to remove tubes, repair punctures and re-inflate

different types of cleaning products and how these are used on different bike parts:

cleaning solutions and sprays for removing mud and dirt

degreasers for cleaning chains and other moving parts

brake cleaners

suspension solutions and sprays

rust prevention polishes and sprays

types and use of tools and equipment used to maintain and repair bicycles including those typically used in the field

spare parts typically carried in the field

safe manual handling techniques for bending, lifting and carrying heavy equipment during maintenance tasks

types and correct use of personal protection equipment for maintaining bicycles

a range of formats for and inclusions of maintenance documents:

fault tags and reports

repair and maintenance reports.

# Assessment Conditions

Skills can be assessed in the field or in an operational depot or a combination, provided that the situation is appropriate to the repair or maintenance task.

A depot can be:

an industry workplace or

a simulated industry environment set up for the purposes of assessment.

Assessment must ensure use of:

personal protective equipment for repair and maintenance activities

bicycles of different types which can include:

road bikes

mountain bikes

hybrid bikes

spare parts for all bicycle system components

tools used to maintain and repair bicycles:

Allen keys

cassette remover

chain breaker

chain whip

crank puller and crank wrench

pliers, cable cutting pliers

pressure gauge

pumps and connectors

puncture repair kit

screwdrivers

spanners: ring, open ended, shifting, two cone, pedal, headset, sockets and ratchets

spoke key

tyre levers

lubricants

cloths, stiff and soft brushes used for cleaning

cleaning products for bikes to include:

agents for removing mud and dirt

degreasers

brake cleaners

suspension cleaners

rust prevention agents

participant profiles

activity plans

manufacturer’s cleaning, repair and maintenance instructions

template maintenance documents:

fault tags and reports

repair and maintenance reports

organisational:

procedures for completing repairs, maintenance and safety and serviceability checks

maintenance schedules.

Assessors must satisfy the Standards for Registered Training Organisations requirements for assessors, and:

have a collective period of at least three years’ experience with an organisation providing recreational cycling programs, or as a bicycle technician, where they have applied the skills and knowledge covered in this unit of competency; the three years’ experience can incorporate full and or part time experience.

# Links

Companion Volume Implementation Guides - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=1ca50016-24d2-4161-a044-d3faa200268b