# Assessment Requirements for SISODRV001 Drive AWD/4WD vehicles on unsealed roads

Release: 1

Assessment Requirements for SISODRV001 Drive AWD/4WD vehicles on unsealed roads

# Modification History

Not applicable.

# Performance Evidence

Evidence of the ability to complete tasks outlined in elements and performance criteria of this unit in the context of the job role, and:

complete three trips in all-wheel or four feel drive vehicles on three different graded formed dirt roads

during each trip consistently follow safety procedures and safely negotiate hazards and road conditions

complete a minor water crossing on three occasions

change a total of three wheels.

# Knowledge Evidence

Demonstrated knowledge required to complete the tasks outlined in elements and performance criteria of this unit:

organisational safety and emergency response procedures for driving activities

difference between all-wheel and four feel drive vehicles, at a basic level of understanding

manufacturers’ specifications for wheel replacement

features, function and operation of AWD and 4WD vehicle equipment and controls used for dirt road driving and when negotiating minor water crossings:

instruments

traction devices

wheel hubs, self-locking and manual locking

how tyre pressure can affect comfort and safety of dirt road driving and how to determine appropriate tyre pressure for a range of conditions and loads

factors that affect a vehicle’s traction on dirt roads

techniques used to maintain traction including those for:

braking

acceleration

steering

meaning of centre of gravity for a vehicle and factors that affect this when driving

driving techniques used when negotiating minor water crossings

features, functions and operation of equipment for:

checking oil and water

changing tyre pressure

securing loads

changing wheels

typical hazards associated with driving on unsealed roads, and techniques used to safely negotiate these:

blind corners

one-lane bridges

slippery road surface

bumps

depressions

potholes

drainage grates

rail crossings

moving and parked vehicles

other users:

cyclists

pedestrians

runners

horse riders.

# Assessment Conditions

Skills must be demonstrated on graded formed dirt roads and in environments which feature minor water crossings.

The following resources must be available to replicate industry conditions of operation:

first aid equipment

communication equipment for emergency response.

Assessment must ensure use of:

all-wheel drive or four wheel drive vehicles

wheel changing equipment including jacks

spare fully functioning wheels and tyres

organisational safety and emergency response procedures for driving activities.

Assessors must satisfy the Standards for Registered Training Organisations requirements for assessors, and:

hold a driver’s licence which complies with local state or territory law.

# Links

Companion Volume Implementation Guides - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=1ca50016-24d2-4161-a044-d3faa200268b