# SISOPWC001 Ride personal watercraft in smooth water conditions

Release: 1

SISOPWC001 Ride personal watercraft in smooth water conditions

# Modification History

Not applicable.

# Application

This unit describes the performance outcomes, skills and knowledge required to ride personal water craft, usually as part of a group, in smooth water conditions and according to planned courses.

A personal watercraft (PWC) is a vessel with an engine used for propulsion, a fully enclosed hull and is designed not to retain water when capsized. The operator sits, stands or kneels on the vessel and uses handle bars to steer the craft. Craft are often referred to by their trademarked brand names which include Jet Ski®, WaveRunner® and Sea-Doo®.

This unit provides skills to ride craft in smooth water conditions within bodies of water which might include lakes, rivers, bays, harbours and sheltered coastline waters within breakwaters or headlands. Smooth water could feature small formed rippled wavelets which do not break but can include no more than small breaking white capping waves up to 0.5 metre. Riders at this level can operate craft in light winds up to 10 knots (Beaufort Scale).

It applies to leaders, guides or instructors, who use these skills when leading participants during riding activities. Leadership skills are provided in complementary units. The unit can also apply to assistants and support staff.

This unit applies to any type of organisation that delivers outdoor recreation activities including commercial, not-for-profit and government organisations.

Recreational boating is regulated by specific laws in each Australian state and territory with variable rules. General and specific waterway rules apply to the operation of personal watercraft. All training and assessment activities must comply with the local state or territory requirements

Most states and territories require personal water craft riders to hold a licence but they are not required to be certified as competent in this unit.

# Pre-requisite Unit

Nil

# Competency Field

Personal Water Craft

# Unit Sector

Outdoor Recreation

# Elements and Performance Criteria

| ELEMENTS | PERFORMANCE CRITERIA |
| --- | --- |
| Elements describe the essential outcomes | Performance criteria describe the performance needed to demonstrate achievement of the element. |
| 1. Prepare for the ride. | 1.1. Select lifejacket, exposure protection swim wear or exposure suit suitable for conditions, and check for safe working condition. 1.2. Complete PWC pre-start safety and serviceability checks according to manufacturers’ recommendations and correct basic deficiencies. 1.3. Waterproof, pack and stow clothing, personal resources, and food according to access requirements during ride. 1.4. Interpret planned course from activity plans and maps to assist in following the course. 1.5. Confirm activity safety and emergency response procedures to ensure compliance during activities. 1.6. Confirm protocols for communications between riders. 1.7. Fit and adjust exposure protection wear and lifejacket to ensure comfort and safety. |
| 2. Ride and manoeuvre personal water craft in smooth water conditions. | 2.1. Confirm number of passengers does not exceed craft design limits. 2.2. Launch and board craft, attach kill switch lanyard to self and start craft while maintaining stability. 2.3. Control speed and direction of craft forward, in reverse and throughout turns using appropriate level of throttle. 2.4. Maintain appropriate posture, balance and weight shift to efficiently handle and control the craft. 2.5. Monitor waterway hazards and other users consistently, and execute powered turns and emergency stops to avoid collisions. 2.6. Maintain visual contact and effective communication with party members throughout all activities. 2.7. Utilise bunch riding formation, monitor and make required adjustments to formation and speed. 2.8. Ensure compliance of self and party members with waterway rules for personal water craft. |
| 3. Rescue self and others following ejection or capsize in deep water. | 3.1. Identify immediate hazards, assess risks and negotiate to avoid injury to self and others. 3.2. Ensure engine has shut down and craft is stationary. 3.3. Follow manufacturer’s directions to avoid engine damage, and roll capsized craft to upright position. 3.4. Approach upright craft from stern, after ejection or capsize, and reboard while maintaining craft stability. 3.5. Manoeuvre rescue craft to ejected or capsized party member, provide clear and concise instructions. 3.6. Assist rescuee to return to their craft using rescue technique appropriate to situation. 3.7. Conduct a craft to craft tow to return disabled personal water craft to shore. |
| 4. Land and secure personal water craft. | 4.1. Land, alight and retrieve craft at shore while maintaining stability. 4.2. Secure craft for road transport using suitable methods. |

# Foundation Skills

| Foundation skills essential to performance in this unit, but not explicit in the performance criteria are listed here, along with a brief context statement. |  |
| --- | --- |
| SKILLS | DESCRIPTION |
| Reading skills to: | interpret detailed and familiar organisational safety and emergency response procedures interpret factual activity plan information in familiar formats interpret potentially unfamiliar and complex information about waterway rules. |
| Oral communications skills to: | use clear and unambiguous verbal and non-verbal communications to make intent known. |
| Numeracy skills to: | interpret and calculate numerical data involving distances, times and speeds. |
| Planning and organising skills to: | manage own timing to complete activities within planned timeframes. |
| Self-management skills to: | critically analyse all circumstances and implications to provide a prompt and considered response to rescue requirements. |

# Unit Mapping Information

No equivalent unit.

# Links

Companion Volume Implementation Guides - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=1ca50016-24d2-4161-a044-d3faa200268b

Assessment Requirements for SISOPWC001 Ride personal watercraft in smooth water conditions

# Modification History

Not applicable.

# Performance Evidence

Evidence of the ability to complete tasks outlined in elements and performance criteria of this unit in the context of the job role, and:

complete three group personal water craft (PWC) sessions

during each session, consistently:

follow safety procedures and safely negotiate hazards

follow waterway rules for PWC

control speed and direction of craft forward, in reverse and throughout turns using appropriate level of throttle

participate in simulations to:

complete three self-rescues following a capsize

complete four deep water rescues to assist a single rider:

two ejections, craft upright

two capsizes, craft upturned

conduct two craft to craft tows to return PWC to shore.

# Knowledge Evidence

Demonstrated knowledge required to complete the tasks outlined in elements and performance criteria of this unit:

organisational safety and emergency response procedures for personal water craft activities

exposure protection swim wear suitable for PWC activities:

types of swim wear and fabrics that protect against the effects of weather and water conditions including sun, temperatures, winds, and precipitation of different levels and extremes

features and uses of exposure suits, booties, water shoes, gloves and mittens of different styles and grades suited to different conditions

features and functions of lifejackets suitable for PWC activities in different types of waters and how to fit and adjust these for comfort and safety

location and function of these PWC parts:

deck and hull

handle bars, throttle and reverse levers

steering cable

ignition and kill switch

battery

engine and drive shaft

water pump, impellor and steering nozzle

intake grate

fuel tank

how personal water craft work, at a basic level of understanding

manufacturers’ recommendations for use

types of personal water craft suitable for use in smooth water conditions and:

different construction materials, effects on performance, advantages and disadvantages

design features, handling characteristics and limitations: manoeuvrability, stability and speed

types of pre-start safety and serviceability checks completed prior to riding, how to complete these and what tools would be used:

checking fuel, oil and battery fluid supplies

ensuring engine compartment is adequately ventilated with no excess fumes

looking for leaks in fuels lines, oil tanks, exhaust system

ensuring battery terminals are secure

checking that the pump/intake area is free of debris

checking for cracks and other damage to the hull

ensuring engine hood cover is securely latched

testing throttle is working

ensuring kill switch is working and lanyard is attached to self

techniques used to effectively stow items:

for ease of access and maximum usage of space

for stability of craft and minimal effect on trim

to waterproof clothing, food and resources

equipment features and techniques used to secure PWCs for transportation

communication protocols for group riding activities to include:

calls

hand signals

whistles

typical hydrology features and hazards for smooth waters, and techniques used to safely negotiate hazards:

currents

tides

built objects – piers, navigation markers

steep, slippery or rocky shores

sections of dark, deep or cold water

marine animals

other boating traffic

features of different locations that can be used to launch, board, land, alight and retrieve PWC including:

boat ramps

jetties

the shore

for each of the above locations:

factors that affect selection

advantages and disadvantages of use

techniques used

techniques used for riding and manoeuvring PWC in smooth water conditions:

sitting and standing

weight shifting

throttle control for speed and direction

controlling direction – forward, reverse, turns

for ejections, capsizes and rescues:

appropriate swimming strokes and techniques to use while wearing lifejackets in smooth water conditions

where to locate manufacturer’s directions for rolling PWC in water and importance of following

methods used to right upturned PWC

importance of avoiding craft with engine running and for reboarding from stern

equipment and towing techniques used for PWC to PWC tows

rules specific to the operation of personal water craft and specific to the local state or territory:

requirements for PWC registration and display of registration numbers and labels

requirement for safe operation sticker, where this is affixed and information inclusions

requirements to hold a carry a PWC licence

restrictions that apply to licence holders under 16 years old

alcohol and drug restrictions for riders and others

restrictions on hours of operation

restrictions for number of passengers

give way rules – passing, crossing and overtaking powered and non-powered vessels

distance limitations (“distance off”) – rules for swimmers, designated swimming zones, dive flags, unpowered and powered vessels stationary or underway, moored or anchored vessels, built structures (bridges, jetties, navigation markers, residential properties)

prohibited local areas – also called “exclusion” and “no go zones”

speed limits and any associated distance limitations, e.g. 4 knots when within 50metres of…

freestyling distance limitations; including what constitutes freestyling and how this is described in local rules, e.g. surfing, crossing or jumping waves, driving in irregular manner, erratic and non-directional operation

towing requirements – limitations, observer requirements, distance and speed limitations

incident reporting requirements including those for verbal and written reports

life jacket requirements

safety equipment requirements including those for kill switch safety lanyards and fire extinguishers.

# Assessment Conditions

Skills can be demonstrated in bodies of water which can include lakes, rivers, bays, harbours and sheltered coastline waters within breakwaters or headlands.

The environment must feature the following:

smooth water defined as water which features small formed rippled wavelets which do not break, or no more than small breaking white capping waves up to 0.5 metre

ideally light winds up to 10 knots (Beaufort Scale); winds could vary but warnings should not exceed light wind conditions.

The following resources must be available to replicate industry conditions of operation:

first aid equipment

communication equipment for emergency response.

Assessment must ensure use of:

a group of participants with whom the individual interacts during PWC activities

exposure protection swim wear suitable for conditions, and exposure suits as required

Australian Standard, or equivalent, compliant lifejackets of a grade that meets maritime regulator requirements

personal water craft

basic tool and repair kit

ropes and tie down straps

towlines

activity plans to include details of planned route

marine charts and guides

template safety and serviceability checklists

boating guide issued by the local state or territory maritime authority

organisational safety and emergency response procedures for personal water craft activities.

Assessors must satisfy the Standards for Registered Training Organisations requirements for assessors, and:

have a collective period of at least three years’ experience as a personal water craft leader, guide or instructor, where they have applied the skills and knowledge covered in this unit of competency; the three years’ experience can incorporate full and or part time experience

where required by local state or territory law, hold a compliant personal water craft driving licence.

# Links

Companion Volume Implementation Guides - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=1ca50016-24d2-4161-a044-d3faa200268b