# SISORSC002 Perform vertical rescues

Release: 1

SISORSC002 Perform vertical rescues

# Modification History

Not applicable.

# Application

This unit describes the performance outcomes, skills and knowledge required to establish ropes and belays for rescue rope systems, and to use single rope techniques and mechanical advantage systems to rescue self and others during single pitch activities.

Vertical single rope rescue skills are required in a range of activity contexts which can include abseiling, climbing, canyoning and caving.

It applies to leaders, guides or instructors, and others who use discretion and judgement to manage problems and emergencies when they arise.

This unit applies to any type of organisation that delivers outdoor recreation activities including commercial, not-for-profit and government organisations.

No occupational licensing, certification or specific legislative requirements apply to this unit at the time of publication.

# Pre-requisite Unit

Nil

# Competency Field

Rescue Operations

# Unit Sector

Outdoor Recreation

# Elements and Performance Criteria

| ELEMENTS | PERFORMANCE CRITERIA |
| --- | --- |
| Elements describe the essential outcomes | Performance criteria describe the performance needed to demonstrate achievement of the element. |
| 1. Determine rescue requirements and prepare for rescue. | 1.1. Assess situation, identify hazards promptly, assess risks and manage to avoid injury to self and others. 1.2. Determine appropriate rescue methods and techniques for situation and according to needs of person to be rescued. 1.3. Select equipment required for single rope rescues. 1.4. Determine roles and responsibilities of others; complete a briefing and provide clear and concise instructions throughout rescue. 1.5. Implement rescue operations according to organisational policies and procedures for emergency response. |
| 2. Establish systems for vertical rescues. | 2.1. Determine need for use of mechanical advantage systems, identify and establish type appropriate for the rescue situation. 2.2. Select and rig or establish multiple anchors, ensuring equalisation and minimise potential for shock loading. 2.3. Promptly establish rescue systems required to raise and lower persons from a single pitch. 2.4. Rig single rescue rope to minimise effects of rope stretch and rub points and for efficient descent and ascent during rescue. 2.5. Establish belays suitable for rescue circumstance and to ensure personal safety during rescues. 2.6. Verify manufacturers’ recommendations, equipment limitations and safety margins, and rig equipment accordingly. |
| 3. Descend and ascend a pitch using single rope techniques. | 3.1. Attach and adjust belay equipment to self and tie knots suitable for the established belay system. 3.2. Descend and ascend a fixed rope efficiently, using and adjusting techniques suitable for circumstances. 3.3. Safely approach, avoid or negotiate hazards to descend and ascend in a controlled manner. 3.4. Utilise techniques and equipment to adjust the single rope rig. 3.5. Choose optimum position and complete safe and efficient changeovers between descending and ascending. 3.6. Use techniques that minimise damage to equipment and the surface during activities. |
| 4. Rescue others. | 4.1. Establish communication with rescuee and provide clear and concise instructions and information throughout the rescue. 4.2. Operate rescue system to raise and lower persons from a single pitch. 4.3. Safely and efficiently escape a loaded belay system and transfer from belay device to direct connection to anchor. 4.4. Maintain safety of self, others and rescuee throughout rescue operations. |
| 5. Rescue self. | 5.1. Determine need for self rescue, assess situation and identify suitable techniques to resolve. 5.2. Anticipate hazards and utilise techniques to approach, avoid or negotiate throughout self rescue. 5.3. Implement extrication techniques to release from compromised situation. 5.4. Maintain personal safety throughout rescue operations. |
| 6. Debrief and complete reports. | 6.1. Participate in incident management debrief and identify future response improvements. 6.2. Complete required incident reports according to organisational procedures. |

# Foundation Skills

| Foundation skills essential to performance in this unit, but not explicit in the performance criteria are listed here, along with a brief context statement. |  |
| --- | --- |
| SKILLS | DESCRIPTION |
| Reading skills to: | interpret detailed familiar organisational policies and procedures for emergency response. |
| Writing skills to: | record detailed and accurate factual information about rescue responses. |
| Numeracy skills to: | complete simple calculations and approximations of loads for anchors, ropes and pulleys visually estimate distances to calculate lengths for ropes. |
| Teamwork skills to: | pro-actively and cooperatively work within teams of operational personnel to coordinate rescue logistics, solve arising problems and effect efficient rescues. |
| Self-management skills to: | critically analyse all circumstances and implications to coordinate a considered response to rescue requirements. |

# Unit Mapping Information

No equivalent unit.

# Links

Companion Volume Implementation Guides - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=1ca50016-24d2-4161-a044-d3faa200268b

Assessment Requirements for SISORSC002 Perform vertical rescues

# Modification History

Not applicable.

# Performance Evidence

Evidence of the ability to complete tasks outlined in elements and performance criteria of this unit in the context of the job role, and:

establish systems for and complete the following simulated vertical rescues:

two rescues of conscious persons

two self rescues

across the four rescues, collectively utilise the following equipment, techniques and methods:

select and tie at least four different types of knots suitable for the system type established, and appropriate for the intended load and function

mechanical advantage systems which include simple pulley systems with one moving pulley

raising rescuee

lowering rescuee

single rope techniques including:

changeovers from descending to ascending

changeovers from ascending to descending

changeover to another system while suspended on a rope

bypassing knots

for each rescue operation

designate roles and responsibilities to team members and participants to effectively complete rescues

maintain effective communication with the rescuee

participate in a debrief and complete an incident report.

# Knowledge Evidence

Demonstrated knowledge required to complete the tasks outlined in elements and performance criteria of this unit:

organisational policies and procedures for emergency response

roles and responsibilities that can be allocated to team members and participants to effectively complete a single rope rescue

features, functions, advantages and disadvantages of  different types of anchors:

fixed artificial

naturally occurring

artificial removable:

the effects of rope stretch during single rope operations and implications if not minimised:

abrasions

rope bounce

undue stress on the anchor system

stretch increases haulage effort

potential for rescuer and rescuee to impact with hazards, or become entangled or further entangled

features and functions of equipment used for single rope descents and ascents, and their specific application to rescue operations:

belay systems:

top rope top belay

bottom brake

self-belay

descending devices:

assisted locking

inline

plate

figure 8

tubular

improvised

belay devices:

assisted locking

inline

plate

figure 8

tubular

improvised

harnesses of different types

carabiners

maillon rapides

static and dynamic rope and when each might be used

rope protectors

tape

sewn sling

personal attachment systems, including cow’s tails

Prusik cords

foot loops

mechanical ascenders

mechanical advantage systems; simple pulley systems with one moving pulley

how the following factors affect the selection and rigging of above equipment for single rope rescues:

site characteristics including position of rub points

weather and environmental conditions

rescuee’s size, weight, and ability

manufacturers’ specifications for equipment use

techniques used to:

establish belay systems for single rope rescues

close the system; advantages and disadvantages of doing so

when different knots are used in rigging, advantages and disadvantages, and how to tie them:

fixed eye

mid line tied in the bight

end to end joining

termination

load control hitches

slide and grip hitches

safety requirements at the pitch head including use of personal attachment systems

techniques used to adjust the rope rig to include those for changing:

cord length

types of ascenders and descenders

type of ascending action used

single rope rescue techniques used to:

raise and lower conscious persons from single pitches

descend and ascend a fixed rope

lock off during descent

changeover from descending to ascending

changeover from ascending to descending

changeover to another system while suspended on a rope

bypass knots

secure rescuee and escape from a loaded belay system

typical hazards associated with single rope descents and ascents, and techniques used to safely negotiate.

# Assessment Conditions

Skills must be demonstrated in an above or below ground single pitch environment which might include cliff faces, canyons or caves with vertical to near vertical surfaces.

The following resources must be available to replicate industry conditions of operation:

first aid equipment

communication equipment for emergency response.

Assessments can only be completed through simulated activities. Simulations must incorporate time critical requirements.

Assessment must ensure use of:

people who act as team members, participants and rescuees with whom the individual interacts during simulated rescues

personal protective equipment to include:

gloves, as required

abseiling or climbing helmets

harnesses

anchors which can include:

fixed artificial

naturally occurring

artificial removable

single rope abseiling equipment:

carabiners

maillon rapides

rope which can include static and or dynamic rope

rope protectors

tape or sewn sling

personal attachment systems, including cow’s tails

Prusik cords

foot loops

mechanical ascenders

descending devices

belay devices

mechanical advantage systems

template incident reports

organisational policies and procedures for emergency response.

Assessors must satisfy the Standards for Registered Training Organisations requirements for assessors, and:

have a collective period of at least three years’ experience as a recreational activity leader, guide or instructor; or as a rescue operator where they have applied the skills and knowledge covered in this unit of competency; the three years’ experience can incorporate full and or part time experience.

# Links

Companion Volume Implementation Guides - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=1ca50016-24d2-4161-a044-d3faa200268b