# Assessment Requirements for SISOSCB005 Complete underwater search and recovery dives

Release: 1

Assessment Requirements for SISOSCB005 Complete underwater search and recovery dives

# Modification History

Not applicable.

# Performance Evidence

Evidence of the ability to complete tasks outlined in elements and performance criteria of this unit in the context of the job role, and:

plan and complete four search and recovery dives

across the four dives, collectively recover objects of these different weights:

small objects of minimum 1kg and maximum 5 kgs by hand without lift bags

objects of minimum 10kgs and maximum 45 kgs using lift bags

across the four recovery dives, use one of the following compass search patterns:

U search

expanding square

across the four recovery dives, use one of the following search patterns utilising search lines and reels:

circular

semi-circular

jackstay

during each dive consistently:

safely negotiate hazards

communicate effectively with buddy diver using underwater signals.

# Knowledge Evidence

Demonstrated knowledge required to complete the tasks outlined in elements and performance criteria of this unit:

differences between commercial and recreational search and recovery divers, at a basic level of understanding

roles and responsibilities of different recreational divers and support crew in search and recovery operations

different types of search and recovery dives

searching for specific lost items

searching for non-specific items of interest and reasons for not retrieving items of cultural or heritage significance

recovering harmful debris

how site characteristics of recovery dives may differ from optimum recreational diving conditions and how the following affects search and recovery techniques:

bottom topography and composition

water movement – current, tides and surge

hazards and risks associated with underwater search and recovery operations and techniques used to safely negotiate these:

sharp objects and debris

entanglement

low visibility

currents

falling objects

boating traffic

steps in planning search and recovery dives and why these are important:

defining the dive objective

collecting and analysing all available information; environmental conditions and hazards, characteristics of any lost objects, equipment required

selecting diving mode, search and recovery techniques

selecting diving buddy and other team members

briefing

techniques used on the surface to assist with location of submerged items:

tracking sinking items from the shore using landmarks as navigation aids

tracking sinking items from the water or on a boat using landmarks as navigation aids

taking compass bearings and using a slate for recording

for the following underwater and shore based search patterns, suitability for different circumstances, effectiveness and limitations:

U search

expanding square

circular

semi-circular

jackstay

shore walk

snorkeller led

for the above search patterns

how to mark starting points and searched areas

how to execute

how to use a compass in conjunction, as appropriate

how to secure and use search lines as appropriate

communication protocols, specific to search and recovery operations:

underwater line (rope) signals used between buddy divers

underwater line signals used between divers and surface crew

features, functions and operation of search and recovery equipment:

underwater compasses

buoys and anchors

bottom markers

slates

underwater reels and search lines

features, functions and operation of commercial lift bags:

advantages over home-made lift bags

weight ratings of different lift bags up to 45kgs

air sources for lift bags and techniques used to inflate both underwater and at the surface to achieve effective buoyancy

when different knots are used with lift bags, advantages and disadvantages, and how to tie them:

bowline

sheet bend

two half-hitch

dangers of using buoyancy control devices as lift bags

safe position of divers in relation to lift bags throughout recovery operations.

# Assessment Conditions

Skills must be demonstrated in an open water site. Dives can be completed from either boats or from the shore.

During assessment:

direct vertical access to the surface must be possible; aquatic plants are not considered an overhead obstruction

the following resources must be available to replicate industry conditions of operation:

first aid equipment

oxygen equipment

communication equipment for emergency response

diver flags of a type and size that meet local maritime regulator requirements.

Assessment must ensure use of:

a diving buddy with whom the candidate can dive

fins

masks

snorkels

cylinders and valves

buoyancy control devices with low pressure inflators

regulators with submersible pressure gauges

alternative air source

weight ballast systems

exposure suits, gloves and booties suitable for conditions

timing devices

depth gauges

underwater compasses and slates

surface floats and markers

anchors and bottom markers

underwater reels with search lines

commercial quality lift bags and rope

objects weighing a minimum 1kg and maximum 45kgs, with at least some objects weighing 10kgs.

Assessors must satisfy the Standards for Registered Training Organisations requirements for assessors, and:

be an individual who is currently certified and sanctioned by an industry authorised organisation to teach and assess underwater search and recovery skills.

# Links

Companion Volume Implementation Guides - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=1ca50016-24d2-4161-a044-d3faa200268b