# TLID0016 Operate a forklift

Release: 1

TLID0016 Operate a forklift

# Modification History

Release 1. This is the first release of this unit of competency in the TLI Transport and Logistics Training Package.

# Application

This unit involves the skills and knowledge required to operate a forklift. It includes checking forklift condition, driving forklift to fulfil operational requirements, and monitoring and maintaining forklift performance and site conditions in compliance with the relevant state/territory authority licence requirements and regulations, in a variety of operational contexts. Operation of a forklift is performed under supervision, generally within a team environment. No licensing, legislative or certification requirements apply to this unit at the time of publication.

# Pre-requisite Unit

Not applicable.

# Competency Field

D – Load Handling.

# Unit Sector

Not applicable.

# Elements and Performance Criteria

| Elements describe the essential outcomes. |  | Performance criteria describe the performance needed to demonstrate achievement of the element. |  |  |
| --- | --- | --- | --- | --- |
| 1 | Check forklift condition | 1.1 |  | Forklift pre-start is documented and completed prior to commencement of work and relevant to equipment |
|  |  | 1.2 |  | Forklift is checked for compliance in accordance with relevant legislation, manufacturer specifications and workplace procedures |
|  |  | 1.3 |  | Correct personal protective equipment (PPE) is selected and worn relevant to workplace procedures |
|  |  | 1.4 |  | Equipment attachments are selected, checked against data plate, secured correctly and appropriate to the task and terrain |
|  |  | 1.5 |  | Equipment faults and non-compliances are reported to relevant personnel |
| 2 | Drive forklift | 2.1 |  | Route is planned in accordance with traffic management plans for the worksite |
|  |  | 2.2 |  | Forklift is accessed correctly, using three points of contact, seatbelt is correctly worn and fastened, and mirrors and seats are adjusted for safe operation by driver |
|  |  | 2.3 |  | Forklift is started, steered, manoeuvred, positioned and stopped in accordance with regulations, manufacturer instructions and workplace procedures |
|  |  | 2.4 |  | Engine power is managed to ensure efficiency and performance and to minimise engine and gear damage |
|  |  | 2.5 |  | Operational hazards are identified and/or anticipated and avoided or controlled through defensive driving and appropriate hazard control techniques |
|  |  | 2.6 |  | Forklift is driven in reverse, maintaining visibility and achieving accurate positioning |
| 3 | Operate forklift to handle loads | 3.1 |  | Lifting task to be undertaken is appropriately planned, and correct lifting truck and allowed attachments selected |
|  |  | 3.2 |  | Load is lifted, carried, lowered and set down in accordance with WHS/OHS legislation, manufacturer specifications and workplace procedures |
| 4 | Monitor site conditions | 4.1 |  | Hazards and traffic flow are identified when selecting the most efficient route and appropriate adjustments are made |
|  |  | 4.2 |  | Site conditions are continuously monitored to enable safe operations and to ensure no injury to people or damage to property, equipment, loads or facilities occurs |
| 5 | Monitor and maintain forklift performance | 5.1 |  | Performance and efficiency of vehicle operation is monitored during use |
|  |  | 5.2 |  | Defective/irregular performance and malfunctions are reported to relevant personnel |
|  |  | 5.3 |  | Forklift records are maintained/updated in accordance with workplace procedures and legislative requirements |
| 6 | Perform shutdown | 6.1 |  | Forklift is parked, shut down, checklist completed and equipment secured in accordance with manufacturer specifications, regulations and workplace procedures |
|  |  | 6.2 |  | Defective/irregular performance, malfunctions or maintenance are recorded and reported to relevant personnel |
|  |  | 6.3 |  | PPE is stored correctly in accordance with workplace procedures |

# Foundation Skills

Foundation skills essential to performance are explicit in the performance criteria of this unit of competency.

# Range of Conditions

Range is restricted to essential operating conditions and any other variables essential to the work environment. Non-essential conditions can be found in the TLI Transport and Logistics Training Package Companion Volume Implementation Guide.

# Unit Mapping Information

This unit replaces and is not equivalent to TLID2010 Operate a forklift.

# Links

Companion Volume Implementation Guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=df441c6e-213d-43e3-874c-0b3f7036d851