# TLID4050 Discharge a dangerous goods tanker

Release: 1

TLID4050 Discharge a dangerous goods tanker

# Modification History

Release 1. This is the first release of this unit of competency in the TLI Transport and Logistics Training Package.

# Application

This unit involves the skills and knowledge required to pump discharge a dangerous goods tanker. It includes conducting pre-discharge activities, preparing for discharge, following discharge requirements and procedures, working at heights on a dangerous good tanker and finishing unloading. This unit applies to those working in the road transport fuel/gas industry. No licensing, legislative or certification requirements apply to this unit at the time of publication.

# Pre-requisite Unit

Not applicable.

# Competency Field

D – Load Handling

# Unit Sector

Not applicable.

# Elements and Performance Criteria

| ELEMENTS |  | PERFORMANCE CRITERIA |  |
| --- | --- | --- | --- |
| Elements describe the essential outcomes. |  | Performance criteria describe the performance needed to demonstrate achievement of the element. |  |
| 1 | Conduct pre-discharge activities | 1.1 | Delivery location is checked against site plan and delivery job sheet |
|  |  | 1.2 | Job hazards are identified and required action is taken to minimise, control or eliminate identified hazards |
|  |  | 1.3 | Tanker is positioned to enable discharging to be carried out in accordance with site procedures without injury to people or damage to property |
|  |  | 1.4 | Park brake is applied, engine turned is off and battery is isolated, as required |
|  |  | 1.5 | Personal protective equipment is used in accordance with site and organisational procedures |
|  |  | 1.6 | Site authority to proceed is obtained, as required, and followed in accordance with site organisational procedures |
| 2 | Prepare for discharging | 2.1 | Delivery location is checked for hazards and set up, in accordance with regulatory requirements and organisational procedures |
|  |  | 2.2 | Lids on spillbox are lifted and spillbox is drained of liquid |
|  |  | 2.3 | Identification (ID) markers and tumblers are checked against safe discharge unload plan to verify load will fit, as required |
|  |  | 2.4 | Hoses are removed from tanker and test date is checked |
| 3 | Follow discharge requirements and procedures | 3.1 | Site product type, quantity and discharged quantity are checked to determine whether receiving vessel can accept to load, and discrepancies are rectified in accordance with organisational procedures |
|  |  | 3.2 | Locations of emergency equipment and emergency procedures are identified before discharging commences |
|  |  | 3.3 | Product transfer equipment is connected in accordance with regulatory and organisational requirements |
|  |  | 3.4 | Internal valve is opened |
|  |  | 3.5 | Checks are completed prior to commencing discharge, in accordance with organisational procedures |
|  |  | 3.6 | Valve is fully opened and product is discharged safely, in accordance with regulatory and organisational requirements and site procedures |
|  |  | 3.7 | Discharging process is monitored in accordance with regulatory requirements, and company and site procedures |
|  |  | 3.8 | Tanks and compartments are changed in accordance with regulatory and organisational requirements and site procedures |
| 4 | Work at heights on a dangerous tanker | 4.1 | Safety equipment is connected to tanker prior to ascending tanker ladders, as required, in accordance with safety regulations and site procedures |
|  |  | 4.2 | Safety equipment is connected when operating on top of tanker, as required, in accordance with site and safety regulations |
|  |  | 4.3 | Work is completed in accordance with regulatory and organisational requirements and site procedures |
| 5 | Finish unloading | 5.1 | All valves on tanker and receiving facility are closed and transfer equipment is secured in accordance with site procedures |
|  |  | 5.2 | Site is re-dipped and readings are recorded on delivery documentation |
|  |  | 5.3 | Site is restored to a clean and tidy condition in accordance with site procedures and site defects are reported |
|  |  | 5.4 | Final safety inspection of tanker and site is conducted |
|  |  | 5.5 | Tanker is moved from unloading area without damage to property or injury to people and in accordance with site procedures |
|  |  | 5.6 | Delivery documentation is completed, checked and adjusted, and secured in tanker in accordance with regulatory and organisational requirements at completion of discharge |
|  |  | 5.7 | Truck and tanker are checked to ensure correct placarding before departure |
|  |  | 5.8 | Site plan is followed for exiting site |

# Foundation Skills

Foundation skills essential to performance are explicit in the performance criteria of this unit of competency.

# Range of Conditions

Range is restricted to essential operating conditions and any other variables essential to the work environment.

Non-essential conditions can be found in the Companion Volume Implementation Guide.

# Unit Mapping Information

This unit replaces and is equivalent to TLID4050A Discharge a dangerous goods tanker.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=df441c6e-213d-43e3-874c-0b3f7036d851