# Assessment Requirements for TLILIC0018 Licence to operate a non-slewing mobile crane (greater than 3 tonnes capacity)

Release: 1

Assessment Requirements for TLILIC0018 Licence to operate a non-slewing mobile crane (greater than 3 tonnes capacity)

# Modification History

Release 1. This is the first release of this unit of competency in the TLI Transport and Logistics Training Package.

# Performance Evidence

Evidence required to demonstrate competence in this unit must be relevant to and satisfy all the requirements of the elements and performance criteria on at least one occasion and include:

applying relevant mathematical calculations in conjunction with lift plan and load chart, radius requirements and relevant lifting gear to perform work/task to enable crane to be configured for load, including: boom fly-jib (where fitted) line pull travelling type of hook side slope derations articulation derations applying relevant crane movements, including: boom/jib up and down (luffing) positioning and using main and auxiliary hook and lifting gear to connect to load safely raising and lowering hoist telescope in and out travelling and articulating (as required) communicating with other workplace personnel through using appropriate worksite procedures, including: two-way radio listening making and interpreting hand signals questioning to confirm understanding signage verbal language visual aids whistles written instructions complying with Commonwealth, state and territory work health and safety (WHS)/occupational health and safety (OHS) legislation and safe work procedures completing pre-start checks, including: battery power level as required by manufacturer requirements engine/mechanical fluid level checks as required by manufacturer requirements presence of correct logbook evidence of damage fluid leaks lights work effectively locating, identifying and confirming all controls mirrors and seat are adjusted appropriately safety equipment checks signage and labels to ensure they are visible and legible checking for signs of paint separation and stressed welds indicating potential structural weakness tyres and wheels for damage/wear and correct inflation (water/air) updating records as required visual damage or equipment faults conducting and applying risk and hazard strategies, including: confirming work area operating surface suitability based on crane and task requirements articulation of crane dynamic loads ground conditions, including surface and slopes impact of tyre inflation/condition load swing overloading pick and placement of load asymmetric loads overhead hazards restricted site/s and poorly ventilated area/s risk of collision with people, moving plant and fixed structures adequate lighting weather conditions completing operational checks ensuring: all controls are located, identified and tested for functionality all hydraulic functions are operational lifting gear movements and control functions are smooth and comply with lift plan hazard warning systems, safety, audible and visual warning devices are checked to ensure they are functional, including: reversing beepers lights horns crane computer alarm (where fitted) anti-two block alarms (where fitted) start-up is in accordance with manufacturer requirements and workplace procedures there are no unusual noises steering, transmission and brake functions comply with operating requirements confirming and following traffic management plan procedures relevant to crane operator role in the work area identifying hazards and using appropriate risk controls and safety measures and equipment relevant to slinging loads confirming appropriate lifting gear and slinging techniques to the type of load, its mass and centre of gravity confirming the Working Load Limit (WLL) tags of the lifting equipment and gear and calculating the deration of the WLL resulting from the slinging techniques applied determining any defects or faults with operation of crane and reporting to relevant person/s ensuring risk control measures within the work area are effective in accordance with workplace procedures ensuring stability of load and avoidance of hazards, including: allowing for boom deflection boom/jib as low as possible boom/jib in line with crane carrying load near to ground surface crane stability whilst manoeuvring load into position with drive/steering wheels and articulating as required gently accelerating and braking to minimise load swing lowering load safely and stably onto appropriate dunnage taking into consideration swing and restrictions of area minimum boom/jib length minimum speed using handheld tag lines/bridling following directions of dogger or rigger interpreting and confirming relevant documentation for the work task and relevant area inputting crane configuration into crane computer (where fitted) and checking operation to accurately reflect crane configuration interpreting and acting on communications signals, including: hoist down - hand and whistle and radio hoist up - hand and whistle and radio luff boom down - hand and whistle and radio luff boom up - hand and whistle and radio articulate left - hand and whistle and radio articulate right - hand and whistle and radio stop - hand and whistle and radio telescope in - hand and whistle and two-way radio (where manufacturer requirements allow) telescope out - hand and whistle and two-way radio (where manufacturer requirements allow) travel - hand and radio maintaining three points of contact whilst accessing crane and ensure rungs/steps are free of hazards monitoring load disconnection from hook is safe and ensuring no movement of controls observing relevant communication signals from relevant person operating an articulated non-slewing mobile crane with a Rated Capacity (RC) of 12 tonnes or greater to lift four different loads using the main hook through an obstacle course using all crane operational controls while the load is in full view of the crane operator. Loads must consist of: a load of >50% of the RC of the crane with a boom length of >75%, and a round load with a minimum diameter of 300 mm and minimum length of three m that requires a dogger to sling, and an asymmetrical load that requires a dogger to sling, and travelling with a load of stillage containing at least ten scaffolding standards or containing a load of steel pipes of equivalent weight that requires a dogger to sling and a boom length of <75% positioning the non-slewing mobile crane for safe operation for: application of the task manoeuvring in the workplace aligning of crane boom to the load stability of the non-slewing mobile crane and the load whilst driving to load set down position recording and maintaining accurate information relating to crane operations reporting to relevant person/s on site risk control measures that are not in place or deficient setting up of: fly jib (where fitted) manual boom section (where fitted) setting up and validating an exclusion zone shutting down a non-slewing mobile crane in accordance with manufacturer requirements and workplace procedures planning for and managing load stability, including: confirming and inspecting appropriate lifting gear and applying slinging techniques appropriate to the type of load, its mass and centre of gravity confirming the WLL tags of the lifting equipment and gear and calculating the deration of the WLL resulting from the slinging techniques applied stabilising a non-slewing mobile crane for operation by ensuring level and articulation (if required) is checked and within deration load chart requirements test-lifting load just clear of lifting plane to allow for checks to be safely made in consultation with associated personnel to ensure: slinging is correct all crane equipment is functioning properly load centre of gravity is correct loads of unusual shape or weight distribution are correctly slung test-lifting load just clear of lifting plane to allow for checks of crane computer (where fitted) to ensure: load measuring equipment can be used to verify calculated weight of load near capacity loads do not overload crane.

# Knowledge Evidence

Evidence required to demonstrate competence in this unit must be relevant to and satisfy all of the requirements of the elements and performance criteria and include knowledge of:

appropriate worksite communication procedures, including: listening hand signals questioning techniques signage two-way radios written instructions whistles crane configuration mathematical calculations to: estimate loads establish counterweight/s requirements (where fitted) establish radius requirements relevant lifting gear to perform work/task characteristics and impact of factors affecting non-slewing mobile crane stability whilst mobiling loads, including: side slope derations articulation derations of crane correct tyre pressure (inflation/condition) driving safely on roadways pick up and carry the load crane and lifting gear load chart/s and manufacturer requirements lift-impacting factors, including: centre of gravity dynamic nature of load deflection of boom length radius of lift weight side slope derations articulation derations of crane tyre inflation pressures hazards, including: pack up and crane stability, crane tipping and demolition sites ground stability, including ground condition, recently filled trenches and slopes insufficient lighting obstacles or obstruction catching load swing appropriately other specific hazards and dangerous materials overhead hazards, including: electric lines service pipes fixed structures vegetation (trees) Traffic, including pedestrians, vehicles and other plant operations on unusual, uneven or difficult terrains impact of factors affecting non-slewing mobile crane stability, including: overloading pick up and placement of load unbalanced loads articulation of crane correct tyre pressures (inflation/condition) side slope derations manufacturer requirements and instructions on shutting down and packing up crane mobile non-slewing crane characteristics and capabilities to allow crane configuration to suit a range of loads relevant workplace instructions, safety information and emergency procedures relevant documentation requirements and procedures for recording, reporting and maintaining workplace records and information risk assessment management and mitigation strategies, including hierarchy of control: elimination substitution isolation engineering controls administrative controls personal protective equipment (PPE) roles and responsibilities of duty holders in accordance with legislative obligations of WHS/OHS requirements and safe work/workplace procedures hazards commonly encountered while slinging: instability of landing surfaces overhead and underground hazards insufficient lighting traffic weather pedestrian traffic work at heights selection, inspection, care, handling, application, limitations and storage of lifting equipment and gear: flexible steel wire rope (FSWR) sling synthetic sling chain sling (including shortener) spreader bar or lifting beam tag line shackles eyebolts methods of making temporary connections to loads using fibre and synthetic ropes: single sheet bend clove hitch rolling hitch bowline pre-start and operational checks required for a non-slewing mobile crane starting procedure of crane in accordance with manufacturer requirements set up of: jib fly jib (where fitted) manual boom section (where fitted) weather bureau forecasts and environmental conditions that could impact operation workplace standards, requirements, policies and procedures for conducting operations for the mobile non-slewing crane problems and applying appropriate response procedures to unplanned and/or unsafe situations and environmental conditions work area suitability based on relevant ground reports, including: backfilled ground bitumen concrete hard compacted soil pre-contaminated soils rock rough, uneven ground soft soils.

# Assessment Conditions

Assessors must hold credentials specified within the Standards for Registered Training Organisations current at the time of assessment. Assessment must satisfy the Principles of Assessment and Rules of Evidence and all regulatory requirements included within the Standards for Registered Training Organisations current at the time of assessment. Assessment processes and techniques must be appropriate to the language, literacy and numeracy requirements of the work being performed and the needs of the candidate. Assessment must occur in workplace operational situations where it is appropriate to do so; where this is not appropriate, assessment must occur in simulated workplace operational situations that replicate workplace conditions. Simulators must not be used in the assessment of this unit of competency. Resources for assessment must include access to: non-slewing articulated mobile crane with an MRC 12 tonnes or greater in safe/serviceable working order in accordance with manufacturer specifications appropriate loads as outlined in the Performance Evidence requirements associated personnel to sling and direct loads, including: licenced dogger or rigger communications equipment, including: two-way radios whistles personal protective equipment (PPE) relevant documentation for operating a non-slewing mobile crane over three tonnes, including: approved codes of practice and relevant guidance material relevant Australian technical standards manufacturer guidelines (instructions, requirements or checklists), relevant industry standards and operating procedures (where applicable).

# Links

Companion Volume Implementation Guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=df441c6e-213d-43e3-874c-0b3f7036d851