# TLIS0004 Measure and mark track for resurfacing

Release: 1

TLIS0004 Measure and mark track for resurfacing

# Modification History

Release 1. This is the first release of this unit of competency in the TLI Transport and Logistics Training Package.

# Application

This unit involves the skills and knowledge required to undertake measurement and marking of track for resurfacing operations in accordance with rail network standards, safeworking and regulatory requirements, and workplace procedures. It includes identifying and marking track obstructions, using appropriate measurement tools to measure the range of geometry elements, calculating recording and marking data for resurfacing operations, and recording and reporting results. People achieving competence in this unit will need to fulfil the applicable state/territory legislated rail safety requirements and to comply with relevant codes of practice and/or guidelines. Work is performed under minimal supervision, generally within a team environment. It involves the application of routine operational principles and procedures when measuring and marking track for resurfacing across a variety of operational contexts. No licensing, legislative or certification requirements apply to this unit at the time of publication.

# Pre-requisite Unit

Not applicable.

# Competency Field

S – Construction and Installation

# Unit Sector

Rail sector.

# Elements and Performance Criteria

| ELEMENTS |  | PERFORMANCE CRITERIA |  |
| --- | --- | --- | --- |
| Elements describe the essential outcomes. |  | Performance criteria describe the performance needed to demonstrate achievement of the element. |  |
| 1 | Plan for resurfacing | 1.1 | Scope of resurfacing work, including ramp-in and ramp-out points, mode of machine application and requirements for measurements are determined |
|  |  | 1.2 | Appropriate survey or relevant documentation is selected in accordance with workplace procedures |
|  |  | 1.3 | Survey plaques and monuments are located and accurately interpreted as required |
|  |  | 1.4 | Hazards are identified, risks are assessed and control measures are implemented |
| 2 | Mark or protect obstructions | 2.1 | Track obstructions are identified |
|  |  | 2.2 | Track obstructions are marked or protected, as required, in accordance with workplace procedures |
| 3 | Measure geometry and record results | 3.1 | Equipment appropriate to task is selected and checked in accordance with manufacturer instructions and workplace procedures |
|  |  | 3.2 | Equipment is operated in accordance with manufacturer or work instructions to provide appropriate data for analysis |
|  |  | 3.3 | Track geometry is measured in accordance with workplace procedures |
|  |  | 3.4 | Measurements are recorded in accordance with workplace procedures |
| 4 | Mark track | 4.1 | Offsets and machine adjustment values are calculated as required and recorded in accordance with workplace procedures |
|  |  | 4.2 | Track data is marked on track as required in accordance with workplace procedures |
| 5 | Use/report data | 5.1 | Data is compared to track maintenance/construction tolerances in accordance with workplace procedures |
|  |  | 5.2 | Results are reported in accordance with workplace procedures |

# Foundation Skills

Foundation skills essential to performance are explicit in the performance criteria of this unit of competency.

# Range of Conditions

Range is restricted to essential operating conditions and any other variables essential to the work environment.

# Unit Mapping Information

This unit replaces and is equivalent to TLIS3039 Measure and mark track for resurfacing.

# Links

Companion Volume Implementation Guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=df441c6e-213d-43e3-874c-0b3f7036d851