# Assessment Requirements for TLIW2023 Manufacture pallets using manual methods

Release: 1

Assessment Requirements for TLIW2023 Manufacture pallets using manual methods

# Modification History

Release 1. This is the first release of this unit of competency in the TLI Transport and Logistics Training Package.

# Performance Evidence

Evidence required to demonstrate competence in this unit must be relevant to and satisfy all of the requirements of the elements and performance criteria on at least one occasion and include:

adapting to differences in materials and equipment in accordance with operating procedures applying precautions and required action to minimise, control or eliminate identified hazards applying relevant legislation and workplace procedures carrying out visual inspection of board quality in accordance with relevant Australian Standards communicating and working effectively with others identifying and correctly using relevant equipment maintaining tools and equipment used in the manufacture of pallets using manual methods modifying activities depending on operational contingencies, risk situations and environments monitoring and prioritising work activities in terms of planned schedule planning own work, predicting consequences and identifying improvements reading, interpreting and following relevant instructions, procedures, information and signs reporting and/or rectifying identified problems, faults or malfunctions promptly, in accordance with regulatory requirements and workplace procedures selecting and using required personal protective equipment conforming to industry and work health and safety (WHS) /occupational health and safety (OHS) standards working systematically with required attention to detail without injury to self or others, or damage to goods or equipment.

# Knowledge Evidence

Evidence required to demonstrate competence in this unit must be relevant to and satisfy all of the requirements of the elements and performance criteria and include knowledge of:

documentation and record requirements housekeeping standards and procedures operational work systems, equipment, management and site operating systems for manufacturing pallets using manual methods problems that may occur and appropriate action that can be taken to resolve these problems regulations relevant for manufacturing pallets using manual methods relevant WHS/OHS and environmental protection procedures and guidelines site layout and obstacles tools and equipment used for manufacturing pallets using manual methods and the precautions and procedures that should be followed in their use workplace procedures and policies for manufacturing pallets using manual methods.

# Assessment Conditions

As a minimum, assessors must satisfy applicable regulatory requirements, which include requirements in the Standards for Registered Training Organisations current at the time of assessment. As a minimum, assessment must satisfy applicable regulatory requirements, which include requirements in the Standards for Registered Training Organisations current at the time of assessment. Assessment must occur in workplace operational situations where it is appropriate to do so; where this is not appropriate, assessment must occur in simulated workplace operational situations that replicate workplace conditions. Assessment processes and techniques must be appropriate to the language, literacy and numeracy requirements of the work being performed and the needs of the candidate. Resources for assessment include: a range of relevant exercises, case studies and/or simulations applicable documentation including workplace procedures, regulations, codes of practice and operation manuals relevant materials, tools, equipment and personal protective equipment currently used in industry.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=df441c6e-213d-43e3-874c-0b3f7036d851