# TLIB3016A De-rig, pack and store tools and equipment

Release: 1

TLIB3016A De-rig, pack and store tools and equipment

# Modification History

Not Applicable

# Unit Descriptor

| Unit Descriptor | This unit involves the skills and knowledge required to de-rig a mobile crane and to pack and store tools and equipment, including inspecting tools and equipment, securing outriggers, packing and storing tools and equipment and completing necessary records. Licensing, legislative, regulatory or certification requirements are applicable to this unit. |
| --- | --- |

# Application of the Unit

| Application of the Unit | Work must be carried out in compliance with the licence/permit requirements and regulations of the relevant state/territory authorities pertaining to mobile crane operations. Work is performed under general supervision, with some accountability and responsibility for self and others in achieving the prescribed outcomes. It involves the application of routine principles and procedures to the de-rigging of a mobile crane and the packing and storage of tools and equipment in a variety of operational contexts. |
| --- | --- |

# Licensing/Regulatory Information

Refer to Unit Descriptor

# Pre-Requisites

Not Applicable

# Employability Skills Information

| Employability Skills | This unit contains employability skills. |
| --- | --- |

# Elements and Performance Criteria Pre-Content

| Elements describe the essential outcomes of a unit of competency. | Performance criteria describe the required performance needed to demonstrate achievement of the element. Assessment of performance is to be consistent with the evidence guide. |
| --- | --- |

# Elements and Performance Criteria

| ELEMENT | PERFORMANCE CRITERIA |
| --- | --- |
| 1 Inspect tools and equipment | 1.1 Rigging gear is removed and inspected for damage and defects 1.2 Damaged and defective gear is tagged and segregated for testing/repair/disposal by a qualified person |
| 2 Secure outriggers | 2.1 Any outriggers, stabilisers, counterweights and attachments are secured and stowed in accordance with manufacturers instructions 2.2 Outrigger packing is dismantled and stowed securely |
| 3 Pack and store tools and equipment | 3.1 Gear is cleaned as required and stored appropriately 3.2 Additional gear is returned to depot, marked off and stored according to workplace procedures 3.3 Other personnel are assisted as necessary to ensure efficient pack up |
| 4 Complete records | 4.1 Required workplace records are updated and processed accurately and promptly |

# Required Skills and Knowledge

REQUIRED KNOWLEDGE AND SKILLS

This describes the essential knowledge and skills and their level required for this unit.

Required knowledge:

Relevant road rules, regulations, permit and licence requirements pertaining to mobile crane operation

Relevant OH&S and environmental procedures and regulations

Mobile crane applications, capacities, configurations, safety hazards and control mechanisms

Operational procedures for crane crews concerning the de-rigging of cranes and the packing and storing of tools and equipment

Defects that may be identified when de-rigging a crane and appropriate action that should be taken

Guidelines relating to the safe use of machinery and equipment

Focus of operation of work systems and equipment

Application of relevant agreements, codes of practice or other legislative requirements

Required skills:

Communicate effectively with others when de-rigging a mobile crane

Read and interpret instructions, procedures, information and signs relevant to the de-rigging of a mobile crane

Interpret and follow operational instructions and prioritise work

Operate electronic communication equipment to required protocol

Work collaboratively with others when de-rigging a mobile crane

Adapt appropriately to cultural differences in the workplace, including modes of behaviour and interactions with others

Promptly report and/or rectify any identified problems, faults or malfunctions in accordance with regulatory requirements and workplace procedures

Implement contingency plans for unexpected events that may occur when de-rigging a mobile crane

Apply precautions and required action to minimise, control or eliminate hazards that may exist during work activities

Prioritise and multi-task work

Plan own work including predicting consequences and identifying improvements

Modify activities depending on differing operational contingencies, risk situations and environments

Apply fatigue management knowledge and techniques

Work systematically with required attention to detail without injury to self or others, or damage to goods or equipment

Operate and adapt to differences in equipment in accordance with standard operating procedures

Select and use required personal protective equipment conforming to industry and OH&S standards

Check condition of equipment when de-rigging a mobile crane

Identify and correctly use equipment, processes and procedures

# Evidence Guide

| EVIDENCE GUIDE |  |
| --- | --- |
| The evidence guide provides advice on assessment and must be read in conjunction with the performance criteria, required knowledge and skills, the range statement and the assessment guidelines for this Training Package. |  |
| Critical aspects for assessment and evidence required to demonstrate competency in this unit | The evidence required to demonstrate competency in this unit must be relevant to and satisfy all of the requirements of the elements and performance criteria of this unit and include demonstration of applying: the underpinning knowledge and skills relevant legislation and workplace procedures other relevant aspects of the range statement |
| Context of and specific resources for assessment | Performance is demonstrated consistently over a period of time and in a suitable range of contexts Resources for assessment include: a range of relevant exercises, case studies and/or other simulated practical and knowledge assessment, and/or access to an appropriate range of relevant operational situations in the workplace In both real and simulated environments, access is required to: relevant and appropriate materials and equipment, and applicable documentation including workplace procedures, regulations, codes of practice and operation manuals |
| Method of assessment | Assessment of this unit must be undertaken by a registered training organisation As a minimum, assessment of knowledge must be conducted through appropriate written/oral tests Practical assessment must occur: through activities in an appropriately simulated environment at the registered training organisation, and/or in an appropriate range of situations in the workplace |

# Range Statement

| RANGE STATEMENT |  |
| --- | --- |
| The range statement relates to the unit of competency as a whole. It allows for different work environments and situations that may affect performance. |  |
| Operations may be conducted: | day or night in a variety of weather conditions |
| Environment may include: | movement of equipment, goods, materials and vehicular traffic |
| Customers may be: | internal or external |
| Mobile crane may be involved in work in a range of industry sectors including: | construction and demolition manufacturing waterfront mining primary industry utilities (electricity, gas, water) arboricultural swimming pool quarrying |
| Hazards may include: | power lines noise, light, energy sources overhead service lines surrounding buildings, structures, facilities underground services obstructions uneven or unstable ground and recently filled trenches stationary and moving machinery and equipment hazardous or dangerous materials traffic hazards and congestion other vehicles and personnel |
| Hazard management is: | consistent with the principle of hierarchy of control with elimination, substitution, isolation and engineering control measures being selected before safe working practices and personal protective equipment |
| Consultative processes may involve: | other employees and supervisors management union representatives clients industrial relations and OH&S specialists other professional or technical staff |
| Personal protective equipment may include: | gloves safety headwear and footwear sunscreen, sunglasses and safety glasses two-way radios high visibility clothing |
| Depending on the type of organisation concerned and the local terminology used, workplace procedures may include: | company procedures enterprise procedures organisational procedures established procedures |
| Documentation/information may include: | operations manuals including load charts and crane and rigging manuals induction documentation competency standards and training materials job specifications and procedures manufacturers specifications workplace operating procedures and policies supplier and/or client instructions communications technology equipment, oral, aural or signed communications personal and work area work procedures and practices conditions of service, legislation and industrial agreements including: workplace agreements and awards occupational health&safety procedures standards and certification requirements quality assurance procedures emergency procedures |
| Applicable regulations and legislation may include: | relevant state/territory regulations and licence/permit requirements pertaining to mobile cranes relevant state/territory OH&S legislation relevant state/territory environmental protection legislation |

# Unit Sector(s)

Not Applicable

# Competency Field

| Competency Field | B - Equipment Checking and Maintenance |
| --- | --- |