# TLILIC3004A Licence to operate a derrick crane

Release: 1

TLILIC3004A Licence to operate a derrick crane

# Modification History

Not Applicable

# Unit Descriptor

| Unit Descriptor | This unit specifies the outcomes required to operate a slewing strut-boom crane with its boom pivoted at the base of a mast which is either guyed (guy-derrick) or held by backstays (stiff-legged derrick) and which is capable of luffing under load for licensing purposes. |
| --- | --- |

# Application of the Unit

| Application of the Unit | This unit requires the operator to plan the work, conduct routine checks, transfer loads, and shut down and secure the derrick crane. This unit is based on the requirements of the National Standard for Licensing Persons Performing High Risk Work. This unit in its current form meets state and territory licensing requirements. Any alteration will result in a unit which is not acceptable to regulators for the purpose of licensing. |
| --- | --- |

# Licensing/Regulatory Information

Refer to Unit Descriptor

# Pre-Requisites

Not Applicable

# Employability Skills Information

| Employability Skills | This unit contains employability skills. |
| --- | --- |

# Elements and Performance Criteria Pre-Content

| Elements describe the essential outcomes of a unit of competency | Performance criteria describe the required performance needed to demonstrate achievement of the element. Where bold italicised text is used, further information is detailed in the required skills and knowledge and/or the range statement. Assessment of performance is to be consistent with the evidence guide. |
| --- | --- |

# Elements and Performance Criteria

| ELEMENT | PERFORMANCE CRITERIA |
| --- | --- |
| 1. Plan work | 1.1 Potential workplace hazards are identified 1.2 Hazard control measures are identified consistent with appropriate standards to ensure the safety of personnel and equipment 1.3 The weight of the load is identified and estimated in consultation with associated personnel 1.4 Crane is appropriate to the load/s and workplace conditions 1.5 The appropriate path for the movement of loads in the work area is determined and inspected 1.6 Appropriate communication methods are identified with associated personnel |
| 2. Conduct routine checks | 2.1 Ground stability is checked 2.2 Crane is visually checked for any damage or defects 2.3 All signage and labels are visible and legible according to the appropriate standard 2.4 Appropriate crane configuration for work task is determined according to procedures (where applicable) 2.5 Routine pre-operational crane checks are carried out according to procedures 2.6 All controls are located and identified 2.7 Crane service logbook is checked for compliance 2.8 Crane is start according to procedures and checked for any abnormal noises 2.9 All crane safety devices are tested according to procedures 2.10 Post-start operational checks are carried out according to procedures 2.11 All communication equipment is checked for serviceability 2.12 All damage and defects are reported and recorded according to procedures, and appropriate action is taken |
| 3. Transfer load | 3.1 Lifts are determined within the capacity of the crane 3.2 Appropriate hazard prevention/control measures are applied to the work area according to procedures 3.3 Boom/jib and hoist block is positioned over load following directions from associated personnel 3.4 Test lift is carried out according to procedures 3.5 Loads are transferred using all relevant crane movements according to procedures and the appropriate standard 3.6 All required communication signals are interpreted correctly according to procedures and the appropriate standard 3.7 Crane is operated according to procedures. 3.8 Load movement is monitored constantly ensuring safety to personnel and load, and structural stability 3.9 Unplanned and/or unsafe situations are responded to in line with procedures |
| 4. Shut down and secure crane | 4.1 Relevant motion locks and brakes are applied (where applicable) 4.2 Crane equipment is stowed and secured where appropriate according to procedures and the appropriate standard 4.3 Crane is shut down according to procedures 4.4 Routine post-operational crane checks are carried out according to procedures 4.5 All damage and defects are reported and recorded according to procedures, and appropriate action is taken |

# Required Skills and Knowledge

| REQUIRED SKILLS AND KNOWLEDGE |  |
| --- | --- |
| This describes the essential skills and knowledge and their level required for this unit. |  |
| Required skills: |
| Accurately record and maintain information relating to crane operations Use communication techniques in the workplace including whistles, hand signals and use of two-way radios Use interpersonal communication skills at a level sufficient to communicate with other site personnel Operate a derrick crane for the lifting and moving of loads to the safe working rated capacity in conjunction with other associated personnel Apply risk assessment and hazard control strategies, including hierarchy of control as applied to the safe operation of the crane (particular awareness of the risks associated with overhead powerlines/electrical cables and other personnel or vehicles) Use and interpret crane manufacturer's specifications and data, or engineers specifications, assessments or designs, including load charts, or load limits at various radii, to enable the crane to be configured for the load Verify problems and equipment faults and demonstrate appropriate response procedures |
| Required knowledge: |
| Appropriate mathematical procedures for estimation of loads Commonwealth, state or territory OH&S legislation, standards and codes of practice relevant to the full range of processes for the crane class Derrick crane characteristics and capabilities to allow for the operation of the crane to suit the range of loads Level of literacy to be able to read and comprehend manufacturer's instructions, procedures and safety signs Organisational and workplace standards, requirements, policies and procedures for conducting operations for the crane class Derrick crane operating techniques Understanding of the hierarchy of hazard identification and control Procedures for the recording, reporting and maintenance of workplace records and information Rated capacity and working load limits (including use of crane load charts) Typical routine problems encountered in the operation of the crane and equipment and adjustments required for correction |

# Evidence Guide

| EVIDENCE GUIDE |  |
| --- | --- |
| The evidence guide provides advice on assessment and must be read in conjunction with the performance criteria, required skills and knowledge, the range statement and the assessment guidelines for the Training Package. |  |
| Overview of assessment | Successful assessment of this unit meets the competency requirement of the National Standard for Licensing Persons Performing High Risk Work. State/territory OH&S regulators have mandated the use of Assessment Instruments and Instructions for Assessment for this unit which have been endorsed by the national body responsible for OH&S matters. |  |
| Critical aspects for assessment and evidence required to demonstrate competency in this unit | Compliance with OH&S licensing legislation. Effectively communicate and work safely with others in the work area. Risk assessment and management procedures (particular awareness of the risks associated with overhead powerlines/electrical cables, ground conditions, crane tipping and demolition sites). Operation of a derrick crane for the lifting and moving of loads to the safe working rated capacity in conjunction with other associated personnel. Appropriate mathematical procedures for the estimation of loads. |  |
| Context of and specific resources for assessment | Assessment of the safe and effective application of knowledge and skill to workplace tasks (performance) must be undertaken using the endorsed Assessment Instrument. Assessment of performance must be undertaken either in the workplace or in a realistically simulated workplace setting. Assessors must ensure that the assessment in the workplace is organised to ensure that all the required equipment and materials and a suitable working area is made available to suit the assessment and the workplace. Assessment must occur under standard and authorised work practices, safety requirements and environmental constraints. Assessment is to comply with relevant appropriate standard requirements. Applicants must have access to: Personal Protective Equipment (PPE) for the purpose of the Performance Assessment. appropriate derrick crane and associated equipment in safe condition specified by the endorsed Assessment Instrument suitable loads as specified by the endorsed Assessment Instrument communication equipment (e.g. two-way radios, whistles, etc.) other associated personnel to sling and direct the loads. |  |
| Method of assessment | Assessment must be conducted using the endorsed Assessment Instruments. These Instruments provide advice on their application. The use of 'simulators' in the assessment of this unit of competency is not acceptable. Assessment may be in conjunction with the assessment of other units of competency. Assessment methods must confirm consistency and accuracy of performance together with application of underpinning knowledge. Assessment must confirm a reasonable inference that competency is not only able to be satisfied under the particular circumstance, but is able to be transferred to other circumstances. |  |
| Guidance information for assessment | Further information about endorsed Assessment Instruments may be obtained from state/territory OH&S regulators. |  |

# Range Statement

| RANGE STATEMENT |  |
| --- | --- |
| The range statement relates to the unit of competency as a whole. It allows for different work environments and situations that may affect performance. Bold italicised wording, if used in the performance criteria, is detailed below. |  |
| Hazards | May include but not limited to: ground stability (e.g. ground condition or slopes for load placement) overhead hazards (e.g. powerlines, service pipes) insufficient lighting traffic (e.g. pedestrians, vehicles, plant) environmental conditions (e.g. wind, lightning, storms) other specific hazards (e.g. dangerous materials) |
| Hazard control measures | Refers to the systematic process of eliminating or reducing the risk to personnel and property through the application of controls It includes the application of the hierarchy of control, the six-step preference of control measures to manage and control risk: 1 elimination 2 substitution 3 isolation 4 engineering control measures 5 using safe work practices 6 personal protective equipment |
| Appropriate standards | May include: codes of practice legislation Australian standards manufacturer's instructions industry standards (where applicable) |
| Associated personnel | May include but not limited to: riggers doggers |
| Crane | Includes derrick cranes comprising a strut-boom crane with its boom pivoted at the base of a mast which is either guyed (guy-derrick) or held by backstays (stiff-leg derrick) and which is capable of luffing under load |
| Appropriate | May include but not limited to: crane capabilities environmental conditions (e.g. wind, lightning, storms etc.) |
| Communication method | May include but not limited to: verbal and non-verbal language written instructions signage hand signals listening questioning to confirm understanding appropriate worksite protocol |
| Ground stability | May include but not limited to: environmental conditions (e.g. wind, lightning, storms, etc.) |
| Signage and labels | May include but not limited to: crane data plates/labels load charts crane decals control labels |
| Crane configuration | May include but not be limited to: boom/jib backstays and counterweights guys |
| Procedures | May include but not limited to: manufacturer's guidelines (instructions, specifications or checklists) industry operating procedures workplace procedures (work instructions, operating procedures, checklists) |
| Controls | May include but not limited to: luffing levers hoisting and lowering levers slewing levers including brake |
| Service logbook | May include bur not limited to: any logbook service book history record system where the service and maintenance history is kept |
| Crane safety devices | May include but not limited to: audible and visual warning devices lights function limits |
| Communication equipment | May include but not limited to: two-way radios whistles bells buzzers NB: where radio communication equipment is used the transmitting frequencies of the equipment must be selected to prevent interference to or from other radio equipment being used in the vicinity of the crane |
| Hazard prevention/control measures | May include but not limited to: safety tags on electrical switches/isolators insulated powerlines safety observer used inside exclusion zone disconnected power traffic barricades and control/s pedestrian controls trench covers movement of obstructions personal protective equipment adequate illumination |
| Test lift means | The load is lifted just clear of the lifting plane to allow for checks to be safely made in consultation with associated personnel to ensure that: near capacity loads do not overload the crane loads of unusual shape or weight distribution are correctly slung load measuring equipment can be used to verify the calculated weight of the load all crane equipment is functioning properly adjustments to the slinging can be made in a safe manner |
| Relevant crane movements | May include but not limited to: luffing slewing hoisting and lowering loads |
| Communication signals | May include but not limited to: stop - hand stop - whistle hoist up - hand hoist up - whistle hoist down - hand hoist down - whistle luff boom down - hand luff boom down - whistle luff boom up - hand luff boom up - whistle slew left - hand slew left - whistle slew right - hand slew right - whistle |
| Unplanned and/or unsafe situations | May include but not limited to: failure/loss of control (e.g. slew brakes, hoist drum) failure of equipment (e.g. hydraulic system) environmental conditions (e.g. wind, lightning, storms, etc.) |
| Shut down | May include but not limited to: raising boom/jib to clear buildings and structures (where applicable) retracting hoist rope and hook block idling engine to stabilise temperature (where applicable) turning off engine (where applicable) isolating power supply to crane removing key (where applicable) locking and securing cabin (where applicable) |

# Unit Sector(s)

Not Applicable