# UEEEL0045 Diagnose and rectify faults in traction lift systems

Release: 2

UEEEL0045 Diagnose and rectify faults in traction lift systems

# Modification History

Release 2. Minor Change. Typographic error in prerequisites corrected.

Release 1. This is the first release of this unit of competency in the UEE Electrotechnology Training Package.

# Application

This unit involves the skills and knowledge required to diagnose and rectify faults in traction lift systems and equipment. It includes working safely; replacing and/or adjusting lift circuit and associated components; diagnosing and repairing faults in lift circuits and associated components, including governors, brakes, safety gear, safety devices, lift machines, door components and controllers. The skills and knowledge described in this unit require a licence or permit to practice in the workplace where work is carried out on electrical installations which are designed to operate at voltages greater than 50 volt (V) alternating current (a.c.) or 120 V direct current (d.c.). Competency development activities in this unit are subject to regulations directly related to licensing. Where a licence or permit to practice is not held, a relevant contract of training, such as an Australian Apprenticeship, may be required. Additional and/or other conditions may apply in some jurisdictions subject to regulations related to electrical work. Practice in the workplace and during training is also subject to work health and safety (WHS)/occupational health and safety (OHS) regulations.

# Pre-requisite Unit

UEECD0007 Apply work health and safety regulations, codes and practices in the workplace UEECD0019 Fabricate, assemble and dismantle utilities industry components UEECD0020 Fix and secure electrotechnology equipment UEECD0051 Use drawings, diagrams, schedules, standards, codes and specifications UEEEL0003 Arrange circuits, control and protection for general electrical installations UEEEL0020 Solve problems in low voltage a.c. circuits UEEEL0023 Terminate cables, cords and accessories for low voltage circuits UEEEL0019 Solve problems in direct current (d.c.) machines UEEEL0021 Solve problems in electromagnetic devices UEEEL0014 Isolate, test and troubleshoot low voltage electrical circuits UEEEL0008 Evaluate and modify low voltage heating equipment and controls UEEEL0009 Evaluate and modify low voltage lighting circuits, equipment and controls UEEEL0010 Evaluate and modify low voltage socket outlets circuits UEEEL0024 Test and connect alternating current (a.c.) rotating machines UEEEL0025 Test and connect transformers and UEECD0043 Solve problems in direct current circuits or UEECD0044 Solve problems in multiple path circuits UEECD0046 Solve problems in single path circuits

# Competency Field

Electrical

# Unit Sector

Electrotechnology

# Elements and Performance Criteria

| ELEMENTS |  | PERFORMANCE CRITERIA |  |
| --- | --- | --- | --- |
| Elements describe the essential outcomes. |  | Performance criteria describe the performance needed to demonstrate achievement of the element. |  |
| 1 | Prepare to diagnose and rectify lift system faults | 1.1 | WHS/OHS requirements and workplace procedures for a given work area are identified and applied |
|  |  | 1.2 | WHS/OHS risk control measures and workplace procedures in preparation for work are followed |
|  |  | 1.3 | Scope of work to be undertaken is determined from workplace procedures or fault/breakdown reports and/or discussions with appropriate person/s |
|  |  | 1.4 | Advice is sought from the work supervisor to ensure work is coordinated effectively with others |
|  |  | 1.5 | Tools, equipment and testing devices to carry out work are obtained in accordance with workplace procedures and checked for correct operation and safety |
| 2 | Diagnose and rectify lift system faults | 2.1 | WHS/OHS risk control measures and workplace procedures for carrying out work are followed |
|  |  | 2.2 | Need to test or measure live electrical work is determined in accordance with WHS/OHS requirements and, as required, conducted within workplace safety procedures |
|  |  | 2.3 | Circuits/machines/plant are checked and isolated in accordance with WHS/OHS requirements and workplace procedures |
|  |  | 2.4 | Safety hazards resulting from fault or breakdown are documented on job safety assessments, risks assessed and control measured implemented in consultation with appropriate person/s |
|  |  | 2.5 | Logical diagnostic methods are applied to diagnose lift system apparatus faults employing measurements and estimations of system operating parameters referenced to system operational requirements |
|  |  | 2.6 | Suspected fault-finding scenarios are tested as being possible cause/s of lift system fault in accordance with workplace procedures |
|  |  | 2.7 | Cause of lift system fault/s is identified and appropriately competent person/s engaged to rectify the fault where it is outside the scope of the lift systems |
|  |  | 2.8 | Fault/s in the lift components of the system is rectified to apparatus manufacturer specifications and lift system relevant operational standards |
|  |  | 2.9 | System is inspected and tested to verify that the lift system operates as intended and to manufacturer specified requirements |
|  |  | 2.10 | Control methods for dealing with unplanned situations are responded to on the basis of workplace safety and specified work outcomes |
|  |  | 2.11 | Diagnosis and fault/s rectification activities are carried out efficiently without unnecessary waste of materials or damage to apparatus, the surrounding environment or services using sustainable energy practices |
| 3 | Complete rectification activities and documentation | 3.1 | WHS/OHS work rectification risk control measures and workplace procedures are followed |
|  |  | 3.2 | Reusable, faulty or worn components are tagged and despatched for repair in accordance with workplace procedures |
|  |  | 3.3 | Rectification of fault/s is documented in accordance with workplace procedures |
|  |  | 3.4 | Appropriate person/s is notified that the lift system fault/s has been rectified in accordance with workplace procedures |

# Foundation Skills

Foundation skills essential to performance are explicit in the performance criteria of this unit of competency.

# Range of Conditions

| Range is restricted to essential operating conditions and any other variables essential to the work environment. Non-essential conditions may be found in the UEE Electrotechnology Training Package Companion Volume Implementation Guide. |  |
| --- | --- |
| Replacing and/or adjusting of lift equipment must include at least three of the follow types of lift equipment: | electric traction lift electro-hydraulic lift goods lift passenger lift |
| Diagnosing and rectifying faults in lift systems must include at least three the following: | brakes controllers door components governors lift machines release passengers from a lift, which has become immobilised safety devices safety gear |

# Unit Mapping Information

This unit replaces and is equivalent to UEENEEG116A Diagnose and rectify faults in traction lift systems.

# Links

Companion Volume implementation guides are found in VETNet - - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b8a8f136-5421-4ce1-92e0-2b50341431b6