# UEPOPS114 Operate H.V. condition changing apparatus

Release: 1

UEPOPS114 Operate H.V. condition changing apparatus

# Modification History

Release 1. This is the first release of this unit of competency in the UEP Electricity Supply Industry – Generation Sector Training Package Release 2.0.

# Application

This unit involves the skills and knowledge required to undertake the operation of all high voltage (H.V.) condition changing apparatus. H.V. condition changing apparatus can include on- and off-load transformer tap changers, capacitor banks, rotary converters, voltage regulators, reactors, current limiters, rectifiers and transformers. H.V. changing apparatus works within system parameters such as voltage, current, frequency, volt ampere reactive load and equipment capability. Competency in this unit requires the ability to plan and prepare work, operate H.V. condition change apparatus and report on H.V. condition during operation. Individuals will, in general, work under supervision, in a power generation facility as an operator. Power generation plant operators are trained and authorised to isolate, prepare plant and issue permits to work. No licensing, legislative or certification requirements apply to this unit at the time of publication.

# Pre-requisite Unit

There are no prerequisite units

# Competency Field

Operations

# Unit Sector

Electricity generation

# Elements and Performance Criteria

| ELEMENTS |  | PERFORMANCE CRITERIA |  |
| --- | --- | --- | --- |
| Elements describe the essential outcomes. |  | Performance criteria describe the performance needed to demonstrate achievement of the element. |  |
| 1 | Prepare to operate H.V. condition changing apparatus | 1.1 | Work health and safety (WHS)/occupational health and safety (OHS) regulations, legislative requirements, codes of practice, manufacturers’ recommendations and specifications, environmental requirements and workplace procedures are identified, applied and monitored |
|  |  | 1.2 | Location of H.V. condition changing apparatus is determined from plans, drawings, system diagrams and maps in accordance with workplace procedures |
|  |  | 1.3 | H.V. condition changing apparatus for operation is identified and confirmed with relevant personnel in accordance with workplace procedures |
|  |  | 1.4 | Condition and status of H.V. condition changing apparatus is determined visually in accordance with workplace procedures |
|  |  | 1.5 | H.V. condition changing apparatus is prepared for operation in accordance with workplace procedures |
| 2 | Conduct H.V. condition changing apparatus operation | 2.1 | Relevant personnel are advised of progress during H.V. condition changing apparatus operation in accordance with workplace procedures |
|  |  | 2.2 | Conditions are evaluated prior to H.V. changing apparatus operation in accordance with workplace procedures |
|  |  | 2.3 | H.V. condition changing apparatus is operated in accordance with manufacturers’ recommendations and specifications, and workplace procedures |
|  |  | 2.4 | H.V. condition changing apparatus is controlled and adjusted to achieve outcomes in accordance with workplace procedures |
|  |  | 2.5 | H.V. condition changing apparatus is examined for operation in accordance with manufacturers’ recommendations and specifications, and workplace procedures |
|  |  | 2.6 | Conditions are re-evaluated to confirm H.V. condition changing apparatus outcome in accordance with workplace procedures |
|  |  | 2.7 | H.V. condition changing apparatus is secured in accordance with workplace procedures |
|  |  | 2.8 | Status of H.V. condition changing apparatus is confirmed in accordance with workplace procedures |
| 3 | Complete documentation | 3.1 | Equipment problems, movements and status are reported in accordance with workplace procedures |
|  |  | 3.2 | Documentation is updated in accordance with workplace procedures |

# Foundation Skills

Foundation skills essential to performance are explicit in the performance criteria of this unit of competency.

# Range of Conditions

Range is restricted to essential operating conditions and any other variables essential to the work environment. Non-essential conditions may be found in the UEP Electricity Supply Industry – Generation Sector Training Package Companion Volume Implementation Guide.

# Unit Mapping Information

This unit replaces and is equivalent to UEPOPS351 Operate H.V. condition changing apparatus.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=1715b9fa-e7bd-441c-bb8d-cf22c9c825a8