# UETDRAI003 Perform minor maintenance on electricity network assets

Release: 1

UETDRAI003 Perform minor maintenance on electricity network assets

# Modification History

Release 1. This is the first release of this unit of competency in the UET Transmission, Distribution and Rail Sector Training Package Release 2.0.

# Application

This unit involves the skills and knowledge required to perform minor maintenance on electricity network assets in the electricity supply industry (ESI). It includes signage/labels, cover plates/guards/strips, minor vegetation control and completing workplace documentation. The application of the skills and knowledge described in this unit may require a licence/registration to practice in the workplace. Other conditions may also apply under state and territory legislative and regulatory licensing requirements which must be confirmed prior to commencing this unit.

# Pre-requisite Unit

UEECD0007 Apply work health and safety regulations, codes and practices in the workplace UETDREL002 Comply with environmental requirements UETDREL006 Work safely in the vicinity of live electrical apparatus as a non-electrical worker

# Competency Field

Asset Inspection

# Unit Sector

Not applicable.

# Elements and Performance Criteria

| ELEMENTS |  | PERFORMANCE CRITERIA |  |
| --- | --- | --- | --- |
| Elements describe the essential outcomes. |  | Performance criteria describe the performance needed to demonstrate achievement of the element. |  |
| 1 | Prepare to perform minor maintenance | 1.1 | Legislation, regulations, standards, codes of practice and organisational workplace requirements for the work to be performed are referred to and confirmed |
|  |  | 1.2 | Work plan is obtained and confirmed in accordance with workplace requirements |
|  |  | 1.3 | Tools, equipment and personal protective equipment (PPE) required for work are determined, obtained and confirmed in working order |
|  |  | 1.4 | Materials required for work are determined and obtained in accordance with work plan and workplace requirements |
|  |  | 1.5 | Hazards are identified, risks assessed and control measures identified and applied |
|  |  | 1.6 | Work is prioritised and sequenced for completion in accordance with work plan and workplace requirements |
|  |  | 1.7 | Work permit/approval is organised in accordance with workplace requirements |
|  |  | 1.8 | Liaison and communication with stakeholders is carried out in accordance with workplace requirements |
|  |  | 1.9 | Worksite is prepared in accordance with the work plan and workplace requirements |
|  |  | 1.10 | Traffic management plan is confirmed as being in place in accordance with workplace requirements |
| 2 | Perform minor maintenance | 2.1 | Legislation, regulations, standards, codes of practice and organisational workplace requirements for the work to be performed are applied and monitored |
|  |  | 2.2 | Lifting, working at heights and the use of tools, equipment and PPE are carried out in accordance with workplace requirements |
|  |  | 2.3 | Hazard control measures are monitored in accordance with workplace requirements |
|  |  | 2.4 | Minor vegetation control is performed in accordance with the work plan and workplace requirements |
|  |  | 2.5 | Minor maintenance is performed on electricity network assets in accordance with the work plan and workplace requirements |
|  |  | 2.6 | Incidents or unplanned events are responded to in accordance with workplace requirements |
|  |  | 2.7 | Quality checks of work are undertaken in accordance with work plan and workplace requirements |
| 3 | Complete work and documentation | 3.1 | Completed work is checked for compliance against the work plan and workplace requirements |
|  |  | 3.2 | Incidents or unplanned events are reported in accordance with workplace requirements |
|  |  | 3.3 | Worksite is rehabilitated, cleaned and made safe in accordance with workplace requirements |
|  |  | 3.4 | Tools and equipment are cleaned, checked and returned to storage in accordance with workplace requirements |
|  |  | 3.5 | Surplus materials are returned to storage or disposed of in accordance with workplace requirements |
|  |  | 3.7 | Work permits are signed off, if applicable, in accordance with workplace requirements |
|  |  | 3.8 | Work records, reports and documentation are completed, and appropriate personnel notified in accordance with workplace requirements |

# Foundation Skills

Foundation skills essential to performance are explicit in the performance criteria of this unit of competency.

# Range of Conditions

Range is restricted to essential operating conditions and any other variables essential to the work environment. Non-essential conditions may be found in the UET Transmission, Distribution and Rail Sector Training Package Companion Volume Implementation Guide.

# Unit Mapping Information

This unit replaces and is equivalent to UETTDREL20 Undertake minor vegetation control and routine minor maintenance of poles and electrical apparatus.

# Links

Companion Volume Implementation Guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=229bace1-b7bc-4653-9300-dffb13ecfad7

Assessment Requirements for UETDRAI003 Perform minor maintenance on electricity network assets

# Modification History

Release 1. This is the first release of this unit of competency in the UET Transmission, Distribution and Rail Sector Training Package Release 2.0.

# Performance Evidence

Evidence required to demonstrate competence in this unit must be relevant to and satisfy all of the requirements of the elements and performance criteria on at least two separate occasions and include:

applying relevant legislation, regulations, standards, codes of practice and organisational workplace requirements, including: work health and safety (WHS)/occupational health and safety (OHS) safe approach distances identifying hazards, assessing risks, identifying, applying and monitoring control measures obtaining, inspecting and using relevant personal protective equipment (PPE) performing minor vegetation control performing minor maintenance on electricity network assets organising relevant work permits/approvals dealing with an unplanned event on at least one (1) occasion completing relevant work records, reports and documentation.

# Knowledge Evidence

Evidence required to demonstrate competence in this unit must be relevant to and satisfy all of the requirements of the elements and performance criteria and include knowledge of:

relevant legislation, regulations, standards, codes of practice, and organisational workplace requirements, including: WHS/OHS safe approach distances records, reports and documentation procedures hazard, risk assessment and risk control requirements, including potential hazards types and application of PPE safe use of tools and equipment application, purpose and types of work permits/approvals events constituting an unplanned event or incident procedures for responding to an unplanned event or incident minor vegetation control procedures types and procedures for minor maintenance on electricity network assets.

# Assessment Conditions

Assessors must hold credentials specified within the Standards for Registered Training Organisations current at the time of assessment. Assessment must satisfy the Principles of Assessment and Rules of Evidence and all regulatory requirements included within the Standards for Registered Training Organisations current at the time of assessment. Assessment must occur in workplace operational situations, where it is appropriate to do so. Where this is not appropriate, assessment must occur in simulated conditions involving realistic and authentic activities that replicate operational workplace conditions. Assessment processes and techniques must be appropriate to the language, literacy and numeracy requirements of the work being performed and the needs of the candidate. Resources for assessment must include access to: relevant and appropriate materials, tools, equipment and PPE currently used to perform minor maintenance on electricity network assets applicable documentation, including workplace requirements, equipment specifications, regulations, codes of practice and operation manuals.

# Links

Companion Volume Implementation Guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=229bace1-b7bc-4653-9300-dffb13ecfad7