# UETDRIS018 Perform low voltage field switching operation to a given schedule

Release: 1

UETDRIS018 Perform low voltage field switching operation to a given schedule

# Modification History

Release 1. This is the first release of this unit of competency in the UET Transmission, Distribution and Rail Sector Training Package Release 2.0.

# Application

This unit involves the skills and knowledge required to perform low voltage (LV) field switching operation to a given schedule in the electricity supply industry (ESI). It includes isolating, paralleling and restoring LV overhead and underground electricity networks using circuit breaking and isolating equipment. It also includes the approval process, using specialist tools, personal protective equipment (PPE), testing, short circuiting for switching and preparing, issuing and cancelling work permits. The application of the skills and knowledge described in this unit may require a licence/registration to practice in the workplace. Other conditions may also apply under state and territory legislative and regulatory licensing requirements which must be confirmed prior to commencing this unit.

# Pre-requisite Unit

UEECD0007 Apply work health and safety regulations, codes and practices in the workplace UEECD0044 Solve problems in multiple path circuits UEECD0046 Solve problems in single path circuits UEECD0051 Use drawings, diagrams, schedules, standards, codes and specifications UEEEL0020 Solve problems in low voltage a.c. circuits UEEEL0021 Solve problems in magnetic and electromagnetic devices UETDREL001 Apply environmental requirements UETDREL005 Work safely in the vicinity of live electrical apparatus

# Competency Field

Industry Specific Cross Discipline

# Unit Sector

Not applicable.

# Elements and Performance Criteria

| ELEMENTS |  | PERFORMANCE CRITERIA |  |
| --- | --- | --- | --- |
| Elements describe the essential outcomes. |  | Performance criteria describe the performance needed to demonstrate achievement of the element. |  |
| 1 | Prepare for LV field switching | 1.1 | Legislation, regulations, standards, codes of practice and organisational workplace requirements for the work to be performed are referred to and confirmed |
|  |  | 1.2 | Switching schedule is obtained and confirmed in accordance with workplace requirements |
|  |  | 1.3 | Plant, tools, equipment and PPE required for work are determined, obtained and confirmed in working order |
|  |  | 1.4 | Hazards are identified, risks assessed and control measures identified and applied |
|  |  | 1.5 | Work is prioritised and sequenced for completion in accordance with switching schedule and workplace requirements |
|  |  | 1.6 | Work permit/approval is organised in accordance with workplace requirements |
|  |  | 1.7 | Worksite is prepared in accordance with workplace requirements |
| 2 | Carry out LV field switching | 2.1 | Legislation, regulations, standards, codes of practice and organisational workplace requirements for the work to be performed are applied and monitored |
|  |  | 2.2 | Lifting, climbing, working at heights and the use of plant, tools, equipment and PPE are carried out in accordance with workplace requirements |
|  |  | 2.3 | Hazard control measures are monitored in accordance with workplace requirements |
|  |  | 2.4 | Communication with switching controller is established and maintained during switching in accordance with workplace requirements |
|  |  | 2.5 | Approval is obtained to commence LV field switching in accordance with switching schedule and workplace requirements |
|  |  | 2.6 | LV field switching is performed to isolate supply in accordance with the switching schedule and workplace requirements |
|  |  | 2.7 | Work permit is written and issued in accordance with the switching schedule and workplace requirements |
|  |  | 2.8 | Work permit is cancelled or relinquished in accordance with workplace requirements |
|  |  | 2.9 | Approval is obtained to commence restoration of LV supply in accordance with switching schedule and workplace requirements |
|  |  | 2.10 | LV field switching is performed to restore supply in accordance with the switching schedule and workplace requirements |
|  |  | 2.11 | Switching controller is notified LV restoration has been completed in accordance with switching schedule and workplace requirements |
|  |  | 2.12 | Incidents or unplanned events are responded to in accordance with workplace requirements |
| 3 | Complete work and documentation | 3.1 | Incidents or unplanned events are reported in accordance with workplace requirements |
|  |  | 3.2 | Worksite is rehabilitated, cleaned and made safe in accordance with workplace requirements |
|  |  | 3.3 | Plant, tools and equipment are cleaned, checked and returned to storage in accordance with workplace requirements |
|  |  | 3.4 | Work records, reports and documentation are completed in accordance with workplace requirements |

# Foundation Skills

Foundation skills essential to performance are explicit in the performance criteria of this unit of competency.

# Range of Conditions

Range is restricted to essential operating conditions and any other variables essential to the work environment. Non-essential conditions may be found in the UET Transmission, Distribution and Rail Sector Training Package Companion Volume Implementation Guide.

# Unit Mapping Information

This unit replaces and is equivalent to UETTDRIS43 Perform low voltage field switching operation to a given schedule.

# Links

Companion Volume Implementation Guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=229bace1-b7bc-4653-9300-dffb13ecfad7